3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513198

# Prague, CZ, 13th – 17th Oct, 2025

Title: 	Discussion on performance requirements for conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	6.17.2.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and the core part of WI for NR_Mob_Ph4 was completed on time. In this contribution, we discuss and provide our initial views on the performance requirements for conditional Intra-CU LTM for Rel-19 mobility enhancements.

- Discussion

- Test configurations and parameters

Regarding the test cases for conditional Intra-CU LTM, the legacy test configurations and parameters used in R18 LTM test cases can be a baseline.

Proposal 1: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases.

- Test scope

In RAN4#112-bis, only ‘PCell switch to a neighboring LTM candidate cell’ and ‘PCell switch to an LTM candidate cell that is a serving SCell in MCG’ were agreed for applicable scenarios, so we believe that only conditional LTM PCell Switch test cases should be defined.

Proposal 2: RAN4 to define new test cases for conditional LTM PCell Switch.

For intra/inter-frequency, we think that the test cases for CLTM should cover both intra and inter-frequency CLTM.

Proposal 3: the test cases for CLTM should cover both intra and inter-frequency CLTM.

In core part, RAN4 has agreed to define the requirements for L3 measurement based conditional LTM for FR1 and not to define the requirements for L3 measurement based conditional LTM for FR2, so we think that the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined as R18 LTM.

Proposal 4: RAN4 to define the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined.

Moreover, RAN4 agreed to cover both RACH-based and RACH-less conditional intra-CU LTM, so the test case for conditional LTM also need to cover RACH-based and RACH-less cases.

Proposal 5: The test cases for conditional LTM need to cover RACH-based and RACH-less cases.

For PDCCH ordered RACH, RAN4 has agreed that there is	no need to change existing PDCCH ordered RACH delay requirements, so we think that there is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

Proposal 6: There is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

For subsequent CLTM test scope, to save the test effort, we prefer to define the following test cases:

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

Proposal 7: For subsequent CLTM test scope, RAN4 to define the following test cases:

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

- Conclusion

This document discussed performance requirements for conditional Intra-CU LTM and presented following proposals:

Proposal 1: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases.

Proposal 2: RAN4 to define new test cases for conditional LTM PCell Switch.

Proposal 3: the test cases for CLTM should cover both intra and inter-frequency CLTM.

Proposal 4: RAN4 to define the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined.

Proposal 5: The test cases for conditional LTM need to cover RACH-based and RACH-less cases.

Proposal 6: There is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

Proposal 7: For subsequent CLTM test scope, RAN4 to define the following test cases:

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

- Reference

[1]    R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple