3GPP TSG-RAN WG4 Meeting #116-bis	                                                                                       R4-2513231

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on Rel-19 NR NTN Ph3 Demod requirements

Agenda item:	6.19.4

Document for:	Approval

- Introduction

In RAN4 #116 meeting, a WF [1] for NR NTN phase3 demodulation has been approved. However, there are still some open issues need to be discussed. This paper is initially shares some views on NR NTN phase3 SAN demodulation.

- Discussion

In RAN4 #116 meeting, a WF for NR NTN phase3 SAN demodulation has been approved, as shown below:

Issue 1-2-3: Frequency offset of two multiplexed UEs. Way forward: - Consider setting the same frequency offset for two UEs. - Evaluate two frequency offsets: 0 Hz and 200 Hz. Down select to one value according to the simulation results. - If there is no performance difference, select 200Hz. Issue 1-2-4: Channel model Way forward: - Apply the same channel model to the multiplex UEs. - Consider NTN-TDLC5-200. Interested companies can provide the simulation results with NTN-TDLA100-200. - Down select one channel model according to the simulation results. Issue 1-2-6: Test metric Way forward: - SNR at 70% of maximum throughput. - Measure the PUSCH throughputs per UE. - UE1 uses Orthogonal sequence index 0 - UE2 uses Orthogonal sequence index 1 - Set the same SNR for PUSCH from two UEs if there is no performance difference. - FFS how to write the specification e.g., define PUSCH demodulation requirements for each UE. Issue 1-2-8: Antenna configuration Way forward: - 1Tx – 1Rx - 1Tx – 2Rx - FFS whether to apply the existing SAN PUSCH demodulation requirements applicability between 1Rx and 2Rx requirements or not. Issue 1-2-10: Simulation alignment Way forward: In RAN4#116bis, interested companies are encouraged to provide simulation results of SAN demodulation for PUSCH with OCC length 2.

For frequency offset, we believed that the assumption of offset of two UEs captured by WF is reasonable, which the offset for two UEs should be same. Meanwhile, since the typical RF requirements for SAN frequency offset is 0.1ppm, the same assumption could be considered by demodulation as well. However, considering that we usually assume pre-compensation has been correctly implemented, a 0 Hz offset is also acceptable.

Proposal 1: For frequency offset of two multiplexed UEs, same CFO should be considered, and both 0Hz and 200 Hz offset are acceptable.

Similar to the discussion on the aforementioned frequency offset issue, it is reasonable to consider applying the same channel model for multiplexed UEs, which could alleviate the test burden. Meanwhile, the NTN-TDLA100-200 channel model is usually used for NLoS scenarios, while the NTN-TDLC5-200 is used for LoS scenarios. Considering the actual deployment scenario of NTN, we believe it is reasonable to use LoS scenario i.e. NTN-TDLC5-200 to conduct the further evaluation.

Proposal 2: For channel model of multiplexed UEs, same channel model should be applied, and the NTN-TDLC5-200 is acceptable.

For test metrics, SNR at 70% of maximum throughput is usually used in FR1 NTN to evaluate the performance requirements. Therefore, this metric could also be applied.

Proposal 3: For test metric, the SNR at 70% of maximum throughput could be considered.

- Conclusion

This contribution provides our initial views on SAN performance requirements for NR NTN phase3. The following proposals are concluded as follows:

Proposal 1: For frequency offset of two multiplexed UEs, same CFO should be considered, and both 0Hz and 200 Hz offset are acceptable.

Proposal 2: For channel model of multiplexed UEs, same channel model should be applied, and the NTN-TDLC5-200 is acceptable.

Proposal 3: For test metric, the SNR at 70% of maximum throughput could be considered.

- References

- R4-2512644, Way Forward for [116][331] NR_NTN_Ph3_demod,Ericsson, RAN4 #116.