3GPP TSG-RAN WG4 Meeting 116-bis	R4-2513278

# Prague, Czech Republic, October 13th – 17th, 2025

Source: 	Xiaomi

Title: 	Overview for 6GR RRM

Agenda Item:	8.7

Document for:	Discussion

- Overview

The design of Radio Resource Management (RRM) is foundational to the performance of any cellular system. As 6GR emerges, its unprecedented complexity—from diverse use cases and proliferating frequency bands to cross-domain measurement demands—exposes a fundamental limitation of current RRM architectures: their reliance on isolated feature design. This approach, which optimizes individual procedures in silos, has reached its scalability limit in 5G, resulting in systemic inefficiencies that can no longer be incrementally patched.

This document presents a comprehensive analysis and a forward-looking proposal for a Unified RRM requirements framework for 6GR overall system design in day 1. It is structured in three primary parts:

- Unified Measurement Framework: In 5G, measurement procedures were designed in isolation for individual features. This approach, which optimized for local rather than system-wide performance, inevitably led to the fragmented rules and suboptimal efficiency we observe today. A coordinated, cross-layer and cross-feature framework is therefore essential to achieve global optimization.

- Unified Measurement Gaps: Based on lessons from 5G (e.g. the afterward coordinate on the variable measurement gaps and unnecessary maintenance on RRM requirements impact due to the bundled measurement gap aspects, we identify several practical areas for simplification and enhancement in 6GR, including re-categorizing measurement types, rationalizing gap patterns, unifying configuration mechanisms, and optimizing for spectral efficiency.

- Unified modular based UE RRM Capability Definitions: Furthermore, we identify that the prevailing procedure-specific model for defining UE capabilities is inherently non-systemic, leading to specification fragmentation and implementation inefficiency. We propose a Unified modular based UE RRM capability definition for common functional blocks.

Proposal 1: RAN4 is invited to initiate a Study Item on Unified RRM Requirements Framework for 6GR. The work would be organized around the following study aspects:

- Unified Measurement Architecture — Study a common coordination framework for cross-layer and cross-feature measurement tasks, to clarify resource sharing, eliminate redundancies, and improve overall UE/Network efficiency.

- Unified Measurement Gap Design – Study a unified measurement-gap mechanism with emphasis on efficiency and reduced overhead

- Unified modular based UE RRM Capability Definitions – Study common, reusable capability blocks for fundamental UE abilities (e.g., beam-sweeping factor, message-processing footprint) to improve consistency and scalability across RRM procedures.

Collectively, this document outlines a path for 6GR RRM to evolve from a collection of isolated features into a coherent, efficient, and predictable system, ensuring robust performance and sustainable evolution from the start.

- Unified Measurement framework

This section frames a global view of RRM measurement for 6GR. The goal is not to prescribe solutions but to motivate a RAN4 study on Cross-Layer and Cross-Feature Coordination for baseline measurements.

      - 2.1 Measurement in 5G

### 2.1.1 Measurement as RRM's Primary Workload

In wireless systems, Radio Resource Management (RRM) is predominantly defined by one core activity: measurement. A structural analysis of TS 38.133 reveals that approximately 70% of its normative content directly defines measurement tasks, while another 10% governs measurement-triggered procedures. The time required for measurement constitutes a significant portion of total procedure latency.

This demonstrates that measurement is not merely one aspect of RRM, but its central workload. Optimizing measurement architecture through a unified approach is therefore equivalent to optimizing the core of RRM.

Observation 2-1: Measurement constitutes the core of RRM, and its unified architectural design defines overall RRM performance.

### 2.1.2 Observations from 5G

The 3GPP RAN4 working group has systematically evolved measurement requirements to support diverse NR features, culminating in the layered UE measurement framework of TS 38.133. We classify the related measurement tasks into three categories:

- L3 measurement for mobility

- L1 measurement for beam management and link adaptation, including L1-RSRP measurement for serving cell, RLM measurement, CBD measurement, BFD measurement, and L1-RSRP measurement for a cell with different PCI from serving cell

- L1 measurement for mobility, e.g., LTM L1 measurement on neighbor cells

Fig. 2-1 Illustration of NR measurement tasks across different domains

However, we saw some issues.

Challenge: Lack of Coordinated Resource Management

The current framework treats measurement categories as independent entities rather than parts of an integrated system. This is exemplified by the rigid, fixed-priority resource sharing scheme between L3 and L1 measurements, where L3 is always granted highest priority (factor P) while L1 is confined to residual resources (factor PL1_sharing). This artificial partitioning creates scheduling restrictions and degrades overall system performance, as it cannot adapt to the dynamic and concurrent nature of actual measurement demands.

Fig. 2-2 illustration on relationship of measurement resource allocation for multiple measurement tasks

Observation 2-2: The fixed resource allocation scheme, a legacy of treating measurement categories in isolation, results in improper scheduling and increased UE burden, underscoring the critical need for a Unified resource management strategy.

Challenge: Systemic Measurement Redundancy

This challenge arises from feature-specific design processes that occur without cross-feature coordination. A prime example is the parallel operation of L3 mobility measurements and Layer-1 (L1) neighbor cell tracking (LTM), both serving the same mobility purpose but configured and executed under separate, independent rules. This functional overlap leads to a direct waste of system resources, as the UE is forced to perform redundant measurements, unnecessarily increasing its power consumption and creating avoidable network scheduling overhead.

Observation 2-3: Redundancy, such as between L3 and L1 mobility measurements, is a systemic issue born from isolated design. It leads to wasted UE power and network resources, highlighting the necessity for early cross-feature coordination.

Challenge: Inconsistent System Behavior

The UE's measurement behavior is fragmented because requirements for similar tasks are specified in isolation. For instance, various L1 measurement tasks—including L1-RSRP for the serving cell, Radio Link Monitoring (RLM), and Beam Failure Detection (BFD)—are defined with separate and often differing performance requirements, despite being processed by the same UE module. This lack of harmonization is compounded by the absence of clear rules for handling time-domain conflicts, which complicates UE implementation, undermines network scheduling predictability, and ultimately degrades system performance.

Fig. 2-3 illustration on relationship of L1 measurement tasks

Observation 2-4: Inconsistent requirements for fundamentally similar task force divergent UE behaviors for the same objective, increasing implementation complexity and reducing overall system predictability.

### 2.1.3 Root cause analysis

The structural weaknesses observed in 5G measurement architecture originate from two fundamental missteps:

- Delayed RRM Involvement: RRM work began too late in the initial 5G standardization process. This left insufficient time to design a flexible and forward-looking measurement framework, resulting in a narrow initial design that struggled to accommodate subsequent features, ultimately limiting overall system performance.

- Feature-Specific Isolation: The prevailing methodology defined each measurement procedure in isolation, optimizing for individual feature performance while ignoring the collective impact of concurrent operations on the UE. This approach failed to manage the UE's measurement workload from a global perspective, leading to systemic conflicts and inefficiencies.

Maintaining the isolated design model of 5G would inevitably retain its system-level deficiencies while failing to meet 6G's demands. A new measurement architecture design is driven by two reasons:

- Systemic Deficiencies from 5G: Issues like redundancy, inconsistent, low efficiency resource allocation are systemic problems born from isolated design. They will persist and worsen in 6G unless resolved through a foundational, system-wide redesign.

- The 6G Concurrency Explosion: Predictable 6G trends will explode the number of concurrent measurement tasks, pushing the isolated model beyond its breaking point and making system-level efficiency non-negotiable.

Observation 2-5: The root causes of 5G's measurement fragmentation are the delayed inception of RRM work and the persistent feature-specific design paradigm. A proactive and unified architectural approach from the outset is essential for 6G.

      - 2.2 Strategic Directions for 6GR

### 2.2.1 Precious Window for Proactive Design

For 6G, we need a fundamental shift: instead of managing and optimizing each measurement feature on its own, we should integrate them into a single, efficient measurement system.

The start of 6G standardization is a critical window to get this right. If we don't plan early, we will face the same reactive situation we saw in 5G. Our experience with 5G now gives us a stable baseline of requirements, allowing us to move from reactive patching to proactive design. We can build a unified structure from the beginning that is designed to handle multiple tasks and shared resources natively.

In short, these ideas lead to a unified and extensible measurement architecture. This framework will act as a core coordinator, ensuring the UE's measurement engine works as a coherent and high-performance system.

Taken together, these principles point toward a unified and Extensible measurement architecture for 6GR, as illustrated in Fig. 2-4. This framework is designed to function as a central coordinating core, ensuring that the UE's measurement engine behaves as a coherent, high-performance system.

Fig. 2-4: A Unified 6GR measurement framework

### 2.2.2 Research Directions for a Unified Architecture

Every generation of mobile systems depends on a stable set of baseline measurements that enable fundamental connectivity and mobility. These are not optional features but the non-negotiable foundation of system operation, making their efficiency and consistency the natural starting point for 6G architectural redesign. RAN4 can start to study Unified Measurement framework for baseline measurement without waiting for other WG. Baseline measurement work can refer to current 5G known measurement types which is discussed in section 2.1.2.

We propose the following research directions for a Unified Measurement Framework. It is important to note that these options are complementary and not mutually exclusive. They represent different entry points to address the same core problem: the isolated design of measurement features.

We propose following research directions for Unified Measurement Framework, directly addressing current points of inefficiency and inconsistency:

- 1. Unifying Cross-Layer Measurements for Similar Function:

  - Pain Point: Measurement redundancy, e.g., L1-RSRP used in LTM (Layer 1 Mobility) and L3-RSRP used for mobility (Layer 3 Mobility), both serving the purpose of mobility management.

- 2. Unifying Cross-Function Measurements within the Same Layer:

  - Pain Point: Disparate requirements and scheduling conflicts, e.g., L1-RSRP for MIMO and L1-RSRP for LTM, both being L1 measurements but originating from distinct functional requirements.

- 3. Unifying Cross-Layer and Cross-Function Measurement Coordination:

  - Pain Point: Complex sharing factor and low efficiency between L1-RSRP for MIMO and L3-RSRP for mobility

- 4. Unifying Intra-Layer Link-Level Measurement Behavior:

  - Pain Point Addressed: similar motivation for BFD && RLM, and L1-RSRP && candidate beam detection. Besides, different scaling factor for BFD and L1-RSRP measurement in mTRP

Directions 1, 2, and 4 are core tasks that can be worked on independently and in parallel. For example:

- Direction 1 tackles redundancy between layers.

- Direction 2 solves conflicts within a single layer.

- Direction 4 harmonizes behaviors for similar procedures.

By making progress on these core tasks, we would automatically resolve a large part of the complex coordination problem described in Direction 3. Success in Directions 1 and 2 would naturally lead to a more efficient and less conflicted relationship between L1 and L3 measurements.

This structure provides a flexible research strategy. We can pursue a comprehensive solution across all fronts, or we can make significant progress through targeted, step-by-step advancements from these different angles. The ultimate goal is a cohesive framework where improvements in any one dimension contribute to the overall system's efficiency and consistency.

Observation 2-6: These research directions are interconnected. Progress on the core unification tasks (1, 2, 4) will inherently resolve the broader coordination challenges (3), enabling both comprehensive and incremental implementation paths.

      - 2.3 Summary

The unprecedented complexity of 6GR—from coexisting use cases to cross-domain requirements—necessitates a new approach to measurement design. Achieving Unified measurement efficiency requires architectural forethought that complements feature-specific designs.

Proposal 2-1: It is proposed that RAN4 initiate a Study on Unified Measurement Framework with the following possible directions:

  - Unified L1 and L3 measurement for mobility

  - Unified L1 measurement for MIMO and LTM

  - Unified L1 measurement for MIMO and L3 measurement for mobility

  - Unified L1 Link-Level Measurement

By adopting a Unified architecture, 6GR RRM can achieve foundational improvements:

- Enhanced System Efficiency & Reduced Complexity: lower UE power consumption and implementation cost by eliminating internal conflicts and redundancies.

- Predictable System Performance: clearer, more consistent UE behavior that simplifies network operations.

- Sustainable System Evolution: Create a future-proof foundation where new capabilities integrate seamlessly into the coordinated system.

# 3 Unified Measurement gap

As we discussed in 5G, for the measurement gap design were focus on the following perspectives at least:

- Measurement definitions by MG

- Measurement gap pattern

- How to configure the measurement gap

- When the measurement gap shall be applied or not

Therefore, in this section we can firstly provide our preliminary views on design principles for 6GR measurement from these aspects above respectively.

      - 3.1 Measurement types definitions depending on MG

In NR [TS38.133], RRM measurement requirements are categorized by intra-frequency, inter-frequency and inter-RAT measurement individually. Initially in LTE, the motivation to introduce the intra-frequency and inter-frequency measurement in RAN4 [TS36.133] was to differentiate the measurement reporting delay requirements because UE measurements cycle can be different because of MG applied or not simplify.

However, the such difference was vanished in NR. For an example in the figure below, the measurement SSB can be located within or out the active BWP, the intra-frequency measurement can be performed within the measurement gap. Similarly, for the inter-frequency measurements, it is possible to be performed without the measurement gap. Thus, from the requirements themselves, the difference between intra-frequency and inter-frequency measurement from the requirements perspective [TS38.133] is a little bit vague.

Figure 3-1. NR intra-frequency measurement with gap

Observation 3-: In NR the measurement requirements categorized by the intra/inter-frequency is not a precise way by which UE’s measurement behaviours and requirements can be defined distinctly.

On the other hand, for 6GR RAN4 requirement, we believe both within and out-of gap measurements are still needed indeed. The UE measurement behaivor can be essentially dependent with the gap since the available measurement occasion can be determined by the measurement period and reference signal jointly.

Figure 3-2. Gap based and gap less measurements

Observation 3-: For 6GR UE, both the gap-based and gapless measurements are needed.

Therefore, based on the observations above we can propose that:

Proposal 3-1: In 6GRR, the measurement requirements can be categorized by gap-based and gapless.

      - 3.2 Measurement gap pattern

In current TS38.133 the multiple measurement gap patterns were specified. Practically in 5G the implemented measurement gap pattern was only limited a few mandatory gap patterns due to UE implementation. For an example, in the current RAN4 test cases in TS38.133 A.6 [] there are few test cases with gp#0, gp#1, gp#3 used.

Observation 3-: More than 26 measurement patterns are defined. But there are only a few of them were implemented in 5G.

On the other hand, the too many unnecessary gap patterns will lead the higher UE implementation complexity. For an example, supporting the shorter MGRP need UE perform the measurements more frequently and the short MGL needs UE to process the measurement faster. Additionally, the testing efforts due to the more gap patterns will be increased significantly. From RAN4 standardization effort ourselves the more gap patterns shall be manageable also.

So we see the main extra burden comes from implementation and especially testing of the additional mandatory gap patterns.

Observation 3-: The burden because of the more gap patterns comes from UE implementing, testing efforts and especially RAN4 standardization work loading itself.

This means that base on 5G lessons, it would be undesirable to define too many gap patterns which are not in practice used by any 6GRR deployment.

Proposal 3-2: RAN4 shall focus on the mandatory measurement gap pattern(s) in 6GRR to minimize the gap pattern number.

For the measurement gap, the other important parameters discussed in NR is the gap pattern applicability as given in the table below. In short, there are both per-UE and per-FR MGs defined as in the table below.

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE [TS38.133]

| Measurement gap pattern configuration | Serving cell | Measurement PurposeNote 5 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | E-UTRA + FR1, or | non-NR RAT Note1,2 | 0,1,2,3 |
| Measurement gap | E-UTRA + FR2, or E-UTRA + FR1 + FR2 | FR1 and/or FR2 Note 6,7 | 0-11, 24, 25 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only Note 6,7 | No gap |
| Per-FR | FR2 if configured |  | 12-25 |
| measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | Non-NR RAT Note1,2 and FR1 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |

For the per-UE measurement gap, there is not any data/signals transmitted from the serving gNB to UE and UE is not required to receive any data/signals from the serving cell within the measurement gap duration on all frequency carriers/bands. In 5G, the per-FR measurement gap was introduced to reduce the system impacts on the other FRs. For an example, if the per-FR measurement gap was configured instead of per-UE, UE may transmit/receive the data in the other FR.

In 6GR，since UE needs to be operated in the more multiple bands [1], in order to achieve more efficient UE implementation the RF front-end of UE for the range of bands can be integrated together. That is the “band-group” operation can be supported by UE. For an example, the band group of sub3G can include the “LB+MHB+LMHB+MHB”. This direction can be potentially simplify CA in 6GR as well.

Observation 3-: if the measurement gap can be specified by per band-group, UE’s reception and transmission on the other band-group can be allo1wed

Proposal 3-3: the other applicability beside per-UE and per-FR can be studied upon RAN4 RF study on the CA simplified operation.

      - 3.3 Measurement gap configuration mechanism

In 5G NR Release 17 one of key enhancement for measurement gaps is to introduce the pre-configured MG by which the measurement gap can be inactivated when being configured. Technically speaking, in comparison with this pre-configured MG the legacy MG (Type 1 and 2) has not any advantages.

Observation 3-：The pre-configured MG can be more efficient to reduce the measurement delay in comparison with the legacy type 1& Type 2 MG without any signaling or throughput degradation.

However, in order to keep the backward compatibility in NR, both the legacy tyep1 and type 2 MG and pre-configured MG are supported. This also increase the specification complexity. For an example, the different UE behavior under the MG configuration needed to be specified in RAN4.

Observation 3-: It is redundant to define the two types of measurement gap configuration in RAN4. the unified MG configuration mechanism is needed.

There are two ways to activate the MG in NR also. One is based on the network signaling the other UE autonomously activation. However, there is little benefits with the multiple methods to activate the pre-configured MG.

Observation 3-8: For the pre-configured MG itself, the multiple activation MG methods are unnecessary.

Therefore, we can propose:

Proposal 3-4 : RAN4 can consider to unify the measurement gap configuration and activation mechanism in 6GRR.

      - 3.4 Gap efficiency optimization in 6GRR

Spectrum efficiency improvement has been highlighted as key considerations of 6GRR study in the last RANP 6GR workshop. Similarly, from RAN4 RRM study perspective, the enhanced measurement gap design in 6GRR needs to minimize the impacts to system spectrum efficiency because the network scheduling data and reception within the measurement gap is not allowed. In other words, in 6GRR UE needs to minimize the interruption on the network scheduling as possible.

Observation 3-9: the measurement gap enhancement to reduce the interruption time to the network data scheduling can help to improve the overall system spectrum efficiency.

- NCSG

In 5G, several useful methods were introduced to improve the measurement gap ultization. For an example, with NCSG, UE can perform the measurement on the other frequency carriers via the other vacant RF chain. Therefore, there is no any interruption on PCell within the measurement window but a small one when at the begin and end of this measurement window.

Figure x. NCSG in NR

Observation 3-10: The existing measurement gap design in NR (e.g. NCSG) can avoid the too large impacts on the system spectrum efficiency.

Therefore, we can propose that:

Proposal 3-5: RAN4 can leverage NCSG design in NR as one candidate solutions to improve the measurement gap efficiency in 6GRR.

- Gap-less measurement

The gapless measurements can bring gains for system throughput and/or mobility latency performance under certain conditions. But how did UE indicate and require the measurement without gap can be based on the different approaches in NR. And the UE behavior under the gapless measurement is not clearly defined.

Therefore, for 6GR, in order to reduce UE power consumption and improve the network throughput, the measurement without the gap can be further studied.

In NR RAN4 spec, whether UE needs to follow the requirements of the measurement without gap can be up to multiple specific conditions. For an example, for the intra-frequency measurement, the following conditions are defined.

Observation 3-11: UE can support the measurement without gap when:

- the UE's active bandwidth part (BWP) contains the measurement occasion bandwidth of neighbor cells .

- the UE supports simultaneous reception across the required frequencies .

However, in 6GRR since whether BWP operation can be kept is still open, the similar conditions for 6GR gap-less measurement which can be reused also needs RAN1 or other WGs’s decision.

Proposal 3-6: RAN4 can firstly study the capability and the conditions of measurement without gap to avoid the ambiguous UE behavior when the necessary physical layer design are stable in RAN1.

      - 3.5 Assumption on the UE implementation architecture

In principle, when RAN4 considering on when the measurement gap can be applied during UE’s measurement procedure, the fundamental factors shall be considered is how many available RF chain and BB processing units are available. For an example, in NR if there is vacant RF chain for CA operation, UE can measure the other carriers without measurement gap but NCSG.

And the assumption on the UE implementation baseline for the measurement is critical to design on the gap. For an example, when considering the minimum gap length the following factors needs to be evaluated.

- RF Switching delay : For sub 6GRHz bands, we expect that 0.5ms from LTE may be evaluated as a starting point

- Vacant RF chain for the parallel measurements and data reception

Moreover, for other important RRM requirements (e.g. the interruption or scheduling restriction) the different requirement can be applied for the different UE implementation architecture. For an instance, in NR the measurement without gap/interruption are highly rely on whether there are other RF chains beside the one for serving cell data reception.

Observation 3-12: the UE architecture including RF and baseband receivers can significantly impact RAN4 RRM requirements especially for measurement gap applicability and interruption.

Therefore, we can suggest that:

Proposal 3-7: Before the more concreted discussions on measurement gap and interruption requirements, RAN4 shall align on the baseline UE architecture.

For an example, in LTE the UE’s architecture was illustrated as below, which was taken as the baseline assumption for the corresponding analysis on whether the gap is needed for CA measurements.

Figure x. Possible UE Architectures in three aggregation scenarios in LTE [4,TR36.815]

On the other hand, considering the different types of devices in 6GRR (e.g. TN, NTN, MBB, FWA), in order to avoid too complicated HW implementation, the unified assumption on the UE baseline architecture is expected.

Observation 3-13: The UE architecture agonistic with use cases (e.g. NTN, MBB, FWA) is beneficial to converge RAN4’s baseline assumption .

# 4	Unified modular based UE RRM Capability Definitions

This section applies the system-level philosophy to a second foundational area: the definition of UE capabilities. We argue that modeling capabilities as properties of specific procedures, rather than as intrinsic properties of the UE system, has led to fragmentation and obscured true performance. A modular approach is proposed as the solution.

      - 4.1 The Problem: Procedure-Specific Modelling of System Capabilities

In 5G NR, UE capabilities are foundational for feature enablement. The prevailing approach is to define these capabilities in a manner tightly coupled with specific procedures. However, many distinct RRM procedures are constructed from a common set of underlying, reusable functional blocks (e.g., physical layer measurement, message processing). Defining these blocks differently for each procedure leads to systemic inefficiency.

Case 1: The Rx Beam Sweeping Factor
The fundamental function of receiver beam sweeping (with a factor 'N') impacts multiple procedures like L1-RSRP measurement, Radio Link Monitoring, and SCell activation. However, the mechanism to account for a "reduced sweeping" effort is defined differently in each context. This forces the same underlying UE hardware ability to be specified and tested in multiple, inconsistent ways.

Case 2: RRC Message Processing
The UE's ability to process messages quickly is a core hardware capability. In 5G, this is handled inconsistently: a specific capability exists for Layer Triggered Measurement (LTM), while Handover (HO) procedures use a fixed budget. This creates an artificial distinction for what is fundamentally the same UE processing power.

Case 3: Baseband/FFT Processing

The same underlying hardware ability—to handle certain FFT operations or scheduling restrictions—appears in disconnected contexts. For example, the capability to handle LTM RTD > CP and the capability to manage scheduling restrictions between SSB and data with different SCS are defined as separate, unrelated fields, despite drawing from the same baseband resource.

Observation 4-1: Defining fundamental UE abilities as attributes of specific procedures, rather than as reusable system modules, leads to specification fragmentation, increased testing burden, and obscures the true performance potential of the UE.

      - 4.2 The Path Forward: A Modular Capability Framework

The solution is to shift from procedure-specific capabilities to a modular framework of Common Functional Blocks. This involves:

- Identifying Reusable Blocks: Systematically identifying fundamental UE abilities that are reused across multiple RRM procedures

- Referencing, not Redefining: Allowing individual procedure requirements to reference these common capability modules, rather than embedding their own definitions.

This framework would make the RRM requirement specification more consistent, simplify the introduction of new features, and provide a clearer view of the UE's systemic performance.

Observation 4-2: A unified modular capability framework, based on common functional blocks, is key to reducing redundancy, improving consistency, and enhancing the scalability of RRM requirements in 6GR.

      - 4.3 Summary

The analysis in this section demonstrates that a unified, modular approach to capability definition is technically necessary to address the fragmentation and inefficiency inherited from 5G. The cases for beam sweeping, message processing, and baseband processing provide a clear justification for study direction 3 of the proposed Study Item.

It is therefore recommended that RAN4 agrees to include the study of a modular capability framework within the broader "Study on Unified RRM Requirements Framework for 6G".

# 5 Conclusion

In this contribution, we provide the following proposals:

Proposal 1: RAN4 is invited to initiate a Study Item on Unified RRM Requirements Framework for 6GR. The work would be organized around the following study aspects:

      - Unified Measurement Architecture — Study a common coordination framework for cross-layer and cross-feature measurement tasks, to clarify resource sharing, eliminate redundancies, and improve overall UE/Network efficiency.

      - Unified Measurement Gap Design – Study a unified measurement-gap mechanism with emphasis on efficiency and reduced overhead

      - Unified modular based UE RRM Capability Definitions – Study common, reusable capability blocks for fundamental UE abilities (e.g., beam-sweeping factor, message-processing footprint) to improve consistency and scalability across RRM procedures.

Observation 2-1: Measurement constitutes the core of RRM, and its unified architectural design defines overall RRM performance.

Observation 2-2: The fixed resource allocation scheme, a legacy of treating measurement categories in isolation, results in improper scheduling and increased UE burden, underscoring the critical need for a Unified resource management strategy.

Observation 2-3: Redundancy, such as between L3 and L1 mobility measurements, is a systemic issue born from isolated design. It leads to wasted UE power and network resources, highlighting the necessity for early cross-feature coordination.

Observation 2-4: Inconsistent requirements for fundamentally similar task force divergent UE behaviors for the same objective, increasing implementation complexity and reducing overall system predictability.

Observation 2-5: The root causes of 5G's measurement fragmentation are the delayed inception of RRM work and the persistent feature-specific design paradigm. A proactive and unified architectural approach from the outset is essential for 6G.

Observation 2-6: These research directions are interconnected. Progress on the core unification tasks (1, 2, 4) will inherently resolve the broader coordination challenges (3), enabling both comprehensive and incremental implementation paths.

Proposal 2-1: It is proposed that RAN4 initiate a Study on Unified Measurement Framework with the following possible directions:

  - Unified L1 and L3 measurement for mobility

  - Unified L1 measurement for MIMO and LTM

  - Unified L1 measurement for MIMO and L3 measurement for mobility

  - Unified L1 Link-Level Measurement

Observation 3-1: In NR the measurement requirements categorized by the intra/inter-frequency is not a precise way by which UE’s measurement behaviours and requirements can be defined distinctly.

Observation 3-2: For 6GR UE, both the gap-based and gapless measurements are needed.

Proposal 3-1: In 6GRR, the measurement requirements can be categorized by gap-based and gapless.

Observation 3-3: More than 26 measurement patterns are defined. But there are only a few of them were implemented in 5G.

Observation 3-4: The burden because of the more gap patterns comes from UE implementing, testing efforts and especially RAN4 standardization work loading itself.

Proposal 3-2: RAN4 shall focus on the mandatory measurement gap pattern(s) in 6GRR to minimize the gap pattern number.

Observation 3-5: if the measurement gap can be specified by per band-group, UE’s reception and transmission on the other band-group can be allo1wed

Proposal 3-3: the other applicability beside per-UE and per-FR can be studied upon RAN4 RF study on the CA simplified operation.

Observation 3-6：The pre-configured MG can be more efficient to reduce the measurement delay in comparison with the legacy type 1& Type 2 MG without any signaling or throughput degradation.

Observation 3-7: It is redundant to define the two types of measurement gap configuration in RAN4. the unified MG configuration mechanism is needed.

Observation 3-8: For the pre-configured MG itself, the multiple activation MG methods are unnecessary.

Proposal 3-4 : RAN4 can consider to unify the measurement gap configuration and activation mechanism in 6GRR.

Observation 3-9: the measurement gap enhancement to reduce the interruption time to the network data scheduling can help to improve the overall system spectrum efficiency.

Observation 3-10: The existing measurement gap design in NR (e.g. NCSG) can avoid the too large impacts on the system spectrum efficiency.

Proposal 3-5: RAN4 can leverage NCSG design in NR as one candidate solutions to improve the measurement gap efficiency in 6GRR.

Observation 3-11: UE can support the measurement without gap when:

  - the UE's active bandwidth part (BWP) contains the measurement occasion bandwidth of neighbor cells .

  - the UE supports simultaneous reception across the required frequencies .

Proposal 3-6: RAN4 can firstly study the capability and the conditions of measurement without gap to avoid the ambiguous UE behavior when the necessary physical layer design are stable in RAN1.

Observation 3-12: the UE architecture including RF and baseband receivers can significantly impact RAN4 RRM requirements especially for measurement gap applicability and interruption.

Proposal 3-7: Before the more concreted discussions on measurement gap and interruption requirements, RAN4 shall align on the baseline UE architecture.

Observation 3-13: The UE architecture agonistic with use cases (e.g. NTN, MBB, FWA) is beneficial to converge RAN4’s baseline assumption.

Observation 4-1: Defining fundamental UE abilities as attributes of specific procedures, rather than as reusable system modules, leads to specification fragmentation, increased testing burden, and obscures the true performance potential of the UE.

Observation 4-2: A unified modular capability framework, based on common functional blocks, is key to reducing redundancy, improving consistency, and enhancing the scalability of RRM requirements in 6GR.

# 6 Reference

[1] R4-2513274, Overview for 6G spectrum, Xiaomi