3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513295

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	4.2.8

Source: 	Murata Manufacturing Co Ltd.

Title: 	MSD analysis for 10MHz CBW of n41/n90

Document for:	Discussion

Introduction

In the RAN4#116 meeting, the optional CBW for Rel-19 FR1 bands was discussed in WF [1]. In this WF, it was agreed that 5MHz CBW of n41/n90 remains optional based on operators request, and update 38.101-1 and 38.101-3 MSD test points for 10MHz CBW.

In this contribution, we analyzed and updated the MSD test points for 10MHz CBW of n41/n90.

Discussion

MSD analysis procedure

It was found that there are many MSD test points which should be changed in 38.101-1 and 38.101-3 TS. And those are all MSD due to 2 UL IMD. It is not efficient to calculate the all MSD requirements case by case by considering RF architecture and linearity of RF components. Therefore, we adopted a simple approach to calculate MSD by changing the CBW from 5MHz to 10MHz of n41 as below.

The first step is to utilize the existing PC3 MSD at n41 5MHz case and find the IMD interferer level by reverse engineering shown in Table 1.

|  | PRX | DRX |
| --- | --- | --- |
| Interferer noise at PRX Ant | Pint_PC3 (A) |  |
| Interferer imbalance | 6 |  |
| REFSENS | RefSens |  |
| Thermal | RefSens+4 (Nth) | RefSens+4 (Nth) |
| Interferer | Pint_PC3 (A) | Pint_PC3-6 (B) |
| Composite | Log[Nth + A] | Log[Nth + B] |
| MRC sensitivity | MRC composite noise -1 |  |
| PC3 MSD | MRC sensitivity - RefSens |  |

We know PC3 MSD and REFSENS from the existing table and assuming 6dB interferer imbalance, we can find the IMD interferer noise at PRX antenna (A).

In addition, simulations were performed to see the IMD interferer difference between 5MHz n41 CBW and 10MHz n41 CBW.

Once the IMD difference was obtained, we can put the resulting IMD interferer for 10MHz CBW in Table 1 (A) again to finally calculate PC3 MRC MSD.

New MSD calculation result for 10MHz CBW

Table 1~Table 4 show the PC3 MSD due to IMD tables which should be modified for CA (38.101-1) and DC (38.101-3).

The channel bandwidth for n41 is changed from 5MHz to 10MHz shown in yellow cells, and the RB length also should be changed to 50 RB if n41 is aggressor band as in green cells. In the sky-blue cells, the current MSD for 5MHz and the calculated MSD for 10MHz are shown together. It should be noted that for intraband non-contiguous DC of DC_25A-41A_n41A with 1RB allocation, the MSD is not changed but the RB start location for 10MHz should be changed to ensure that the IMD7 is falling within the DL frequency range.

Here are two cases for CBW change from 5MHz to 10MHz.

- n41 is one of aggressors, so the IMD bandwidth becomes wider (or IMD PSD lower) depending on the IMD order of n41 UL.

- n41 is victim, the IMD bandwidth remains same but overlaps more with n41 DL.

For 1), victim CBW is same but IMD BW becomes wider, the IMD interferer falling into victim CBW becomes lower. So, the new MSD for 10MHz CBW of n41 aggressor should be slightly smaller than 5MHz.

For 2), the IMD interferer falling into n41 victim 10MHz CBW becomes higher but the reference sensitivity for n41 10MHz CBW is also increased to -94.8dBm (vs -98dBm for 5MHz CBW). Therefore, the resulting MSD for 10MHz CBW of n41 victim is similar or smaller than 5MHz.

During the analysis, we found that some test frequencies are not correct to match the IMD relations (m*FUL1 + n*FUL2 = FDL) especially for some 2UL/3DL CA cases (38.101-1). These are shown in red cells with the new test frequencies for correct IMD relations.

And if the n41 centre frequency is 2500MHz, then it should be shifted to the upper frequency for 10MHz channel bandwidth such as 2503MHz (2498~2508MHz). This is shown in orange cells to ensure the correct IMD relations between 2 aggressors and victim frequencies.

Finally, one MSD case (UL1: n66, UL2: n85, DL: n41 in CA_n41-n66-n85) can be removed. This is because the maximum frequency of IMD2 interferer between n66 and n85 is 2496MHz, which is just the start frequency of n41 band. This is shown in grey cell.

Table 1 PC3 MSD due to 2 UL IMD – 2DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 -> 14.3 | FDD | IMD3 |
|  | n41 | 2505 | 5- > 10 | 25 -> 50 | 2505 | N/A | TDD | N/A |
| CA_n41-n71 | n41 | 2614 | 5- > 10 | 25 -> 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 -> 10.2 | FDD | IMD4 |

Table 2 PC3 MSD due to 2 UL IMD – 3DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n1-n40-n41 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n40 | 2390 -> 2310 | 5 | 25 | 2390 -> 2310 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2630 -> 2650 | 23 -> 21.3 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2130 | 25.2 -> 23.9 | FDD | IMD31 |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | TDD | N/A |
|  | n41 | 2540 | 5 -> 10 | 25 -> 50 | 2660 | N/A | TDD | N/A |
| CA_n3-n18-n41 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | N/A | TDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2630 | 16 | TDD | IMD3 |
|  | n18 | N/A | 5 | N/A | 865 | 28.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 875 -> 865 | 19 -> 17.7 | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 -> 2585 | 5 -> 10 | 25 -> 50 | 2670 -> 2585 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 28.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 -> 1780 | 5 | 25 | 1810 -> 1875 | N/A | FDD | N/A |
|  | n28 | 743 -> 738 | 5 | 25 | 798- > 793 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2518 | 27.4 -> 25.5 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2687 | 15.9 -> 14.6 | TDD | IMD3 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 -> 10 | 25 -> 50 | 2510 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 26 -> 24.3 | FDD | IMD24 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 26 | FDD | IMD2 |
| CA_n3-n34-n41 | n3 | 1742.5 | 5 | 25 | 1837.5 | N/A | FDD | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2550 -> 2575 | 1 -> 0.8 | TDD | IMD5 |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2640 | 5.3 -> 5 | TDD | IMD5 |
|  | n41 | 2620 | 5 -> 10 | 25 -> 50 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 -> 15.1 | FDD | IMD3 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 16.8 -> 16.1 | TDD | IMD31 |
| CA_n3-n41-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.4 | TDD | IMD3 |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 -> 15.1 | TDD | IMD3 |
|  | n41 | 2620 | 5 -> 10 | 25 -> 50 | 2620 | N/A | FDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
| CA_n5-n25-n41 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2685 | 30 -> 28.1 | TDD | IMD2 |
| CA_n5-n41-n77 | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n41 | 2540 | 5 -> 10 | 25 -> 50 | 2540 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3375 | 29.7 -> 29.2 | TDD | IMD22 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n41 | 2500 -> 2503 | 5 -> 10 | 25 -> 50 | 2500 -> 2503 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4160 -> 4183 | 16.1 -> 15 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2645 | 30.1 -> 29.9 | TDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2510 | 13.2 -> 12.6 | FDD | IMD3 |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 879 | 30.2 -> 29.6 | FDD | IMD2 |
|  | n41 | 2550 | 5 -> 10 | 25 -> 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 900 -> 886 | 15.2 -> 13.8 | FDD | IMD31 |
|  | n41 | 2500 -> 2503 | 5 -> 10 | 25 -> 50 | 2500 -> 2503 | N/A | TDD | N/A |
|  | n77 | 4100 -> 4120 | 10 | 50 | 4100 | N/A | TDD | N/A |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2570 | 5 -> 10 | 25 -> 50 | 2570 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 30.1 -> 29.6 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2630 | 28.5 -> 27.9 | TDD | IMD24 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 29.3 | FDD | IMD21,4 |
| CA_n20-n41-n77 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 5 -> 10 | 25 -> 50 | 2593 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 29.7 -> 29.2 | TDD | IMD22 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2506 | 5 -> 10 | 25 -> 50 | 2506 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4165 | 16.1 -> 15.6 | TDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2593 | 30.1 -> 29.4 | TDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2516 | 13.2 -> 12.6 | TDD | IMD3 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.2 -> 29.7 | FDD | IMD2 |
|  | n41 | 2594 | 5 -> 10 | 25 -> 50 | 2594 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 817 | 15.2 -> 13.8 | FDD | IMD3 |
|  | n41 | 2501 | 5 -> 10 | 25 -> 50 | 2501 | N/A | TDD | N/A |
|  | n77 | 4185 | 10 | 50 | 4185 | N/A | TDD | N/A |
| CA_n24-n41-n48 | n24 | 1649 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 -> 10 | 25 -> 50 | 2610 | N/A | TDD | N/A |
|  | n48 | N/A | 10 | N/A | 3571 | 16.8 -> 15.5 | TDD | IMD3 |
|  | n24 | 1630 -> 1629 | 5 | 25 | 1528.5 -> 1527.5 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2500 -> 2503 | 5.3 -> 4.9 | TDD | IMD5 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n24 | N/A | 5 | N/A | 1530 | 16.4 -> 15 | FDD | IMD3 |
|  | n41 | 2592.5 | 5 -> 10 | 25 -> 50 | 2592.5 | N/A | TDD | N/A |
|  | n48 | 3655 | 10 | 50 | 3655 | N/A | TDD | N/A |
| CA_n24-n41-n77 | n24 | 1630 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | 2685 | 5 -> 10 | 25 -> 50 | 2685 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3735 -> 3740 | 16.8 -> 15.5 | TDD | IMD31,5 |
|  | n24 | 1630 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2610 -> 2620 | 5.3 -> 4.9 | TDD | IMD55 |
|  | n77 | 3755 | 10 | 50 | 3755 | N/A | TDD | N/A |
|  | n24 | N/A | 5 | N/A | 1528.5 -> 1546 | 16.4 -> 15 | FDD | IMD32,5 |
|  | n41 | 2500 -> 2503 | 5 -> 10 | 25 -> 50 | 2500 -> 2503 | N/A | TDD | N/A |
|  | n77 | 3465 -> 3460 | 10 | 50 | 3465 -> 3460 | N/A | TDD | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 -> 10 | 25 -> 50 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 14.8 -> 13.6 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 -> 10 | 25 -> 50 | 2645 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3775 | 4.2 -> 3.3 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2640 | 5.3 -> 5 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 17.6 -> 16.3 | FDD | IMD35 |
|  | n41 | 2675 | 5 -> 10 | 25 -> 50 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 8.6 -> 8.2 | FDD | IMD4 |
|  | n41 | 2550 | 5 -> 10 | 25 -> 50 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| CA_n25-n41-n78 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 -> 10 | 25 -> 50 | 2610 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3350 | 14.8 -> 13.6 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 -> 10 | 25 -> 50 | 2645 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3775 | 4.2 -> 3.3 | TDD | IMD5 |
|  | n25 | N/A | 5 | N/A | 1950 | 17.6 -> 16.3 | FDD | IMD3 |
|  | n41 | 2565 | 5 -> 10 | 25 -> 50 | 2565 | N/A | TDD | N/A |
|  | n78 | 3180 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 8.6 -> 8.2 | FDD | IMD4 |
|  | n41 | 2550 | 5 -> 10 | 25 -> 50 | 2685 | N/A | TDD | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | TDD | N/A |
| CA_n25-n41-n85 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | TDD | N/A |
|  | n85 | N/A | 5 | N/A | 738 | 28.7 | FDD | IMD24 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2608 | 28.7 -> 26.8 | TDD | IMD2 |
|  | n85 | 708 | 5 | 25 | 738 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1952 | 26 | FDD | IMD2 |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | TDD | N/A |
|  | n85 | 708 | 5 | 50 | 738 | N/A | FDD | N/A |
| CA_n28-n41-n77 | n41 | 2642 | 5 -> 10 | 25 -> 50 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 798 | 30.8 -> 30.2 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 782.5 | 3 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2642 | 29.5 -> 28.8 | TDD | IMD2 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3323 | 28.2 -> 27.7 | TDD | IMD24 |
| CA_n28-n41-n78 | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2642 | 29.5 -> 28.8 | TDD | IMD2 |
|  | n41 | 2642 | 5 -> 10 | 25 -> 50 | 2642 | N/A | TDD | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 798 | 30.8 -> 30.2 | FDD | IMD21 |
|  | n41 | 2565 | 5 -> 10 | 25 -> 50 | 2565 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 29.7 -> 29.2 | TDD | IMD22 |
| CA_n34-n41-n79 | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | 2660 | 5 -> 10 | 25 -> 50 | 2660 | N/A | TDD | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 19.3 -> 19.3 | TDD | IMD2 |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | 2550 | 5 -> 10 | 25 -> 50 | 2550 | 27.2 -> 27.1 | TDD | IMD2 |
|  | n79 | 4570 | 40 | 216 | 4570 | N/A | TDD | N/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 28.6 -> 28.6 | TDD | IMD2 |
|  | n41 | 2585 | 5 -> 10 | 25 -> 50 | 2585 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 7.5 -> 7.5 | TDD | IMD5 |
|  | n41 | 2515 | 5 -> 10 | 25 -> 50 | 2515 | N/A | TDD | N/A |
|  | n79 | 4780 | 40 | 216 | 4780 | N/A | TDD | N/A |
| CA_n41-n66-n77 | n41 | 2600 | 5 -> 10 | 25 -> 50 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 16.1 -> 15 | TDD | IMD31,2 |
|  | n41 | N/A | 5 -> 10 | N/A | 2670 | 5.2 -> 4.8 | TDD | IMD55 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 | 5 -> 10 | 25 -> 50 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 9 -> 7.9 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n41-n66-n78 | n41 | 2560 | 5 -> 10 | 25 -> 50 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 -> 15 | TDD | IMD31 |
|  | n41 | 2530 | 5 -> 10 | 25 -> 50 | 2530 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 9 -> 7.9 | FDD | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n66-n85 | n41 | N/A | 5 | N/A | 2498.5 | 27.6 | TDD | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2197.5 | N/A | FDD | N/A |
|  | n85 | 713.5 | 5 | 25 | 743.5 | N/A | FDD | N/A |
|  | n41 | 2501 | 5 -> 10 | 25 -> 50 | 2501 | N/A | TDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2190 | N/A | FDD | N/A |
|  | n85 | N/A | 5 | N/A | 731 | 31 -> 29.2 | FDD | IMD21 |
| CA_n41-n71-n77 | n41 | 2615 | 5 -> 10 | 25 -> 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 29.1 -> 28.6 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 -> 10 | 25 -> 50 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 16.3 -> 15.8 | TDD | IMD31 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3774 | 10.3 -> 9.4 | TDD | IMD41 |
|  | n41 | N/A | 5 -> 10 | N/A | 2615 | 28.7 -> 28 | TDD | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2564 | 15.5 -> 14.9 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 41 | 2680 | 5 -> 10 | 25 -> 50 | 2680 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 30.8 -> 30.2 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n41-n71-n78 | n41 | 2615 | 5 -> 10 | 25 -> 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3308 | 29.1 -> 28.6 | TDD | IMD21 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3774 | 10.3 -> 9.4 | TDD | IMD41 |
|  | n41 | N/A | 5 -> 10 | N/A | 2615 | 28.7 -> 28 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 -> 10 | 25 -> 50 | 2642 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 798 | 30.8 -> 30.2 | FDD | IMD2 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n41-n77-n85 | n41 | 2687 | 5 -> 10 | 25 -> 50 | 2687 | N/A | TDD | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | N/A | TDD | N/A |
|  | n85 | N/A | 5 | N/A | 733 | 30.8 -> 30.2 | FDD | IMD25 |
|  | n41 | N/A | 5 -> 10 | N/A | 2619 | 29.5 -> 28.8 | TDD | IMD24.5 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
|  | n41 | 2680 | 5 -> 10 | 25 -> 50 | 2680 | N/A | TDD | N/A |
|  | n77 | 3393 | 10 | N/A | 3393 | 28.2 -> 27.7 | TDD | IMD24,5 |
|  | n85 | 713 | 5 | 25 | 743 | N/A | FDD | N/A |

Table 3 PC3 MSD due to 2 UL IMD – 2DL/2UL (38.101-3)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | IMD order |
| Configuration |  | (MHz) | (MHz) | LCRB |  | (dB) |  |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 -> 10.2 | IMD4 |
|  | n41 | 2618 | 5 -> 10 | 25 -> 50 | 2618 | N/A | N/A |

Table 4 PC3 MSD due to 2 UL IMD – 3DL/2UL (38.101-3)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | IMD order |
| Configuration |  | (MHz) | (MHz) | LCRB |  | (dB) |  |
| DC_1A_n3A-n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2507.5 | 5 -> 4.5 | IMD5 |
| DC_1A-8A_n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 927.5 | 1 -> 0.7 | IMD5 |
|  | n41 | 2502.5 | 5 -> 10 | 25 -> 50 | 2502.5 | N/A | N/A |
| DC_2A_n41A-n77A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n41 | 2670 | 5 -> 10 | 25 -> 50 | 2670 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 14.8 -> 13.6 | IMD34 |
|  | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2640 | 5.3 -> 5 | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | N/A |
| DC_2A-66A_n41A | 2 | N/A | 5 | N/A | 1940 | 11 -> 9 | IMD4 |
| DC_2A-66A_n41C |  |  |  |  |  |  |  |
| DC_2A-66A_n41(2A) |  |  |  |  |  |  |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 -> 10 | 25 -> 50 | 2685 | N/A | N/A |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2507.5 | 5 -> 4.5 | IMD5 |
| DC_3A_n3A-n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1865 | 8.2 -> 6.6 | IMD4 |
|  | n41 | 2657.5 | 5 -> 10 | 25 -> 50 | 2657.5 | N/A | N/A |
| DC_3-18_n41 | 18 | N/A | 5 | N/A | 865 | 28.9 | IMD2 |
|  | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | N/A |
|  | 18 | N/A | 5 | N/A | 865 | 19 -> 17.7 | IMD3 |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n41 | 2585 | 5 -> 10 | 25 -> 50 | 2585 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1850 | 28.8 | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | MSD | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 -> 10 | 25 -> 50 | 2510 | N/A | N/A |
|  | 28 | N/A | 5 | N/A | 790 | 26 -> 24.3 | IMD21 |
|  | 3 | N/A | 5 | N/A | 1832.5 | 26 | IMD2 |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 26.1 -> 24.3 | IMD2 |
|  |  |  |  |  |  |  | ¦fn41-fB3¦ |
|  | n41 | 2510 | 5 -> 10 | 25 -> 50 | 2510 | N/A | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2518 | 27.4 -> 25.5 | IMD2 |
|  |  |  |  |  |  |  | ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2687 | 15.9 -> 14.6 | IMD3 |
|  |  |  |  |  |  |  | ¦2*fB3-fn28¦ |
| DC_3A-41A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
| DC_3A-41C_n77A |  |  |  |  |  |  |  |
| DC_3A-41A_n77(2A) |  |  |  |  |  |  |  |
| DC_3A-41C_n77(2A) |  |  |  |  |  |  |  |
| DC_3A_n41A-n77A |  |  |  |  |  |  |  |
| DC_3A_n41A-n77(2A) |  |  |  |  |  |  |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41/n41 | N/A | 5 -> 10 | N/A | 2640 | 5.3 -> 5 | IMD5 |
|  | 41/n41 | 2620 | 5 -> 10 | 25 -> 50 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1840 | 16.4 -> 15.1 | IMD3 |
| DC_5A_n41A-n77A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n41 | 2540 | 5 -> 10 | 25 -> 50 | 2540 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3375 | 29.7 -> 29.2 | IMD22 |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n41 | 2500 -> 2503 | 5 -> 10 | 25 -> 50 | 2500 -> 2503 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 4160 -> 4183 | 16.1 -> 15 | IMD3 |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2645 | 30.1 -> 29.9 | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2510 | 13.2 -> 12.6 | IMD3 |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
| DC_5A_n41A-n78A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n41 | 2540 | 5 -> 10 | 25 -> 50 | 2540 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 -> 29.2 | IMD22 |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2645 | 30.1 -> 29.4 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
| DC_8A_n41A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n41 | 2555 | 5 -> 10 | 25 -> 50 | 2555 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 -> 28 | IMD21 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2650 | 28 -> 27.3 | IMD2 |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_12A_n2A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2608 | 28.7 -> 27 | IMD2 |
| DC_12A_n25A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2608 | 28.7 -> 27 | IMD2 |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1970 | 26 | IMD2 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |
| DC_12A-66A_n41A | 12 | N/A | 5 | N/A | 742 | 31 -> 29.3 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n41 | 2515 | 5 -> 10 | 25 -> 50 | 2515 | N/A | N/A |
| DC_18A_n41A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
| DC_18A_n41A-n78A |  |  |  |  |  |  |  |
|  | n41 | 2570 | 5 -> 10 | 25 -> 50 | 2570 | N/A | N/A |
|  | n77/n78 | N/A | 10 | N/A | 3390 | 30.1 -> 29.6 | IMD2 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n77/n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2630 | 28.5 -> 27.9 | IMD2 |
| DC_25A-41A_n41A | 25 | N/A | 5 | N/A | 1992.5 | 8.5 | IMD7 |
| DC_25A-41C_n41A |  |  |  |  |  |  |  |
| DC_25A-41D_n41A |  |  |  |  |  |  |  |
| DC_25A-25A-41A_n41A | 41 | 2502.5 | 5 | 1 (RBstart=0) | 2502.5 | N/A | N/A |
| DC_25A-25A-41C_n41A |  |  |  |  |  |  |  |
| DC_25A-25A-41D_n41A |  |  |  |  |  |  |  |
| DC_25A-(n)41CA | n41 | 2670 | 5 -> 10 | 1 (RBstart=9) -> 1 (RBstart=22) | 2670 | N/A | N/A |
| DC_25A-(n)41DA |  |  |  |  |  |  |  |
| DC_25A-25A-(n)41CA |  |  |  |  |  |  |  |
| DC_25A-25A-(n)41DA |  |  |  |  |  |  |  |
| DC_66A_n2A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | 11 -> 9.1 | IMD4 |
|  | n41 | 2685 | 5 -> 10 | 25 -> 50 | 2685 | N/A | N/A |
| DC_66A_n41A-n77A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n41 | 2600 | 5 -> 10 | 25 -> 50 | 2600 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 16.1 -> 15 | IMD31 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2670 | 5.2 -> 4.8 | IMD55 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| DC_66A_n41A-n78A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n41 | 2560 | 5 -> 10 | 25 -> 50 | 2560 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 -> 15 | IMD3 |
| DC_71A_n2A-n41A | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2586 | 29.2 -> 27.2 | IMD2 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n2 | N/A | 5 | N/A | 1942 | 26 -> 24.2 | IMD2 |
|  | n41 | 2610 | 5 -> 10 | 25 -> 50 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
| DC_71A_n25A-n41A | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2586 | 29.2 -> 27.2 | IMD29 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1942 | 26 -> 24.2 | IMD2 |
|  | n41 | 2610 | 5 -> 10 | 25 -> 50 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |

MSD update for PC2 and PC1.5

PC2 and PC1.5 MSD were also updated accordingly based on the new PC3 MSD.

The LUT for HPUE MSD rule in [2] was applied here so that there is no need to calculate HPUE MSD case by case.

Table 5 PC2 MSD due to 2 UL IMD – 2DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 24.6 -> 23.3 | FDD | IMD3 |
|  | n41 | 2505 | 5 -> 10 | 25 -> 50 | 2505 | N/A | TDD | N/A |
| CA_n41-n71 | n41 | 2614 | 5 -> 10 | 25 -> 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 16.3 -> 22.2 | FDD | IMD4 |

Table 6 PC1.5 MSD due to 2 UL IMD – 2DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n41-n71 | n41 | 2614 | 5 -> 10 | 25 -> 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 25.4 -> 34.2 | FDD | IMD4 |

Table 7 PC2 MSD due to 2 UL IMD – 3DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n3-n18-n41 | n18 | N/A | 5 | N/A | 865 | 34.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 875 -> 865 | 28 -> 26.7 | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 -> 2585 | 5 -> 10 | 25 -> 50 | 2670 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 34.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
| CA_n3-n28-n41 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 32 -> 30.3 | FDD | IMD24 |
|  | n41 | 2510 | 5 -> 10 | 25 -> 50 | 2510 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 32 | FDD | IMD2 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 25.6 -> 25.1 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1840 | 25.1 -> 24.1 | FDD | IMD32 |
|  | n41 | 2620 | 5 -> 10 | 25 -> 50 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | N/A | 5 | N/A | 2640 | 13 -> 17.8 | TDD | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2570 | 5 -> 10 | 25 -> 50 | 2570 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 33 -> 35.6 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2630 | 32.6 -> 33.9 | TDD | IMD24 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 33.8 | FDD | IMD21,4 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 -> 10 | 25 -> 50 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 23.7 -> 22.6 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 -> 10 | 25 -> 50 | 2645 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3775 | 17.2 -> 15 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2640 | 18.8 -> 17.8 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 26.5 - 25.3 | FDD | IMD35 |
|  | n41 | 2675 | 5 -> 10 | 25 -> 50 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 20 -> 19.2 | FDD | IMD4 |
|  | n41 | 2550 | 5 -> 10 | 25 -> 50 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| CA_n28-n41-n77 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3323 | 34.2 -> 33.7 | TDD | IMD22,4 |
|  | n28 | N/A | 5 | N/A | 798 | 36.8 -> 36.2 | FDD | IMD21,4 |
|  | n41 | 2642 | 5 -> 10 | 25 -> 50 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2642 | 35.5 -> 34.8 | TDD | IMD24 |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n41-n66-n77 | n41 | 2600 | 5 -> 10 | 25 -> 50 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 25 -> 24 | TDD | IMD31,2 |
|  | n41 | N/A | 5 -> 10 | N/A | 2670 | 18.7 -> 17.2 | TDD | IMD55 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 | 5 -> 10 | 25 -> 50 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 20.5 -> 18.6 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 -> 10 | 25 -> 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 35.1 -> 34.6 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 -> 10 | 25 -> 50 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 25.2 -> 24.8 | TDD | IMD31 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3774 | 21.9 -> 21.4 | TDD | IMD41 |
|  | n41 | N/A | 5 -> 10 | N/A | 2615 | 34.7 -> 34 | TDD | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2564 | 24.4 -> 23.9 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 41 | 2680 | 5 -> 10 | 25 -> 50 | 2680 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 36.8 -> 36.2 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |

Table 8 PC1.5 MSD due to 2 UL IMD – 3DL/2UL (38.101-1)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 -> 10 | 25 -> 50 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 33.1 -> 31.6 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 -> 10 | 25 -> 50 | 2645 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3775 | 31.2 -> 29.8 | TDD | IMD5 |
| CA_n41-n66-n77 | n41 | 2600 | 5 -> 10 | 25 -> 50 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | [34.4] -> 33 | TDD | IMD31,2 |
|  | n41 | N/A | 5 -> 10 | N/A | 2670 | [23.4] -> 32 | TDD | IMD55 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 | 5 -> 10 | 25 -> 50 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 32 -> 30.4 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 -> 10 | 25 -> 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 39.8 -> 40.6 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 -> 10 | 25 -> 50 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 29.9 -> 33.8 | TDD | IMD31 |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3774 | 31.3 -> 33.4 | TDD | IMD41 |
|  | n41 | N/A | 5 -> 10 | N/A | 2615 | 39.4 -> 40 | TDD | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2564 | 29.1 -> 32.9 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |

Table 9 PC2 MSD due to 2 UL IMD – 2DL/2UL (38.101-3)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | IMD order |
| Configuration |  | (MHz) | (MHz) | LCRB |  | (dB) |  |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 16.3 -> 22.2 | IMD4 |
|  | n41 | 2618 | 10 | 50 | 2618 | N/A |  |

Table 10 PC2 MSD due to 2 UL IMD – 3DL/2UL (38.101-3)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | IMD order |
| Configuration |  | (MHz) | (MHz) | LCRB |  | (dB) |  |
| DC_2A-66A_n41A | 2 | N/A | 5 | N/A | 1940 | 22.6 -> 21 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 -> 10 | 25 -> 50 | 2685 | N/A | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 -> 10 | 25 -> 50 | 2510 | N/A | N/A |
|  | 28 | N/A | 5 | N/A | 790 | 32 -> 30.3 | IMD211 |
|  | 3 | N/A | 5 | N/A | 1832.5 | 32 | IMD2 |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_3A_n41A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2580 | 5 -> 10 | 25 -> 50 | 2580 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 25.6 -> 24.1 | IMD31 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | N/A | 5 -> 10 | N/A | 2640 | 13 -> 17.8 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |

Conclusion

In this contribution, we analyzed new MSD for 10MHz of n41 and proposed adopting the MSD table in draft CRs (R4-2513296 and R4-2513297) for 38.101-1 and 38.101-3.

Proposal 1: RAN4 to use MSD tables for 10MHz of n41 in draft CRs (R4-2513296 and R4-2513297) for 38.101-1 and 38.101-3.

References

[1] R4-2511779, “WF on optional channel bandwidth”, Skyworks Solutions, Inc., Huawei, Murata, 3GPP TSG RAN WG4 Meeting #116, Bangaluru, India, 25th – 29th Aug, 2025

[1] R4-2511752, “Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification”, Skyworks Solutions, Inc., Nokia, T-Mobile USA, ZTE, Oppo, Samsung, Murata Manufacturing Co Ltd., MediaTek, Vivo, Huawei, Apple, Qualcomm France, 3GPP TSG RAN WG4 Meeting #116, Bangaluru, India, 25th – 29th Aug, 2025