3GPP TSG-RAN WG4 Meeting#116bis		R4-2513301

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	8.10

Source: 	Spreadtrum, UNISOC

Title: 	Views on 5G and 6G spectrum sharing

Document for:	Discussion

Introduction

In RAN4#116 meeting, a Plan [1] about RAN4 6G guidance for information was provided by RAN4 chair. In this contribution, we want to provide some initial views 5G and 6G spectrum sharing.

6G topics; Scope; Scope in Q4’2025 and Q1’2026. System parameters; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. General RF and UE RF; non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues; non-spectrum and non-AI UE RF, spectrum aggregation，UE RF related coverage and energy efficiency. BS RF; BS RF, coexistence study, BS RF energy efficiency, duplexing, MSR; non-spectrum and non-AI BS RF, MSR, coexistence study. Spectrum; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey. RRM; RAN4 driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM related coverage and energy efficiency, other PHY signal/channel/procedure related RRM; RAN4 driven non-AI RRM topics,. Demod; RAN4 driven non-AI demod topics, MIMO operation, channel coding, multi TRP, demod related coverage and energy efficiency, other PHY signal/ channel/procedure related demod; RAN4 driven non-AI demod topics. AI; RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI; RAN4 driven use cases. 5G and 6G spectrum sharing; multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G; Topics, if identified, which are not RAN1/2 dependent. Sensing; PHY functions and procedure, RF, coexistence and testability; Topics, if identified, which are not RAN1/2 dependent. OTA and testability; Testability methodology framework and key assumptions, including both AI and non-AI features; Testability methodology framework and key assumptions, including both AI and non-AI features. RAN4 operation efficiency; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects.

Discussion

MRSS

In NR, the always-on common signals of 5G is reduced much compared with 4G, which reduce the complexity of 5G-6G MRSS design. So MRSS is simpler and more resource efficient than 4G-5G DSS. Therefore, to deploy 6GR in the existing NR bands, 5G-6R Multi-RAT Spectrum Sharing is an important migration mechanism from 5G to 6G. The detailed solutions for MRSS is discussed in RAN1. RAN1#122 meeting agreement as follow.

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

Currently, potential coordinated scheduling includes FDM, TDM and dynamic resource sharing between NR and 6GR. From RAN4 perspective, co-existence between 5G and 6G should be considered from these scheduling scenarios above. For the existing NR bands, there no explicit commercial scenario for FR2-1 bands, therefore we can focus on FR1 bands.

In addition, currently, some companies think that SSB signal should be designed independently between 5G and 6G for MRSS, and some companies think that SSB signal can be partial shared between 5G and 6G for MRSS. From our point of view, no matter how the SSB structure is designed, one single set of sync raster should be designed in 6G to follow the principle to simplify 6G system design.

Proposal 1: Co-existence between 5G and 6G for MRSS should be considered, we can focus on FR1 bands.

Proposal 2: Keep a single set of sync raster s in 6G with and without MRSS.

In order to more align the MRSS between 5G and 6G, channel raster needs to be considered. The motivation of finer channel raster is to make network deployment more flexible. For NR, the channel raster is 100 kHz due to the co-existence with LTE in re-farming bands. In order to address the asymmetric RB numbers between gNB and UE, NR also supports enhanced channel raster as 10 kHz in later release. According to the consensus in 6G, there is no need to consider co-existence with LTE and it just needs to consider the co-existence with NR. Regarding co-existence with NR, channel raster of 6G can be same with NR in re-farming bands. At the same time, considering the flexibility of network deployment. 6GR channel raster can be 10 kHz for the re-farming bands which is in NR can support 10 kHz.

Proposal 3: For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G.

For UL shift 7.5 KHz, the DC location of LTE is between two sub-carriers, the DC location of NR is within one sub-carrier. Regarding the co-existence between LTE and NR, NR defines the Δshift, Δshift = 0 kHz or 7.5 kHz for some bands, where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz. Now there is no need to consider the co-existence between LTE and NR, thus, it is no necessary for NR to perform UL shift 7.5 kHz. In our understanding, for 6GR, there is no need to define Δshift.

Proposal 4: There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G.

Conclusions

In this paper, we provide our initial views on MRSS, we have made the following proposals.

Proposal 1: Co-existence between 5G and 6G for MRSS should be considered, we can focus on FR1 bands.

Proposal 2: Keep a single set of sync raster s in 6G with and without MRSS.

Proposal 3: For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G.

Proposal 4: There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G.

- Reference

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair. TSG-RAN4#116 meeting