3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2513305

Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	Consideration on UE RF study for 6G

Agenda Item:	8.4

Document for:	Approval

Introduction

SI on 6G study was formally approved in RAN#108 meeting [1], and this is the first meeting in RAN4 to start the 6G study. This contribution provides our general consideration on UE RF study for 6G.

Discussion

Power class and HPUE framework

Potential issues in 5G NR

As seen from the 5G NR specifications, the power class dependent requirements include:

- configured Tx power that combines MOP, MPR and A-MPR requirements,

- ACLR,

- reference sensitivity (i.e., REFSENS, RSD, MSD, and etc.).

Generally, the MOP is the upper bound of the UE Tx power without tolerance. In Rel-18, the power boosting feature was introduced, which breaks the limit by 1dB for PC3 and 0.5dB for PC2. The gain comes mainly from the optimization of UE implementation. It’s worth noting that the ACLR as well as receiver sensitivity requirements after boosting are still kept the same as those for the original power class.

On the other hand, PC2 has been enabled in 3GPP specifications for many TDD and FDD bands, so is PC1.5 for TDD bands by the end of Rel-19. And PC1 has been introduced for some NTN bands. In Rel-20, PC1.5 for FDD bands and PC1/32dBm for TDD bands will be specified.

Observation 1-1: Current ways of specifications for the power boosting features are complex and time consuming compared to using higher power class, given that high power classes are enabled for more and more bands. A clean and simple way to apply power boosting should be considered in 6G.

Similar to power boosting, higherPowerLimit is a feature designed to break the MOP limit imposed by the BC power class for a band combination. When one of the component bands can support the same power class as the BC (e.g. PC2), the power class of the BC might become an improper limit on the total power of the ULCA, which could lead to unnecessary/unexpected drop of the SCell.

Observation 1-2: The BC power class requirement on inter-band ULCA could impose improper limit on the total output power. More suitable limit as well as appropriate power allocation for dual-UL transmissions need to be studied in 6G.

In contrast to power boosting, power class fallback is a mechanism to reduce the MOP by the duty-cycle based SAR solution. When it happens, all requirements for lower/default power class apply, including ACLR, MPR and etc. It can be viewed as sort of relaxation for UE implementation, since PC3 ACLR (30dB) is lower than that for PC2 (31dB). Moreover, it’s unclear whether adjusting the MOP in 3dB step-size is the best way to meet the SAR requirements.

Observation 1-3: It’s questionable whether power back-off with power class fallback is necessary.

Inherited from LTE, RB scheduling restrictions are used to meet certain UE coexistence requirements. The restrictions are targeted for PC3, and are defined either in NS flags or the long table of “Requirements for spurious emissions for UE co-existence”. For example, the 54 RB restriction for band n7 as defined in NS_46.

##### 6.5.3.3.25 Requirement for network signalling value "NS_46" When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. Table 6.5.3.3.25-1: Additional requirements for “NS_46” Protected band; Frequency range (MHz); Maximum Level (dBm); MBW (MHz); NOTE. Frequency range; 2570; -; 2575; +1.6; 5; 1, 2. Frequency range; 2575; -; 2595; -15.5; 5; 1, 2. Frequency range; 2595; -; 2620; -40; 1; 1. NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band..

In the course of introducing PC2 for band n1, RAN4 favored the usage of A-MPR than the use of RB scheduling restriction, since the RB scheduling restriction is much less flexible in terms of spectrum utilization than A-MPR, e.g., A-MPR approach may allow UE to transmit more RBs or RBs nearby spectrum close to victim radio system if the UE is close to BS. We, however, acknowledge that the A-MPR approach requires more time to specify the requirements and specifying all the A-MPR for all the 6GR bands from 6GR day 1 is challenging. Therefore, RAN4 needs to find a way to accommodate the amount of work in advance such as granularity of A-MPR side conditions etc.

Observation 1-4: It’s better to avoid RB scheduling restrictions in order to meet additional spectrum emission (ASE) requirements for UE co-existence. Instead, the A-MPR approach is preferred for all power classes for the sake of consistency and scheduling flexibility.

Observation 1-5: It is recommended that RAN4 finds a way on how to deliver all the A-MPR for all the bands with ASE for all the power classes from 6GR day 1.

In RAN4, there have been prolonged discussions on the indication of power class capabilities (i.e., ue-powerClass, powerClass for BC and ue-powerClassPerBandPerBC) as well as the applicable requirements for band combinations. Unfortunately, ambiguity on the applicable power class capability (signaling) remains at least for the following cases:

- Single configured UL CC with DL CA,

- Dual UL or single UL transmission as controlled by PDCCH scheduling,

- Tx switching between different bands and LB CA switching,

- Per CC Power Class for intra band UL CA with two PAs depending on CCs distance and/or usage of UL MIMO

Observation 1-6: The applicable power class requirements and the corresponding signalling need to be clearly defined in 6G for single/dual UL transmissions in the context of band combination and/or Tx switching.

Since Rel-17, lack of MSD requirements usually blocks a band combination from supporting high power class(es) in the UL if the MSD requirements have not been studied and specified. However, the MSD requirements are usually derived on loose RF assumptions and most often do not impose substantial constraints on developing UE with MSD for default or lower power class. An exception has been introduced for the case of single CC UL with DL CA by a dedicated RAN task, in which the MSD for default power class is verified if MSD for higher power classes is not defined yet. But no exceptions for other cases.

Observation 1-7: It’s questionable whether MSD requirements should continue to be the blocking factor for introducing high power class(es) for a band combination if the MSD for the default power class has already been defined.

RAN1 has had the first meeting for the 6G SI. There’s one phrase that frequently appeared in the agreements, i.e., “study and identify the lessons learned from NR”. We have tried to bring up some potential issues in the above observations from UE RF perspective. In the meantime, we think RAN4 as a group can do the same exercise as RAN1.

Excerpts from RAN1#122 Chairman’s notes Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS Agreement - Study and identify the lessons learned from NR BWP framework Agreement - Study and identify the lessons learned from NR spectrum utilization and aggregation framework - DC is subject to RANP decision in June 2026 - Note: MRSS aspects are separate discussion Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different Agreement - Study and identify the lessons learned from NR duplex modes - On 6GR duplexing study, RAN1 considers at least following duplex types - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD - Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded Agreement - For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

Proposal 1-1: Study and identify lessons learned from 5G NR, and remove unnecessary RF requirements/features when moving towards 6GR specifications. The potential issues to be discussed include:

- A clean and simple way to apply power boosting

- Suitable power limit for inter-band ULCA

- whether power class fallback is necessary

- unified approach (A-MPR vs RB scheduling restrictions) for meeting ASE requirements for UE co-existence

- how to deliver all the A-MPR for all the bands with ASE for all the power classes from 6GR day 1

- power class ambiguity for single/dual UL transmissions in the context of band combination and/or Tx switching, including the applicable power class capability signaling for at least the following cases:

  - Single configured UL CC with DL CA

  - Dual UL or single UL transmission as controlled by PDCCH scheduling

  - Tx switching between different bands and LB CA switching

  - Per CC Power Class for intra band UL CA with two PAs depending on CCs distance and/or usage of UL MIMO

Key technical directions

By the end of Rel-20, many NR bands will be able to support PC1.5 and even PC1 or higher power classes. Hence, at the start of 6G WI, high power UEs would become the mainstream. It’s worth reconsidering the default power class for each operating bands, rather than reuse PC3 as in NR. For example, PC2 could be the default power class for TDD bands. Given the larger propagation loss and penetration loss at higher frequencies, higher power class could help improve the UL coverage.

Proposal 1-2: Study the default power class for FDD and TDD bands for 6G, considering the HPUE progress in 5G NR.

In previous meetings, there were discussions on how to maximize the UL transmission power capability for inter-band CA, depending on RRC configuration, activation/deactivation, and/or dynamic scheduling. No conclusions were reached due to lack of time. In 6G SI, such effort can be resumed.

Proposal 1-3: Study how to maximize the UL transmission power capability for CA, subject to RRC configuration, activation/deactivation, and/or dynamic scheduling. The corresponding signaling as well as applicable requirements need to be clearly defined.

The above discussion is mainly from FR1 perspective. The FR2 power class definition in 6G should also take the lessons learned from 5G into account to promote FR2 industry success in 6G considering so many valuable FR2 spectrum available for many operators. Only FR2 PC3 was introduced for handheld UE in 5G, but different power classes are introduced for non-handheld UE considering different scenarios as shown below.

Table 6.2.1.0-1: Assumption of UE Types [TS 38.101-2] UE Power class; UE type. 1; Fixed wireless access (FWA) UE. 2; Vehicular UE. 3; Handheld UE. 4; High power non-handheld UE. 5; Fixed wireless access (FWA) UE. 6; High Speed Train Roof-Mounted UE. 7; RedCap UE. Note: Any power class can be used for Redcap type devices as long as the device can meet the core requirements that are applicable to Redcap devices as defined in clause 4.2.21.1 from TS38.306 [14]. Power class 3 is default power class.

Extensive discussions have been held in 5G on introducing a new one for low-cost handheld UEs, in an effort to counter FR2's limited adoption and low market share where we proposed that the main reason for FR2 not being popular is UE implementation complexity and power consumption to accommodate different scenarios [2].

Proposal 1-4: FR2 power classes defined in 5G should not be inherited by 6G. Issues identified together with scenarios such as hot spot in 5G regarding FR2 power classes should be considered from the very beginning.

MPR/A-MPR assumptions

MPR/A-MPR Potential issues in 5G

MPR studies, and evaluations started from NR Adhoc #2 meeting [3]. The MPR evaluations for sub-6GHz (FR1) and mmwave (FR2) frequency ranges, were designed and performed. In the following sections some of the technical challenges, which were faced during 5G, are explained.

MPR for fixed CBW and flexible CBW

When the CBW configuration for NR was not agreed, there were discussions about MPR evaluations for fixed and flexible CBWs. In [4] it was shown that for 25RB allocation the flexible CBW would have much lower out-of-band emission than a fixed CBW, which could reduce the MPR and A-MPR values in a real world deployment.

Another proposal was to use a linear interpolation (in function of the RB allocations) for MPR if flexible CBW is used. On the other hand for A-MPR as it is based on regulations of different regions, it was proposed to keep it in a case by case form [5]. However, as 5G was specified with a fixed CBW, such MPR reduction measures were not exploited.

Power classes

The MPR/A-MPR evaluations depend on the UE power classes. Initially PC3 (23dBm) and PC2 (26dBm) were considered for 5G. Later on, other power classes were added. In Table 2-1, all the PCs, till Rel-19, are summarized.

Table 2-1 Summary of the power classes in 5G NR till Rel-19

| Power Class | MOP FR1 (Sub-6 GHz) (dBm) (max TRP) | MOP FR2 (mmWave) (dBm) (max TRP) | Typical Use Cases |
| --- | --- | --- | --- |
| 1 | 31 | 25-35 | Fixed wireless access (FWA) UE, etc |
| 1.5 | 29 | N.A. | Handheld UE, FWA UE |
| 2 | 26 | 23 | Handheld UE,Vehicular UE, (e)Redcap, etc |
| 3 | 23 | 23 | Handheld UE, (e)Redcap, etc |
| 4 | N.A. | 23 | High power non-handheld UE, etc |
| 5 | 20 | 23 | Fixed wireless access (FWA) UE, etc |
| 6 | N.A. | 23 | High Speed Train Roof-Mounted UE, etc |
| 7 | N.A. | 23 | RedCap UE |

As a rule of thumb, the higher the MOP of a power classes the more MPR they require, because of their out-of-band emissions, However, some of the PCs can support better ACLR and other enhanced parameters which could improve their MPR values.

Tx linearization: ACLR, SEM, EVM evaluations

ACLR Evaluation:

The ACLR has a direct impact on MPR/A-MPR values. The stricter the ACLR, the more power back off is needed. Two types of ACLR were specified for NR. First, the ACLR which is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. Second, UTRAACLR is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency. The two ACLRs were agreed for PC3 and PC2 on WF [6] and their details can be found in section 6.5.2.4 in TS 38.101-1 and 6.5.2.3 in TS 38.101-2.

SEM Evaluation:

In LTE general SEM or LTE CA general SEM the emission requirement for the first MHz outside the channel edge was scaled proportionately to the channel bandwidth. Logic behind this is that as transmission bandwidth gets larger the PSD of fully populated channel gets lower and UE can meet the tighter requirement. As in 5G, larger CBWs are considered, in some cases such as 1RB edge allocation, the IMD3 can hit the LTE scaled SEM limits. Therefore, for 5G it was agreed to set the first MHz (ΔfOOB = ± 0-1) for the CBW>40MHz to a -24dBm (with MBW=30KHz). For other SEM regions (ΔfOOB > ± 0-1) a similar approach was considered, as can be observed in WF [6]. Changes in SEM limits as a major factor for MPR, can significantly modify the MPR values, however it would need re-evaluation of the regulatory requirements.

EVM evaluations:

In MPR evaluations, the EVM is directly related to the modulation order. For instance, for 64QAM and 256QAM, the EVM are usually the gating factor, while ACLR and SEM are the gating factor on lower modulation order. In 5G it was decided to reuse the same EVM percentages as LTE and reuse those values for FR2 as well, as shown in Table 2-2.

Table 2-2 EVM percentage for different modulation orders [6], [7]

| Modulation | EVM |
| --- | --- |
| Pi/2 BPSK | 35% |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

As the quality of the RF equipment has improved since LTE, the newer products are expected to have a relaxed EVM % per modulation order than mentioned in Table 2, and the MPR should be reduced for 6G. Some methods to enhance the EVM requirements are explained in sections 2.4.

Single Carrier Vs CA

The MPR and A-MPR evaluations are also performed for the case of carrier aggregation. For inter band CA, the same MPR as single band CA is considered. For intra-band CA multiple scenarios have been considered for a given power class.

- Intra-band contiguous CA with contiguous allocation.

- Intra-band contiguous CA with non-contiguous allocations

- Intra-band non-contiguous allocations

The intra-band CA MPR evaluations can be very resource consuming because of the so many RB allocations that can be considered. On the other hand, the specification is quite complicated as they different resource allocation configurations have been specified with different MPR values. For example, for intra-band contiguous CA with non-contiguous allocations Inner, Outer 1 and Outer 2 allocations have been specified in section 6.2A.2.2 of TS 38.101-1, which is a very complex design.

MPR/A-MPR studies for 6G

One of the objectives of 6G is enhancing the UE coverage. Therefore, there is a momentum to reduce the Tx MPR/A-MPR values, where it is possible. In 5G, the following Tx parameters were considered (actually they were taken from 4G era [8]) in MPR evaluations, for PC3 for example:

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

For 6G, it would be wise to revisit these assumptions to reflect todays technologies. In the following sections different solutions to reduce MPR values will be discussed to be considered for 6G SI phase.

ACLR Relaxation

During Rel-17, TR 38.921, for FS_NR_IMT_param, was agreed to specify 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3. Even though the ACLR value is not reflected in the TS 38.101-1, but it shows that a review on relaxation of ACLR requirements could be performed on other bands, if regulations allow it .

As ACLR is one of the contributing factor in MPR evaluations, its relaxation could alleviate the Tx requirements, which would lead to MPR reduction. In [9], it was shown that an ACLR of 26dB (instead of 30) would reduce MPR up to 1 dB and 1.5 dB for QPSK DFT-s-OFDM and CP-OFDM, respectively.

Proposal 2-1: Study the UE ACLR requirements based on the current and expected regulation updates, to relax the ACLR assumptions and enhance the MPR values.

Tx impairments improvements

Improving the Tx impairements could result in reducing the MPR for sub-6GHz frequency range. Improving the carrier leakage, image rejection, CIM3 and CIM5 are the main candidates. For example, for NS_05, it was shown that for n1 PHS protection, a carrier leakage and an image rejection of -34 dB (instead of the common -28 dB value) should be used to meet EVM requirements for 256QAM. However, RAN considered any update on the general evaluation assumptions for PC3, could have a major impact on the 5G specifications, hence it was not agreed by them, which leaves the door open for 6G SI phase. On the other hand, for the new frequency ranges (around 7 GHz) a study would be needed to set reasonable assumption based on the available technologies.

Proposal 2-2: Study the UE Tx impairments requirements (carrier leakage, image rejection, CIM3 and CIM5) for sub 6GHz, to ensure that the MPR assumptions are a reflection of the recent technologies.

Proposal 2-3: Study the UE Tx impairments requirements (carrier leakage, image rejection, CIM3 , CIM5, etc.) for frequencies higher than 6GHz, especially, around 7GHz.

Specifying MPR and AMPR for carrier aggregation

In 5G lots of effort was spent on evaluating the MPR and AMPR values for different power classes such as PC1.5 intra-band CA in Rel-19, for dual PA and Tx-Div RF architecture. However it was later revealed that the gain of such a scenario comparing to PC2 was very limited, if not negligible. Therefore a study on which CA configuration would really be beneficial based on MPR and A-MPR performances would be needed.

Proposal 2-4: Study the benefits of different power classes for different carrier aggregation scenarios ( such as intra-band contiguous and noncontiguous allocations) ensure that the specifying reasonable MPR and A-MPR and other RF requirement.

PA modeling

PA models in 5G

PA is the most contributor to Tx nonlinearities and in-band and out-of-band emissions. On the SI phase of 5G, companies proposed different PA models to be used in 5G evaluations, such as waveform evaluations for RAN1 [10], MPR/A-MPR, spectrum efficiency evaluations within RAN4. The details of the PA models are available in TR 38.803 Annex A, where memory-less and memory-based models were proposed. The memory less models such as Rapp, Saleh and Ghorbani, are suitable for class A, AB and C amplifiers. For high efficiency PAs (such as Doherty) more sophisticated models such Volterra, its variants are needed. These models can include the memory effect, meaning that the output of the PA depends not only on its input, but also on its previous input values.

During MPR/A-MPR studies, companies were free to use any PA model that they deemed to be fit however companies focused on memory-less PA models. The downside of such a simplification is that the dynamic out-of-band spectrums, generated in real PAs due to memory effect are excluded from evaluations, which can have an impact on the cell edge UEs of the operators and make them out of service.

PA models for 6G study

The new models for different frequency ranges, large CBWs and different power classes, will help 3GPP having a proper estimation of the required MPR, otherwise we would end up with the similar over estimated MPR values that 3GPP faced during 5G. It should be mentioned that with the models based on new PAs, we need to focus on memory based poylynomial models and do not settle down on memeory-less models. Otherwise we will under-estimate the PA nonlinearities and the dynamic nonlinearities related to memory effect will be neglected, especially for large CBWs. On the other hand the calibration conditions (calibration point) need to be updated as the characteristics of the PA on wide CBWs (such as 200MHz) is not same as 20MHz.

In [11] a new memory based model was presented for a PC2 PA to be used in 6G study phase. The model was based on memory polynomial model, where only odd coefficients were taken into account as the even nonlinear orders can be suppressed with RF filters. After estimating the model coefficients, with the test bench and methodology explained in [11], different models (for K=9 and M = {0,…8}) were obtained. To be able to characterize the models, the Normalized Mean Squared Error (NMSE) metric, was used to evaluate how close the model outputs are to the measured PA output.

The coefficients were estimated based on a PC2 PA with a 100 MHz DFT-s-OFDM waveform at a center frequency of 3.5GHz, while the PA was calibrated to pass the 3GPP Tx linearity requirements. Figure 3-1 depicts the PSD of the measured PA output and the output of the models for different memory depths for both central frequencies. One can observe that for the memoryless case, the NMSE = -21.6 dB and the more memory depth is included, the more precise the model becomes (i.e., the NMSE decreases). On the other hand, the PSD plot also shows that a quite large gap between the out-of-band emissions of the measured PA and a memoryless model, while the gap is much smaller when the memory effect is taken into account. It is also worth mentioning that there is a PSD asymmetry in the measured transmission CBW (the PSD on the left side is slightly higher than the right side), which is also reflected when memory effects are taken into account. However, the memoryless model produces a symmetrical PSD across the transmission.

.

Fig. 3-1 PSD plot of the PA measurement and PA models with their corresponding NMSE evaluations

In Fig. 3-2 the impact of the dynamic nonlinearities (consequence of the memory effect) can be observed, where for a given output power the memory-less model (M=0) passes the SEM requirements but the memory-based model (M=6) does not pass it.

Fig. 3-2 SEM requirements for the Tx, Fc= 3.5GHz: Memory-less model (SEM pass) and memory-based model (SEM does not pass)

To better observe the impact dynamic out-of-band emissions, MPR evaluations were made for CP-OFM waveforms, which can be observed in Fig. 3-3. Fig. 3-3 (a) and (b) depict that when memory effects are taken into account, for a CP-OFDM waveform, more back-offs are required (∆MPR=3 dB). Using Fig. 3-3 (c) and (d) one can realize that the increase of back-off is due to the SEM limitations, as we had observed in Fig. 3-2, too.

Fig. 3-3 MPR evaluations for CP-OFDM waveform over all possible RB allocations Fc= 3.5GHz: (a) required power back-off when M=0, (b) required power back-off when M=6, (c) power back-off gating factor when M=0, (d) power back-off gating factor when M=6.

Observation 3-1: PA models are essential for RAN4 and RAN1 waveform evaluations, MPR/AMPR studies, spectrum efficiency and etc.

Proposal 3-1: Study new PA models for different power classes, frequency ranges and CBWs as PAs behave differently depending on each of these parameters.

Proposal 3-2: Memory-based PA models need to be considered for 6G study phase and eventually the WI phase, as in 6G 3GPP is seeking to include large CBWs (at least 200 MHz).

UL enhancements

Overview of NR power enhancement

3GPP have studied plenty of techniques targeting UL enhancement e.g. in power or waveform domain. Back in previous releases, RAN4 focused on MPR reduction and power boosting for low order modulation waveform. All standardized solutions are listed in below table.

Table 4-1 RAN4 UL enhancement solutions

| Release | 15 | 18 | 19 |
| --- | --- | --- | --- |
| Solution | FDSS-based power boosting | Power boosting | MPR reduction based on OOB relaxation |
| Applicable cases | BPSK | BPSK QPSK | QPSK 16QAM |
|  | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM and CP-OFDM |
|  | All RB regions | Inner region | Outer region |
|  | PC3 | PC3 and PC2 | PC3 and PC2 |

In Rel-20, HPUE is going to be further extended, including TDD to support 4Tx for CPE/FWA UE with 4×26dBm PA, PC1.5 with 2Tx for FDD, PC1.5 1Tx for TDD for FWA UE. As 6G is going to introduce new frequency bands, the basic of HPUE is unlikely to be changed dramatically. Hence apart from HPUE, it is beneficial to study further UL enhancement based on existing power classes and feasible techniques.

Observation 4-1: Apart from HPUE, uplink enhancement based on variant of techniques is a long-lasting topic in 5G NR, which must still be a worthy of prioritized study in 6G.

Potential scenario for further UL enhancement

To figure out if there are any other scenarios fit for UL enhancement, we did a set of system level simulations as shown in Figure 4-1. Marker “Indoor” represents the UE distribution as 80% indoor and 20% outdoor, while marker “Outdoor” represents the UE distribution as 20% indoor and 80% outdoor. Simulation results indicate an interesting trend that high-order modulation scheduling like 64QAM and 256QAM can reach 20%-35% for those UEs whose scheduled output power are above 20dBm.

Fig. 4-1 System level simulation result on the scheduling probability distribution of modulation order under different transmitted power level

Based on measurements collected from real deployments in dense urban, a cross-check on the above finding is conducted. We can observe that for those PC2 UEs, the scheduling decision tends to be allocating Inner RB allocations with high modulation orders.

Table 4-2 Live-network data collection on modulation order and RB allocation distribution

| Waveform | Modulation | n41 |  | n78 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner |
| CP-OFDM | QPSK | 2.1% | 8.1% | 1.3% | 3.7% |
|  | 16QAM | 1.6% | 9.5% | 1.4% | 19.7% |
|  | 64QAM | 3.6% | 45.0% | 3.0% | 45.4% |
|  | 256QAM | 2.5% | 26.3% | 2.7% | 21.6% |

It can be observed that:

Observation 4-2: Based on simulation and measurement in real dense urban deployment, it is evident that the scheduling decision tends to be Inner RB allocations with high modulation orders i.e. 64QAM and 256QAM in CP-OFDM waveform under high transmitted power, which can be target scenario for UL enhancement.

Power enhancement for high-order modulation

Bottleneck

Figure 4-2 depicts the gating factor for meeting MPR requirements with different RB allocations under 64QAM and 256QAM and CP-OFDM waveform based on measurements for a PC2 UE. The results show that EVM would be the gating factor for the whole Inner RB region when meeting MPR requirement. Consequently, relaxation on Tx EVM is the study direction for the identified scenario.

(a) 64QAM                           (b) 256QAM

Fig. 4-2 Gating factors for meeting PC2 MPR requirements

Observation 4-3: For the target UL enhancement scenario i.e. Inner RB region with 64QAM and 256QAM in CP-OFDM waveform, Tx EVM is the bottleneck for meeting MPR requirements.

Relaxed UE Tx EVM and advanced BS receiver

As per NR Rel-20 New WID [12] captures the below note, both 5G-Adv and 6G should study MPR reduction with the relaxed Tx EVM requirements:

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

As the power increases, EVM deterioration caused by the PA nonlinearity becomes dominant. To alleviate such distortion, transmitter side we can consider digital pre-distortion (DPD). However, DPD somehow is still not enough for improving the whole system performance. Hence we can consider to involve BS compensation by using digital post-distortion (DPoD). From UE perspective, EVM relaxation can further increase the output power and reduce power consumption at the same time. Because UE is allowed to pull up its output power as the PA can be operating closer to saturation region. Also, as the PA efficiency becomes higher, its operating voltage can be reduced for a given average output power. From BS perspective, Digital post-distortion (DPoD) is genuinely useful considering that the information on distortion is easy to get from reference signal (although it is mixed with channel characteristics). One of the DPoD methods proposed in [13] is called reconstruction of distorted signals (RODS), in which the transmitted signal is reconstructed utilizing the estimated information on PA model and channel. Exemplary DPoD procedure is shown in Figure 4-3, where the PA model is estimated by the NR reference signal e.g., DMRS.

Fig. 4-3 Exemplary DPoD procedure

Evaluation methodology and assumptions

Firstly, we think one boundary between 5G-Adv and 6G part of this study should be calrified. As we can see from the RAN4 6G topic planning in [14], the scope of genearal RF and UE RF topic precludes AI objective. So we have the following proposal.

Proposal 4-1: According to the scope of each RAN4 6G topic, clarify that non-AI based Tx EVM relaxation study towards 5G-Advanced is handled under “General RF and UE RF” topic and whether AI-based Tx EVM relaxation study towards 6G can be handled under “AI” topic.

For 6G part, in order to comprehensively evaluate AI-based solution which may perform better when handling complex task, RAN4 should provide neccesary inputs and examine the target cases and evaluation assumptions. For instance, 6G entitles RAN4 the opportunities to look at new PA model, waveform and other factors that could lead to new MPR requirements in specific bands. However, the situation where RAN1 and RAN4 discussion mutually impact on each other should be avoided, as the whole picture of physical layer design together with how deep AI could participate into transmitter and/or reciever are still under study in RAN1.

Proposal 4-2: Adopt following principles for 6G part evaluation.

- RAN4 should refrain from adopting evaluation assumptions that will impact RAN1 ongoing discussion on physical layer design, e.g. DMRS and waveform. Wait for all related RAN1 discussion outcomes;

- RAN4 focus e.g. new PA model and/or other factors that could lead to new MPR requirements can be considered.

For 5G-Adv part, It is deemed neccesary for RAN4 to streamline the evaluation methodology and assumptions for such a feature that impacts both UE transmitter and BS reciever. For evaluation methodology, we think link level simulation should be adopted considering that RAN4 needs to dive into UE Tx RF impairments and DPoD algorithm at BS to leverage the demodulation performance eventually. We propose following contents for consideration.

Proposal 4-3: Adopt following link level simulation assumptions for 5G-Adv part evaluation.

- Existing UE RF requirements except EVM remain the same;

- 5G NR waveform and reference signal design;

- PA model that had been studied in 5G NR;

- Non-AI based DPoD algorithm in BS;

- Main parameters can be found in below table, others can be added if necessary.

|  | Parameter | Value |
| --- | --- | --- |
| UE | UE power class | PC2 |
|  | MCS | 17 for 64QAM and 23 for 256QAM (Table 5.1.3.1-2 in TS 38.214) |
|  | Waveform | CP-OFDM |
|  | RB allocation | Inner RB allocations |
|  | PA model | Polynomial model e.g. R4-164542 |
|  | Tx number | Both 1 and 2 can be considered |
|  | Rank | Both 1 and 2 can be considered |
|  | DMRS type | Type 1 |
| BS | Rx number | 64 |
|  | Carrier frequency | 2.6 GHz |
|  | SCS | 30 kHz |
|  | Channel model | TDL-D |
|  | Channel estimation | Real channel estimation based on DMRS |
|  | Reciever | MMSE-IRC DPoD is enabled (e.g. algorithm in [13]) |

Further, the LLS simulation can be broken down into following steps.

Proposal 4-4: Adopt following link level simulation process.

- Step 1 (baseline):

  - At transmitter, adjust the output power to get the minimum MPR i.e. MPR_0 for 64QAM and MPR_1 for 256QAM that meets all the existing UE RF requirements.

  - At receiver, get the SNR i.e. SNR_0 dB for 64QAM and SNR_1 dB for 256QAM corresponding to 10% BLER.

- Step 2:

  - At transmitter, adjust the output power to get the minimum MPR i.e. MPR_2 for 64QAM and MPR_3 for 256QAM that meets the relaxed EVM and all the other existing UE RF requirements.

  - At receiver, get the SNR i.e. SNR_2 dB for 64QAM and SNR_3 dB for 256QAM corresponding to 10% BLER.

- The net gain is calculated as (MPR_0 - MPR_2 + SNR_0 – SNR_2) and (MPR_1 - MPR_3 + SNR_1 – SNR_3) MPR gain for 64QAM and 256QAM, respectively.

As we propose no degradation on demodulation performance as part of the evaluation metric, we can anticipate the introduction of proper BS demodulaiton requirement and corresponding testing in the end.

Proposal 4-5: RAN4 can study on proper BS demodulation requirement and corresponding testing by assuming there is no demodulation performance degradation with relaxed UE Tx EVM.

Bearing the above simulation process in mind, there are some cases required investigation. The most straightforward case would be for single UE where different Tx EVM relaxation levels can be evaluated, so that how resilient a DPoD algorithm is could be verified and based on that RAN4 can conclude a reasonable range on relaxed EVM to accommodate different UE implementations. Since MU case is equally important as SU from network pint of view, RAN4 can check the gain when there are multiple UEs schedulded in FDM or SDM manner. Therefore we have the following proposal.

Proposal 4-6: RAN4 study can focus on following cases.

- Case #1 (SU): evaluate the MPR gain under relaxed Tx EVM when single UE is scheduled.

  - E.g. a range of relaxed EVM can be considered for 64QAM and 256QAM.

- Case #2 (MU): evaluate the MPR gain under relaxed Tx EVM when multiple UEs are scheduled.

  - E.g. a range of relaxed EVM can be considered for 64QAM and 256QAM.

  - UEs can apply the same or different relaxed EVM level.

  - UEs can be scheduled in FDM or SDM manner.

To facilitate future discussion, we suggest following plan to better utilize the time before the checking point in next year.

Proposal 4-7: RAN4 to calibrate link level simulation based on following case.

- Adopt all the simulation assumptions in Proposal 4-3 and collect simulation results on the Step 1 in Proposal 4-4.

Proposal 4-8: Suggest following working steps for 5G-Advanced Tx EVM relaxation study for the time before 2026 September.

- Step 1: Align the simulation assumptions and methodology, initialize calibration;

- Step 2: Complete calibration, further check on the simulation assumptions and concerned cases;

- Step 3: Simulation results collection, identify impact on UE RF requirements or other aspects.

Power enhancement for low-order modulation

RAN1 has already concluded that existing NR waveforms and modulation are supported as the basis for 6GR which needs the joint study by RAN1 and RAN4 as defined by the SID.

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - … Agreement (first agreement for 6G!!) CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Observation 4-4: QPSK is supported as basis for study of both CP-OFDM and DFT-s-OFDM for data channel of 6GR UL as per RAN1 agreement. Enhancement/modifications on CP-OFDM/DFT-s-OFDM is studied in RAN1.

One key strategy to improve uplink coverage is to reduce the PAPR for lower-order modulation schemes. High PAPR forces the power amplifier to operate with a larger MPR, leading to lower efficiency. By employing PAPR reduction techniques, such as FDSS applied for DFT-s-OFDM proposed in RAN1, the uplink signal can be transmitted at a higher average power for the same peak power constraint, thereby extending the coverage range and enhancing the link budget for devices at the cell edge. Apart from the DFT-s-OFDM enhancement with low PAPR waveform, CP-OFDM for low-order modulation is also commonly applied in commercial networks, because it offers more scheduling flexibility, without the constraints tied to the specific number of RB (e.g., 2, 3, or 5 RBs) that DFT-s-OFDM can be subject to.

Proposal 4-9: RAN4 study PAPR reduction techniques for low-order modulations, including but not limited to RAN1-related mechanisms. The study on techniques transparent to RAN1 can proceed independently, whereas the part relevant to RAN1 waveforms awaits further inputs from RAN1.

Conclusion

This contribution provides our general considerations on UE RF study for 6G.

Power class and HPUE framework

Observation 1-1: Current ways of specifications for the power boosting features are complex and time consuming compared to using higher power class, given that high power classes are enabled for more and more bands. A clean and simple way to apply power boosting should be considered in 6G.

Observation 1-2: The BC power class requirement on inter-band ULCA could impose improper limit on the total output power. More suitable limit as well as appropriate power allocation for dual-UL transmissions need to be studied in 6G.

Observation 1-3: It’s questionable whether power back-off with power class fallback is necessary.

Observation 1-4: It’s better to avoid RB scheduling restrictions in order to meet additional spectrum emission (ASE) requirements for UE co-existence. Instead, the A-MPR approach is preferred for all power classes for the sake of consistency and scheduling flexibility.

Observation 1-5: It is recommended that RAN4 finds a way on how to deliver all the A-MPR for all the bands with ASE for all the power classes from 6GR day 1.

Observation 1-6: The applicable power class requirements and the corresponding signalling need to be clearly defined in 6G for single/dual UL transmissions in the context of band combination and/or Tx switching.

Observation 1-7: It’s questionable whether MSD requirements should continue to be the blocking factor for introducing high power class(es) for a band combination if the MSD for the default power class has already been defined.

Proposal 1-1: Study and identify lessons learned from 5G NR, and remove unnecessary RF requirements/features when moving towards 6GR specifications. The potential issues to be discussed include:

- A clean and simple way to apply power boosting

- Suitable power limit for inter-band ULCA

- whether power class fallback is necessary

- unified approach (A-MPR vs RB scheduling restrictions) for meeting ASE requirements for UE co-existence

- how to deliver all the A-MPR for all the bands with ASE for all the power classes from 6GR day 1

- power class ambiguity for single/dual UL transmissions in the context of band combination and/or Tx switching, including the applicable power class capability signaling for at least the following cases:

  - Single configured UL CC with DL CA

  - Dual UL or single UL transmission as controlled by PDCCH scheduling

  - Tx switching between different bands and LB CA switching

  - Per CC Power Class for intra band UL CA with two PAs depending on CCs distance and/or usage of UL MIMO

Proposal 1-2: Study the default power class for FDD and TDD bands for 6G, considering the HPUE progress in 5G NR.

Proposal 1-3: Study how to maximize the UL transmission power capability for CA, subject to RRC configuration, activation/deactivation, and/or dynamic scheduling. The corresponding signaling as well as applicable requirements need to be clearly defined.

Proposal 1-4: FR2 power classes defined in 5G should not be inherited by 6G. Issues identified together with scenarios such as hot spot in 5G regarding FR2 power classes should be considered from the very beginning.

MPR/A-MPR assumptions

Proposal 2-1: Study the UE ACLR requirements based on the current and expected regulation updates, to relax the ACLR assumptions and enhance the MPR values.

Proposal 2-2: Study the UE Tx impairments requirements (carrier leakage, image rejection, CIM3 and CIM5) for sub 6GHz, to ensure that the MPR assumptions are a reflection of the recent technologies.

Proposal 2-3: Study the UE Tx impairments requirements (carrier leakage, image rejection, CIM3 , CIM5, etc.) for frequencies higher than 6GHz, especially, around 7GHz.

Proposal 2-4: Study the benefits of different power classes for different carrier aggregation scenarios ( such as intra-band contiguous and noncontiguous allocations) ensure that the specifying reasonable MPR and A-MPR and other RF requirement.

PA modeling

Observation 3-1: PA models are essential for RAN4 and RAN1 waveform evaluations, MPR/AMPR studies, spectrum efficiency and etc.

Proposal 3-1: Study new PA models for different power classes, frequency ranges and CBWs as PAs behave differently depending on each of these parameters.

Proposal 3-2: Memory-based PA models need to be considered for 6G study phase and eventually the WI phase, as in 6G 3GPP is seeking to include large CBWs (at least 200 MHz).

UL enhancements

Observation 4-1: Apart from HPUE, uplink enhancement based on variant of techniques is a long-lasting topic in 5G NR, which must still be a worthy of prioritized study in 6G.

Observation 4-2: Based on simulation and measurement in real dense urban deployment, it is evident that the scheduling decision tends to be Inner RB allocations with high modulation orders i.e. 64QAM and 256QAM in CP-OFDM waveform under high transmitted power, which can be target scenario for UL enhancement.

Observation 4-3: For the target UL enhancement scenario i.e. Inner RB region with 64QAM and 256QAM in CP-OFDM waveform, Tx EVM is the bottleneck for meeting MPR requirements.

Proposal 4-1: According to the scope of each RAN4 6G topic, clarify that non-AI based Tx EVM relaxation study towards 5G-Advanced is handled under “General RF and UE RF” topic and whether AI-based Tx EVM relaxation study towards 6G can be handled under “AI” topic.

Proposal 4-2: Adopt following principles for 6G part evaluation.

- RAN4 should refrain from adopting evaluation assumptions that will impact RAN1 ongoing discussion on physical layer design, e.g. DMRS and waveform. Wait for all related RAN1 discussion outcomes;

- RAN4 focus e.g. new PA model and/or other factors that could lead to new MPR requirements can be considered.

Proposal 4-3: Adopt following link level simulation assumptions for 5G-Adv part evaluation.

- Existing UE RF requirements except EVM remain the same;

- 5G NR waveform and reference signal design;

- PA model that had been studied in 5G NR;

- Non-AI based DPoD algorithm in BS;

- Main parameters can be found in below table, others can be added if necessary.

|  | Parameter | Value |
| --- | --- | --- |
| UE | UE power class | PC2 |
|  | MCS | 17 for 64QAM and 23 for 256QAM (Table 5.1.3.1-2 in TS 38.214) |
|  | Waveform | CP-OFDM |
|  | RB allocation | Inner RB allocations |
|  | PA model | Polynomial model e.g. R4-164542 |
|  | Tx number | Both 1 and 2 can be considered |
|  | Rank | Both 1 and 2 can be considered |
|  | DMRS type | Type 1 |
| BS | Rx number | 64 |
|  | Carrier frequency | 2.6 GHz |
|  | SCS | 30 kHz |
|  | Channel model | TDL-D |
|  | Channel estimation | Real channel estimation based on DMRS |
|  | Reciever | MMSE-IRC DPoD is enabled (e.g. algorithm in [13]) |

Proposal 4-4: Adopt following link level simulation process.

- Step 1 (baseline):

  - At transmitter, adjust the output power to get the minimum MPR i.e. MPR_0 for 64QAM and MPR_1 for 256QAM that meets all the existing UE RF requirements.

  - At receiver, get the SNR i.e. SNR_0 dB for 64QAM and SNR_1 dB for 256QAM corresponding to 10% BLER.

- Step 2:

  - At transmitter, adjust the output power to get the minimum MPR i.e. MPR_2 for 64QAM and MPR_3 for 256QAM that meets the relaxed EVM and all the other existing UE RF requirements.

  - At receiver, get the SNR i.e. SNR_2 dB for 64QAM and SNR_3 dB for 256QAM corresponding to 10% BLER.

- The net gain is calculated as (MPR_0 - MPR_2 + SNR_0 – SNR_2) and (MPR_1 - MPR_3 + SNR_1 – SNR_3) MPR gain for 64QAM and 256QAM, respectively.

Proposal 4-5: RAN4 can study on proper BS demodulation requirement and corresponding testing by assuming there is no demodulation performance degradation with relaxed UE Tx EVM.

Proposal 4-6: RAN4 study can focus on following cases.

- Case #1 (SU): evaluate the MPR gain under relaxed Tx EVM when single UE is scheduled.

  - E.g. a range of relaxed EVM can be considered for 64QAM and 256QAM.

- Case #2 (MU): evaluate the MPR gain under relaxed Tx EVM when multiple UEs are scheduled.

  - E.g. a range of relaxed EVM can be considered for 64QAM and 256QAM.

  - UEs can apply the same or different relaxed EVM level.

  - UEs can be scheduled in FDM or SDM manner.

Proposal 4-7: RAN4 to calibrate link level simulation based on following case.

- Adopt all the simulation assumptions in Proposal 4-3 and collect simulation results on the Step 1 in Proposal 4-4.

Proposal 4-8: Suggest following working steps for 5G-Advanced Tx EVM relaxation study for the time before 2026 September.

- Step 1: Align the simulation assumptions and methodology, initialize calibration;

- Step 2: Complete calibration, further check on the simulation assumptions and concerned cases;

- Step 3: Simulation results collection, identify impact on UE RF requirements or other aspects.

Observation 4-4: QPSK is supported as basis for study of both CP-OFDM and DFT-s-OFDM for data channel of 6GR UL as per RAN1 agreement. Enhancement/modifications on CP-OFDM/DFT-s-OFDM is studied in RAN1.

Proposal 4-9: RAN4 study PAPR reduction techniques for low-order modulations, including but not limited to RAN1-related mechanisms. The study on techniques transparent to RAN1 can proceed independently, whereas the part relevant to RAN1 waveforms awaits further inputs from RAN1.

- References

[1]	“RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.”

[2]	“R4-2511322, Discussion on introduction of FR2 PC8, Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC.”

[3]	“R4-1707002, RAN4-NR#2 Meeting report.”

[4]	“R4-1701810, More on flexible channel bandwidth consideration in NR, MediaTek Inc.”

[5]	“R4-1703561, MPR/A-MPR for flexible channel bandwidth, Nokia.”

[6]	“R4-1706980 , WF on MPR evaluation assumption, NTT DOCOMO, INC.”

[7]	“R4-1708814, Way Forward on MPR evaluation for mmW, Qualcomm, Skyworks.”

[8]	“R4-1706690, NR Sub- 6 GHz MPR simulation assumptions, Nokia, Skyworks.”

[9]	“R4-2510200, MPR reduction for upper 6GHz by ACLR relaxation, Huawei.”

[10]	“R1-163943, LS on realistic power amplifier model for NR waveform evaluation, RAN1.”

[11]	M. Abdi Abyaneh and Y. Liu, “Evaluation of PA Memory Effects from 5G-NR to 6G Standards,” presented at the IEEE Conference on Standards for Communications and Networking (CSCN) 2025, Bologna, Italy, Sep. 2025.

[12]	“RP-252952, New SID: UE RF enhancements for NR FR1, Phase 5, Moderator (vivo).”

[13]	“Ziv Alina and Ofer Amrani, On Digital Post-Distortion Techniques, IEEE Trans. on signal processing, vol. 64, no. 3, February, 2016.”

[14]	“RP-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair.”