3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513308

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Consideration on RRM study for 6G

Source: 	Huawei, HiSilicon

Agenda item:	8.7

Document for:	Discussion

Introduction

RAN WG level study for 6GR was approved in RAN#108, and the latest SID can be found in [1]. Based on the work plan and RAN4 Chair guidance, RAN4 is expected to discuss RAN4-driven non-AI related RRM aspects for 6GR.

In this paper, we will provide our initial views on several aspects for 6GR RRM study.

Discussion

6GR is a new generation of RAT, and the SID [1] lists a lot of aspects to be studied in different RAN WGs, including the following RAN4 RRM related aspect.

- 6GR core and performance requirements - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles

As in 5G NR and earlier generations, some aspects of RRM requirements depends on the signal, channel and procedure design from other WGs, mainly RAN1 and RAN2. On the other hand, some aspects are driven by RAN4, and can be studied in parallel with RAN1/2. In this contribution, we identify several RAN4 driven RRM aspects that require study from the beginning of the 6GR SI, and provide our views on the enhancements in those aspects based on learnings from 5G NR and earlier generations.

HO requirements

Mobility is fundamental for wireless networks, and HO is the most basic and important procedure mobility. The delay and interruption during HO procedure also have clear impact on the user experience in the field.  A few solutions have been standardized in 5G NR to reduce the HO delay and interruption, but commercial deployment was not that much.

In basic HO requirements in 5G NR, the HO delay is defined as RRC procedure delay plus the interruption time. The interruption time further includes:

- Time for UE processing e.g. SW processing and RF warmup

- Time for cell search or coarse sync, if the target cell has not been measured before HO is commanded

- Time for fine time and frequency sync in DL to get prepared for data scheduling in the target cell

- Time for obtaining RACH resource to complete HO procedure (and also get UL sync)

From RRM requirements, the typical HO delay in 5G NR would be 60ms with 50ms interruption time, assuming known target cell and 20ms SSB period) where the ending point of interruption is first PRACH transmission. In reality, RACH procedure will take around 100ms, so the total time duration without data scheduling would be 150ms.

5G NR introduced several enhanced HO mechanisms to reduce the HO delay and interruption:

- DAPS: UE supports simultaneous Tx/Rx with source and target cells, so the interruption time can be minimized. From UE implementation complexity perspective, some enhancements of BB and RF resources may be required to support simultaneous Tx/Rx.

- CHO: UE executes HO based on pre-configured target cell configurations when configured conditions are met. It can help to avoid the RRC procedure delay for the HO command transmission, but it does not reduce the HO interruption time.

- RACH-less HO: UE directly transmits HO complete message to the target cell on PUSCH based on specified mechanisms to determine TA. It can reduce the HO delay for the RA procedure (which is not visible in RAN4 requirements), but it does not reduce the HO interruption time.

- LTM: UE performs cell switch based on switch command in MAC CE, and UE may perform pre-sync in both DL and UL. It can reduce both delay and interruption in HO. UE may also pre-obtain the beam and CSI measurement for the target cell to enable efficient data scheduling immediately after the HO.

In our view, RAN4 should study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR. Some principles in 5G NR LTM can be used as starting point, e.g. UE can do pre-sync for the candidate target cells while maintaining data connection with the source cell. This will leave the processing time as the main component of interruption time during HO and the interruption is largely reduced, as shown in Figure 1. This can be done with reasonable UE complexity as UE is required to maintain sync rather than whole data link with the two cells.

Proposal 1: RAN4 to study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR.

Figure 1: Example of HO enhancement for 6G

Measurement without gap

Measurement gap has been a pain point for RRM measurement since 4G LTE. Considering mandatory gap pattern with 6ms MGL and 80ms MGRP, the throughput loss due to use of measurement gap would be 15% from UE perspective. From NW perspective, measurement gap also causes restrictions in scheduling and configuration thus adds implementation complexity.

The need of measurement gap comes from the fact that UE has limited RF resource, so it needs to borrow the RF resource for serving carriers to measure target carriers. Measurement without gap has been defined in 4G LTE and 5G NR based on spare RF resource. In the time of 6GR, UE capable of aggregating spectrum over multiple frequency bands with multiple RF chains would be typical, at least for smartphones. For such UEs, it is straightforward to support measurement without gap to fully utilize their RF capability.

In our view, RAN4 should study solution to support measurement without gap for the first release of 6GR, based on use the spare RF chain as shown in Figure 2.

Figure 2: Example of measurement without gap for 6G

As we know, measurement without gap comes with short interruptions. In 5G NR, 2 solutions are defined to support measurement without gap based on spare RF chain, namely NFG and NCSG... NCSG allows NW to control the location of interruption thus leads to better reliability from system point of view. On the other hand, NFG allow more flexible UE implementation and simpler NW implementation, and has the potential possibility to achieve less interruption than NCSG (which allows worst case interruption). As the 2 solutions have its own pros and cons, UE vendor and NW vendor may have distinctly different interests in the two directions. This may cause segmentation and impact commercial application of measurement without gap in 5G NR.

In 6GR, RAN4 should strive for a unified solution which preferably retains advantages of both NFG and NCSG.

Proposal 2: RAN4 to study solutions to support measurement without gap, and strive for a unified solution the first release of 6GR.

Measurement requirements

RRM measurement is the cornerstone for the timeliness and correctness of mobility decisions, e.g. late HO resulted from large measurement delay is a common mobility issue observed in real field.

It is noted that the measurement delay performance is worse in 5G NR compared to 4G LTE. In 4G LTE, the measurement of intra-frequency carriers can be performed in parallel. In 5G NR the RS (SSB) is transmitted in burst manner with larger periodicity than LTE RS, so UE has to do computationally demanding cell search in a time concentrated manner. In order to keep reasonable UE complexity, parallel measurement capability is limited by the number of so-called “searchers”. In R15 2-searcher is taken as baseline, i.e. UE can measure up to 2 carriers in parallel. In R19 3-searcher is supported as an add-on feature.

In our view, 6GR would likely adopt burst RS pattern similar as 5G NR, so RAN4 should study enhanced capability on simultaneous measurements to achieve better measurement delay performance compared to 5G NR as shown in Figure 3.

Figure 3: Example of measurement delay reduction for 6G

The first priority is to consider certain enhancement for the measurement outside gap because 6GR UE is supposed to aggregate more serving carriers and support measurement without gap in many configurations.

Furthermore, measurement with gap can also be considered since the parallel measurement capability or the searcher number does not change with where the measurement is performed. It is noted that in 5G NR, the number of searchers for measurement within gap is only 1, while in 4G LTE parallel measurement within gap is supported in R14. Similar approach inherited from LTE can be considered as starting point.

Proposal 3: RAN4 to study RRM measurement delay reduction for the first release of 6GR by considering enhanced simultaneous measurements.

RRM spec improvement

In 5G NR, RAN4 has worked on the task to improve the RRM spec 38.133, and several solutions have been identified from drafting perspective. On the other hand, from technical perspective some improvements can also be studied to make 6GR RRM spec more consistent and simpler.

One possible area to improve is the classification of L3 RRM measurement requirements. In 38.133 there are 4 categories of measurement: Intra-frequency without gap, Intra-frequency with gap, Inter-frequency with gap and Inter-frequency without gap. On the other hand, the performance requirements mainly differentiate depending on whether the measurement is performed within or outside MG. Number of sets of requirements can be reduced for better clarity and reduced redundancy, and the terminology of “with/without gap” and “within/outside gap” should be used more careful to avoid possible confusion.

Another possible area to improve is the collision handling requirements. In 5G NR, RRM requirements deal with different kinds of collisions, e.g. between different types of measurements, between data and measurement, and between measurement and other RRM related procedures. There is no clear principle for which collisions RAN4 should define exact requirements, define requirement applicability or nothing at all. This was discussed in a case by case manner in different features, and sometimes effort consuming. Also, different approaches are used to capture the impact of different collisions, e.g. scaling factor using equation (P factor in L1 measurement), scaling factor based on counting (con-MG approach), or extension of sample number (NR-U approach). It is better to have aligned principle and approach for collision handling.

Proposal 4: RAN4 to study at least following aspects or RRM spec improvement in 6GR

- Better classification of L3 RRM measurement requirements

- Consistent principles to address different collisions

Conclusions

In this paper we provided our initial views on several aspects for 6GR RRM study.

Proposal 1: RAN4 to study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR.

Proposal 2: RAN4 to study solutions to support measurement without gap, and strive for a unified solution the first release of 6GR.

Proposal 3: RAN4 to study RRM measurement delay reduction for the first release of 6GR by considering enhanced simultaneous measurements.

Proposal 4: RAN4 to study at least following aspects or RRM spec improvement in 6GR

- Better classification of L3 RRM measurement requirements

- Consistent principles to address different collisions

Reference

- RP‑252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone