3GPP TSG-RAN4 Meeting #116-bis	R4-2513347

# Prague, Czechia, 13rd - 17th October, 2025

Agenda item:		8.12

Source:				Keysight Technologies UK Ltd

Title:					On UE 6G OTA Testability Aspects

Document for:		Approval

# Introduction

This contribution is addressing our preliminary thoughts about the 6G OTA testability aspects for UE testing, specifically the non-AI features, of the new 6G SI [1].

# Discussion - 6G OTA testability aspects

The 6G Radio SI [1] captured various objectives/goals for the different TSG WGs (RAN1, RAN2, RAN4, RAN4). One of the many RAN4 aspects is related to testability and OTA, i.e.,

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

The RAN4 chair accordingly setup 11 different 6G topics in the respective planning document [2] for the upcoming RAN4#116-bis meeting with one of those topics being ‘OTA and testability’, i.e.,

We had previously shared some of our thoughts on OTA testability aspects in the 6G Workshop held earlier this year [3], i.e.,

The following sections provide some additional views on OTA testability aspects for the different frequency ranges considered [1], i.e.,

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

## OTA Testability Aspects for FR1 (up to 7.125 GHz)

The testing aspects for FR1 have been very well established, especially given the legacy radio communication standards over the last few decades. Conformance testing of UE RF [4], demodulation [5], and RRM [6] test cases are performed almost exclusively in a conducted environment where the test equipment is connected directly to the conducted port(s) of the radio front-end (FE). The compliance with the RF and baseband (BB) requirements are thus assessed without the antenna system considered. OTA testing takes into account the full end-to-end performance including antennas, RF, and BB and is commonly performed for two key metrics, i.e., Total Radiated Power (TRP) which follows the Maximum Output Power (MOP) Tx conducted test case but assesses the end-to-end UL performance with a spatial average performance of the power radiated (EIRP) and the Total Radiated Sensitivity (TRS) which follows the conducted reference sensitivity (REFSEN) Rx test cases but assesses the end-to-end DL performance with a spatial average performance of the received power (EIS) [7]. Sample test systems for these OTA metrics are shown in Figure 1 (Figures A.3.1-1 and A.3.1-2 from [7])

Figure 1: Example of FR1 TRP TRS OTA test system with combined axis (top) and distributed axes (bottom)

Additionally, the MIMO DL OTA performance is assessed in the presence of realistic channel models [8] which were reduced though from 3D to 2D (with ZoA set to 90°) for 4G and NR FR1 for testing to simplify the test systems, i.e., the clusters of the CDL channel model are introduced merely from a horizontal plane [9]. The Multi-Probe Anechoic Probe (MPAC) test setup with its 16 uniformly placed probes (NR FR1, LTE uses 8 probes) is shown in Figure 2 (Figure A.2.1-1 from [9]).

Figure 2: MPAC system layout for NR FR1 MIMO OTA testing

Given the established conformance testing approaches over the years and multiple generations of cellular standards in this frequency range, it is proposed to continue the majority of conformance testing (UE RF, Demodulation, and RRM) for 6G FR1 using primarily conducted testing but to selectively verify the end-to-end performance using OTA methodologies as discussed further below.

It is expected that the dynamic nature of multiple active receiver and transmitter operation, e.g., antenna tuners, dynamic switching between transmitters and receivers, etc., is becoming increasingly prevalent going forward which will likely see an increase in conducted conformance test system complexity (including the need for vendor declaration, etc.); still, these assessments are not taking real-world performance into account without the antenna system. Since low bands require much larger antenna sizes than high bands, it is expected that especially for smartphone UEs, dynamic Tx/Rx operation will focus on mid (>3 GHz) to high-band (>5 GHz) operation within FR1. Similarly, the complexity of conducted testing has increased significantly in NR FR1 especially based on the available number of physical Tx and/or Rx ports of the UE, e.g., the number of UE Rx ports were increased from 4Rx (Rel-15) to 8Rx (Rel-18). It is therefore proposed to consider novel OTA test cases to assess real-world performance, especially for mid to high-band operation within FR1, where conducted testing could either yield testability issues due to lack of physical Tx/Rx ports and/or highly dynamic nature of multiple Tx/Rx operation. The maximum sustained data rate test case could be one example where a transition from conducted to radiated/OTA environment might be beneficial.

Proposal 1: For 6G FR1, continue the majority of conformance testing (UE RF, Demodulation, RRM) using conducted testing

Proposal 2: For 6G FR1, consider novel, select OTA test cases to assess real-world performance to selectively verify the end-to-end performance using OTA methodologies

Proposal 3: For 6G FR1, consider OTA testing where conducted conformance testing (UE RF, Demodulation, RRM) could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern

## OTA Testability Aspects for “the range between FR1 and FR2-1 (including around ~7GHz)”

The original 5G NR SI [11] defined Range 1 up to 6 GHz and Range 2 from 6 GHz to 100 GHz with conducted conformance requirements preferred for Range 1 and radiated requirements for Range 2.

Figure 6.2.1-1: Frequency range 1/2 and the threshold.

In Rel-16, a study on the 7-24 GHz range [12] suggested three sub-ranges between 7.125 GHz (FR1 high end) and 24.25 GHz (FR2 low end) with recommendations on conducted vs OTA testing, e.g., Figure 6.2-1 [12].

Fig 6.2-1 Frequency sub-ranges and the RF requirement type

For 5G NR, only the FR1 and FR2 ranges (including sub-ranges) were defined, e.g., Table 5.1-1 of [13]

Table 5.1-1: Definition of frequency ranges Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz.

and the range between FR1 and FR2 was not considered for 5G NR terrestrial operation.

NR FR1 defined conformance requirements predominantly for the conducted environment and NR FR2 defined all UE requirements for the radiated environment due to the lack of conducted ports for this mm-wave frequency range and thus followed the recommendation of the 5G NR SI [11].

The frequency range between FR1 and FR2 obviously generated tremendous interest for 6G over the last few years in industry and was commonly called FR3. Most people familiar with standards and the knowledge that FR3 has not been officially defined yet, were more careful and referred to this range as “FR3”

FR3

It would be desirable to officially give this frequency range a better identifier rather than “the range between FR1 and FR2-1 (including around ~7GHz)”

Observation 1: It would be desirable to officially give the new frequency range of special interest for 6G a better identifier rather than “the range between FR1 and FR2-1 (including around ~7GHz)”

Whether the UE requirements should be defined in mainly conducted vs radiated environments, is primarily up to OEM and operators and not test equipment/system vendors. However, it might be worthwhile to point out the pros and cons of each considered approach which is summarized in Table 1

Table 1: Overview of Key Pros and Cons of Conducted vs Radiated Conformance Requirements/Testing

| Testing Aspects | OTA | Conducted |
| --- | --- | --- |
| Test Time | OTA test time is generally longer, especially if spatially averaged metrics, e.g., TRP, TRS, or CDF/CCDF is required since EIRP/EIS measurements are performed for each grid point | Conducted test times are generally shorter since only a single test point is required, i.e., no spatial aspects are considered. Additionally, conducted testing generally focuses on just pass/fail test approach without sensitivity searches |
| Test Automation | When charging/data cables can have an impact on the radiated performance, e.g., low to mid region of FR1, no test automation can be supported. For FR2, charging cables were shown to have insignificant impact on performance if the cable routing was implemented carefully to avoid blocking of the antenna; thus, test automation could be considered. | Conducted tests commonly leverage full test automation with charging/data cables |
| Dynamic Range | Given the relatively large path losses between the test equipment and the DUT (cables, switching, and free-space path loss), the dynamic range of the test system can be compromised, especially for low-UL/high-DL test cases | Conducted tests typically have significantly larger dynamic ranges since the test systems have lower path losses (cable, switching, and no free-space path losses) |
| Measurement Uncertainty | Generally larger for OTA than conducted given the increased complexity of the test system | Generally smaller for conducted than OTA given the reduced complexity of the test system, e.g., lack of amplification, higher dynamic range. On the other hand, testing with ≫1 conducted ports could require significant switching/cabling. |
| Realism | The OTA test cases are always going to represent a lot more realism since the full end-to-end performance (including antennas) are considered | Conducted testing “only” takes BB and RF into account without the antenna system and can therefore not be considered realistic. Conducted testing often overestimates performance as it is focused on 50Ω environment; multiband antennas in highly integrated UEs are not able to reach a perfect 50Ω impedance and commonly have efficiencies ≪ 100%, especially in realistic talk conditions, e.g., phantoms. |

Observation 2: The key benefit of OTA testing is the inclusion of realism in the device assessment

Proposal 4: When deciding the testing environment of 6G for the range between FR1 and FR2, take testing aspect pros and cons for conducted vs radiated testing, summarized in Table 1, into account

The key question for the actual frequency range dedicated to 6G is whether 6G will operate closer to FR1 or FR2. This can/will likely have a significant impact on the decision towards radiated vs conducted requirements as the presence of a physical conducted port/connector will likely be dependent on that frequency range of operation.

Observation 3: The decision of radiated vs conducted requirements for 6G operation between FR1 and FR2 will likely depend on the actual frequency range of operation and whether a physical conducted port/connector is feasible.

More importantly, the actual frequency range of 6G operation between FR1 and FR2 will determine which existing test systems and methodologies should be leveraged for OTA testing, e.g., should existing FR1 test systems be upgraded to support an extended frequency range towards the top end or should existing FR2 test systems be upgraded to support an extended frequency range towards the bottom end? Generally, it is more straightforward to increase the upper frequency of existing test systems rather than decreasing the lower frequency of operation, e.g., due to absorber performance. Especially given the rather different nature of methodologies for FR1 (DFF with distributed-axes systems) and FR2 (IFF with primarily combined-axes systems), it would be desirable to decide on the baseline system/methodology early.

Proposal 5: Conclude on the actual frequency range of 6G operation between FR1 and FR2 early to determine whether existing FR1 or FR2 systems/methodologies should serve as baseline for OTA testing.

Industry’s interest in this range over the last few years especially focused on larger channel bandwidths and larger number of BS antenna array elements; for Multi User (MU)-MIMO, this was commonly reflected in their terminology such as Massive-MIMO, Giga-MIMO, eXtreme-MIMO, etc. The use of hundreds if not thousands of BS antenna array elements promised pencil beams to individual users including beam forming in AZ as well as EL. This could/should have test and test system implications for MIMO OTA testing, i.e., the simplification of setting the ZoA of the CDL channel models to 90° would no longer be representative/real life. One example extension of a 2D MPAC (with 2D channel models only) to a 3D MPAC (with 3D channel models) is illustrated in schematically in Figure 3.

Figure 3: Example extension of 2D MPAC (5G MIMO OTA, left) to 3D MPAC (6G MIMO OTA, right)

It is proposed to focus MIMO OTA assessments in 6G for the range between FR1 and FR2 on more realistic scenarios including dynamic channel models, link adaptation (similar to those studied in [10]) and 3D channel models. It should be studied whether these aspects should be applied to mid to high bands in 6G FR1 as well.

Proposal 6: Focus MIMO OTA assessments in 6G for the range between FR1 and FR2 on more realistic scenarios including dynamic channel models, link adaptation and 3D channel models. FFS whether these aspects should be applied to mid to high bands in 6G FR1 as well

Additionally, similar considerations as for FR1 should be considered here if industry decided to focus on conducted testing for the range between FR1 and FR2.

Proposal 7: If the focus of 6G conformance testing in the range between FR1 and FR2 is conducted, consider novel, select OTA test cases to assess real-world performance to selectively verify the end-to-end performance using OTA methodologies

Proposal 8: If the focus of 6G conformance testing in the range between FR1 and FR2 is conducted, consider OTA testing where conducted conformance testing (UE RF, Demodulation, RRM) could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern

## OTA Testability Aspects for FR2-1 (24.25 GHz – 52.6GHz)

As mentioned earlier, all UE requirements in the FR2 range were defined for radiated environments given the lack of physical conducted ports/connectors.

During the study and work-item phase of 5G NR FR2, test equipment and system vendors have repeatedly pointed out testability issues of select test cases which often were disregarded (“consider these requirements system level requirements”) and thus caused significant efforts in RAN5 to define relaxations as well as a list of test cases that are untestable due to the lack of dynamic range. In other instances, requirements were defined, e.g., various FR2 power classes or NTN VSAT (Ka/Ku bands), without appropriately defining the testability aspects whatsoever. It is therefore proposed to consider the testability aspects during the definition of 6G requirements.

Proposal 9: 6G OTA testability aspects need to be considered together with feature development going forward (regardless of frequency range) as important synchronizations between WIs and SIs and/or RAN4 and RAN5 resulted in unnecessary delays and confusion, e.g., low-UL/high-DL test cases, Ku/Ka bands, various FR2 power classes.

The two key concerns from industry regarding FR2 testability were related to the large MUs (in excess of 4-5dB) and the test time. The mm-wave frequency range will obviously come with larger test equipment and component uncertainties, and the OTA environment will have additional MU elements with non-insignificant MUs when compared to conducted environment. Whether significant reductions in MU due to test equipment will be possible for the FR2 range for 6G operation is questionable.

Observation 4: Whether significant reductions in MU due to test equipment will be possible for the FR2 range for 6G operation is questionable

However, there is a potential to address the overall test time concerns and some MU impact for 6G FR2 OTA testing. Here, it might be worthwhile to briefly review the nature of classical conducted conformance and OTA test approaches: while conducted conformance test cases commonly check the pass/fail compliance to a given requirement, the OTA test cases generally assess the actual Tx/Rx end-to-end performance by leveraging a parametric test approach (“searches”). For receiver sensitivity assessments, for instance, an iterative search is performed in an OTA test to determine the sensitivity, EIS, that yield a specific TP threshold, e.g., Clause A.3.3.3 of [14]

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [11]).

while in a conducted test the REFSENS power level is applied at the conducted port to assess whether the minimum TP threshold is reached/exceeded (PASS) vs not (FAIL), e.g., Clause 7.3 of [15]

## 7.3 Reference sensitivity ### 7.3.1 General The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In other words, classical OTA testing generally allowed performance ranking of UEs, i.e., not just a pass/fail verdict.

For 5G NR FR2 OTA testing with active antenna arrays, it was decided to leverage the performance ranking and the parametric approaches for some of the key UE RF metrics, e.g., Maximum Output Power (MOP) and REFSENS by mandating beam peak searches with very fine measurement grids to identify the test direction that yields the best MOP or REFSENS performance of the UE. Trade-offs were required between selecting an acceptable uncertainty (to select the absolute best beam) and test time for the 3D searches; those trade-offs still resulted in significant MUs and test times just to be able to assess certain absolute performance Tx/Rx metrics (rather than the compliance with the pass/fail requirements). For 5G NR FR2, these beam peak searches were defined as pre-condition tests for a large number of UE RF, Demodulation, and RRM TCs that merely checked for compliance against pass/fail conditions. A reduction in MU and test time for 6G FR2 could be to avoid beam peak searches to determine the absolute best Tx/Rx performance directions (within a given uncertainty) and allowing testing in directions that pass the minimum requirements for MOP and REFSENS instead. For instance, such directions could be identified during the spherical coverage test cases that are also utilizing 3D searches but with a much coarser measurement grid compared to the beam peak searches. Whether the beam peak direction is absolutely required for some of the regulatory test cases is FFS.

Proposal 10: For 6G FR2 UE RF OTA testing, consider revisiting the need for beam peak searches to reduce MU and overall test time.

In general, however, it is proposed to leverage the UE RF, Demodulation, and RRM test systems and methodologies for 6G FR2 from 5G FR2 as a baseline.

Proposal 11: Leverage the UE RF, Demodulation, and RRM test systems and methodologies for 6G FR2 from 5G FR2 as a baseline

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: It would be desirable to officially give the new frequency range of special interest for 6G a better identifier rather than “the range between FR1 and FR2-1 (including around ~7GHz)”

Observation 2: The key benefit of OTA testing is the inclusion of realism in the device assessment

Observation 3: The decision of radiated vs conducted requirements for 6G operation between FR1 and FR2 will likely depend on the actual frequency range of operation and whether a physical conducted port/connector is feasible.

Observation 4: Whether significant reductions in MU due to test equipment will be possible for the FR2 range for 6G operation is questionable

Proposal 1: For 6G FR1, continue the majority of conformance testing (UE RF, Demodulation, RRM) using conducted testing

Proposal 2: For 6G FR1, consider novel, select OTA test cases to assess real-world performance to selectively verify the end-to-end performance using OTA methodologies

Proposal 3: For 6G FR1, consider OTA testing where conducted conformance testing (UE RF, Demodulation, RRM) could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern

Proposal 4: When deciding the testing environment of 6G for the range between FR1 and FR2, take testing aspect pros and cons for conducted vs radiated testing, summarized in Table 1, into account

Proposal 5: Conclude on the actual frequency range of 6G operation between FR1 and FR2 early to determine whether existing FR1 or FR2 systems/methodologies should serve as baseline for OTA testing.

Proposal 6: Focus MIMO OTA assessments in 6G for the range between FR1 and FR2 on more realistic scenarios including dynamic channel models, link adaptation and 3D channel models. FFS whether these aspects should be applied to mid to high bands in 6G FR1 as well

Proposal 7: If the focus of 6G conformance testing in the range between FR1 and FR2 is conducted, consider novel, select OTA test cases to assess real-world performance to selectively verify the end-to-end performance using OTA methodologies

Proposal 8: If the focus of 6G conformance testing in the range between FR1 and FR2 is conducted, consider OTA testing where conducted conformance testing (UE RF, Demodulation, RRM) could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern

Proposal 9: 6G OTA testability aspects need to be considered together with feature development going forward (regardless of frequency range) as important synchronizations between WIs and SIs and/or RAN4 and RAN5 resulted in unnecessary delays and confusion, e.g., low-UL/high-DL test cases, Ku/Ka bands, various FR2 power classes.

Proposal 10: For 6G FR2 UE RF OTA testing, consider revisiting the need for beam peak searches to reduce MU and overall test time.

Proposal 11: Leverage the UE RF, Demodulation, and RRM test systems and methodologies for 6G FR2 from 5G FR2 as a baseline

# References

- RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, June 2025

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, 3GPP TS_RAN WG4 #116, August 2025

- 6GWS-250191, Keysight views on 6G testability, Keysight Technologies, 3GPP 6G Workshop, March 2025

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

- TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- TS 38.133, Requirements for support of radio resource management

- TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

- TR 38.820, 7 - 24 GHz frequency range

- TS 38.101-3, User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

- TS 38.561, User Equipment (UE) conformance specification; UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone