3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513356

# Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on HD-FDD VSAT RF requirements

Source:	Samsung

Agenda item:	7.6.4.1

Document for:	Approval

Introduction

In last RAN #109 meeting, the meeting approved the WID [1] for further enhancement of both NR-NTN and IoT-NTN. In this agenda, it includes three major aspects: HPUE for NTN, HD-FDD for NTN and DL intra-band intra-SAN CA. And in this paper, we will focus on the NR-NTN objectives as follows:

HD-FDD for NTN Core Part

- Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 2:  Ku band and Ka band with FR2 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

- Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

Disucssions

HD-FDD impacts on RF requirements

In the WID[1], the justifications listed for HD-FDD is given as follows:

Implementation constraints HD-FDD eliminates the need for separate transmit and receive FD-FDD antenna panels, reducing antenna size by about 50%. HD-FDD avoids complex duplexers/filters in a FD-FDD single antenna panel; typically reducing performance by over 3 dB thus requiring at least double the antenna size (area) to compensate.

From the RF requirements point of view, adopting the HD-FDD may only bring improvement to the maximum output power and reference sensitivity requirements. However, the VSAT OTA reference sensitivity requirements were simply derived from the link budget and G/T perspective as given in [2], we believe it may leads to no change by introducing HD-FDD.

##### 7.4.3.3.2 OTA reference sensitivity level If 62dBm EIRP can be assumed for LEO600 SAN with 30 degree elevation angle scenario, the DL budget calculation is shown below to get the DL received signal at OTA (effective isotropic received signal). Table 7.4.3.3.2-1: The calculation of DL budget for LEO600 scenario LEO600. NTN SAN EIRP (dBm); 62. Elevation angle (degree); 30. Slant range (km); 1075. center frequency (GHz); 17. freePathloss; 177.68. VSAT DL Received signal level at OTA (dBm); -115.68. If -5dB G/T can be assumed to calculate the EIS requirements, the following calculation can be used to derived it. EIS=10*log10(k)+30+10*log10(BW)+SNR+IM-G/T BW is channel bandwidth. (candidate value: 50MHz) IM is implementation margin. 2dB can be assumed. SNR is the required SNR for demodulation. -1dB can be assumed. G/T: -5dB. K: is Boltzmann constant, 1.38*10-23 J/K Based on the calculation, the 50MHz minimum EIS for fixed VSAT communicating with LEO only with electronical steering antenna is -115.6dBm, which is valid for DL link budget. If 82dBm EIRP can be assumed for GSO SAN with 30 degree elevation angle scenario, the DL budget calculation is shown below to get the DL received signal at OTA (effective isotropic received signal). Table 7.4.3.3.2-1: The calculation of DL budget for GSO scenario GSO. NTN SAN EIRP (dBm); 87. Elevation angle (degree); 30. Slant range (km); 38611. center frequency (GHz); 17. freePathloss; 208.78. VSAT DL Received signal level at OTA (dBm); -121.78. If 6dB G/T can be assumed to calculate the EIS requirements, the following calculation can be used to derived it. EIS=10*log10(k)+30+10*log10(BW)+SNR+IM-G/T BW is channel bandwidth. (candidate value: 50MHz) IM is implementation margin. 2dB can be assumed. SNR is the required SNR for demodulation. -1dB can be assumed. G/T: 6 dB. K: is Boltzmann constant, 1.38*10-23 J/K Based on the calculation, the 50MHz minimum EIS for other types is -122 dBm, which is valid for DL link budget.

Proposal 1: Given the min EIS was determined by DL link budget for Ku-band VSATs, to discuss whether HD-FDD would bring improvements on min EIS requirements.

From justification, the argument mentions the potential 3 dB gain from HD-FDD implementation compared to FD-HDD VSATs. From our understanding, the justification implies the reduced loss from reduced RF components by adopting HD-FDD compared to FD-HDD and also the available elements from antenna panel. However, it is not quite clear to us whether we should target on a higher min EIRP requirements and if so, how many dB more we could achieve. We believe this requires further study.

Proposal 2: To clarify and study whether to introduce an improvement in eirp requirements for HD-FDD VSAT compared to HD-FDD impacts.

Observation 1: In general, our observation is that no changes may be brought to RF requirements by simply having the HD-FDD VSAT compared to FD-FDD. As a result, maybe a general description on existing Ku-band VSAT requirements can be introduced to declare the requirements can also apply to HD-FDD VSAT.

Ultra small antenna impacts on RF requirements

The objectives for the ultra small antenna are a little bit more confusing to us. It only mentions “e.g. 20x20 cm” as the definition of ultra small antenna, and “a new VSAT type” is targeted also requires further explanation from Rapporteur company.

In our initial review, the change of antenna size assumption of VSAT would generally change the antenna gain, output power in Tx parts and also the referense sensitivity in Rx parts. However, with lack of the specific assumption of this “Ultra small” antenna, the analysis work would be hard to carry out.

Proposal 3: It should be clarified first about the detailed “typical assumption” of this “ultra small antenna”, especially on its element gain, element number, Tx power per element and Rx

Secondly, if this “ultra small antenna” is target on one “new VSAT type”, another key question is whether it’s for Fixed or Mobile, and whether it’s for GEO or LEO. The RF requirements, as previously defined for Type 1~6, are directly depends on them.

Proposal 4: It should also be clarified whether “a new VSAT type” is for Fixed or Mobile, and whether it’s for GEO or LEO.

Observation 2: In our understanding from the WID, the demand is mainly from 5GAA or automotive applications, in that sense, we believe this “a new VSAT type” may be target for Mobile VSAT communicating with LEO (or both LEO and GEO) with electronic steering antenna.

Conclusion

In this contribution, we provide following discussion points on HD-FDD VSAT:

Proposal 1: Given the min EIS was determined by DL link budget for Ku-band VSATs, to discuss whether HD-FDD would bring improvements on min EIS requirements.

Proposal 2: To clarify and study whether to introduce an improvement in eirp requirements for HD-FDD VSAT compared to HD-FDD impacts.

Observation 1: In general, our observation is that no changes may be brought to RF requirements by simply having the HD-FDD VSAT compared to FD-FDD. As a result, maybe a general description on existing Ku-band VSAT requirements can be introduced to declare the requirements can also apply to HD-FDD VSAT.

Proposal 3: It should be clarified first about the detailed “typical assumption” of this “ultra small antenna”, especially on its element gain, element number, Tx power per element and Rx

Proposal 4: It should also be clarified whether “a new VSAT type” is for Fixed or Mobile, and whether it’s for GEO or LEO.

Observation 2: In our understanding from the WID, the demand is mainly from 5GAA or automotive applications, in that sense, we believe this “a new VSAT type” may be target for Mobile VSAT communicating with LEO (or both LEO and GEO) with electronic steering antenna.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2] TR 38.863