3GPP TSG-RAN WG4 Meeting #116bis            	R4-2513403

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.6.2.2

Source:	Samsung

Title:	Simulation results on NTN demodulation requirements for Less than 5MHz

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue on the test scope for performance part of NTN for less than 5MHz was under discussion. The related agreement was captured into WF [1].

In this contribution, both ideal and impairment simulation results are provided for alignment and requirement

# 2	Simulation Results

In this section, the ideal and impairment simulation results are provided for NR NTN PUCCH format 2 are provided, based on the agreed simulation assumption as following

Table 1: Simulation assumption for PUCCH format 2 with NTN less than 5MHz

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Note: The first PRB after frequency hopping is 3 |  |

Table 2: Minimum requirement for PUCCH format 2, 15KHz SCS and 3MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex D) | SNR (dB) (BLER%1) ideal | SNR (dB) (BLER%1) Impairment |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 7.39 | 9.39 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 0.53 | 2.53 |

# 3	Conclusion

In this contribution, both ideal and impairment simulation results for NR NTN PUCCH format 2 are provided for alignment and requirement.

# 4	 References

R4-2512614, Way Forward for [116][324] NR_IoT_NTN_less_than_5MHz_demod, Ericsson