# 3GPP TSG-RAN WG4 Meeting #116bis	  R4-2513429

Prague, Czech Republic, October 13 – 17, 2025

Title: 	Discussions on 6Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2

Document for:	Discussion

Background

A WF [1] for 6Rx performance requirements was agreed in last meeting, this contribution provides our views.

- Discussions

PDSCH requirements

- DMRS configuration for 6 MIMO layers case

- One issue is DMRS configuration for 6MIMO layers case:

- Proposals - Option 1: Rel-15 Type 1 DMRS 2+2 - DMRS ports {1000-1003, 1004, 1006} - Option 2: Rel-18 eType1 DMRS 1+1

- We don’t support to bundle 6Rx requirements with other new features, since the application of Rel-18 eDMRS is still not clear and such additional configuration will reduce the test coverage.

- Proposal 1: Don’t use Rel-18 eType1 DMRS 1+1

- MIMO correlation for 4 MIMO layers for Handheld UE testing

There are two options for MIMO correlation configuration:

- Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

We prefer not to define different requirements for handheld UE and FWA respectively, since supporting 4 layers is basic feature which should be applicable for all UE types. Demodulation requirements are always not defined according to UE types. Furthermore, the antenna correlation defined in TS 38.101-4 is based on uniform antenna placement, which cannot reflect the real UE antenna implementation. But to move forward, we are open to option 2 if companies have strong views,

For option 1a, our simulation results in last meeting show that both option 1a and 1b give reasonable SNR range. Option 1a is preferred if it’s agreed to define single set of requirements for FWA and handheld UE,

Proposal 2: For 4 layers, RAN4 to define single set of requirements for both handheld UE and FWA with correlation parameters α = 0 and β = 0.07179 or define separate requirements with α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

SDR requirements

1024QAM with 4layers

One issue is whether to introduce 1024QAM with 4 layers. To check the feasibility, a simulation was conducted, where the TxEVM is set to 2.5%.  The simulation results are shown in Figure 2.2-1 and Table 2.2-1.

Figure 2.2-1: Simulation for SDR test with 1024QAM/4 layers

Table 2.2-1: Simulation results for SDR test with 1024QAM/4 layers

| MCS | Target SNR(dB)@85% of maxTP |
| --- | --- |
| 23 | 27.4 |
| 24 | 29.4 |
| 25 | 31.4 |
| 26 | N/A |

It can be observed that 1024QAM with 4layers requires quite SNR range, which is even higher (close to 30dB) if considering additional margin. Note that AWGN channel model is assumed for SDR test and SNR may be more unachievable for fading channel. We don’t think such SNR range can occur in the field.

Proposal 3: Don’t introduce SDR requirements for 1024QAM with 4layers.

Simulation results

Figure 2.2-1 and Table 2.2-1 show our initial simulation results for MCS table 1 (64QAM with 6 layers), MCS table 2(256QAM with 6 layers) and MCS table 4 (1024QAM with 2 layers) respectively, TxEVM is assumed as 3%,3%,2.5% for 64QAM, 256QAM and 1024QAM respectively.

Figure 2.2-2: Simulation assumptions for SDR test

Table 2.2-2: Simulation summary for SDR test

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 | MCS Table 2 | MCS Table 4 |
| 28 | 19.9 |  |  |
| 27 | 18.8 | 26.0 |  |
| 26 | 17.7 | 24.6 |  |
| 25 | 16.7 | 23.7 | 28.3 |
| 24 | 15.7 | 22.7 | 26.1 |
| 23 | 14.7 | 20.8 | 24.5 |
| 22 |  | 19.7 |  |

Based on our simulation results above, we propose the following: for scalding factor 1, consider MCS 24 for 256QAM with 6 layers, MCS 26 for 64QAM with 6layers and MCS 24 for 1024QAM with 2layers.

Proposal 4: Consider following MCS look-up table for 6Rx:

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 24 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 26 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

CSI requirements

CQI test

One of issues is whether to define CQI test, the candidate options are:

Issue 4-1-1: CQI test for 6Rx - Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel

The motivation of CQI test is not clear. The SINR difference between 4Rx and 6Rx at receiving side is 10log64=1.7dB, which is less than SINR threshold difference between adjacent CQI index. I.e. 2dB. Therefore, we don’t think calculated CQI index will change much between 4Rx and 6Rx.

Observation 1: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 5: Not define CQI test.

RI test

One of issues is whether to define RI test, the candidate options are:

Issue 4-1-2: RI test for only Rank 6 - Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

The motivation of RI test is not clear, according to the outcome from core part, only FWA device can support 6 layers. However, 8Rx is also only supported by FWA device, but we didn’t define RI test for 8Rx. We don’t see the technical reason why RI test is not defined for high capability FWA device with 8Rx but will be defined for low capability FWA device with 6Rx. Furthermore, according to the study from core part, there is low probability that 6 layers can be scheduled. Therefore, it’s expected that gain is not high for Rank6.

Proposal 6: Not define RI test.

- Conclusion

This contribution provides our overviews on 6Rx performance requirements, the proposals and observations are:

- Proposal 1: Don’t use Rel-18 eType1 DMRS 1+1

Proposal 2: For 4 layers, RAN4 to define single set of requirements for both handheld UE and FWA with correlation parameters α = 0 and β = 0.07179 or define separate requirements with α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Proposal 3: Don’t introduce SDR requirements for 1024QAM with 4layers.

Proposal 4: Consider following MCS look-up table for 6Rx:

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 24 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 26 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

Observation 1: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 5: Not define CQI test.

Proposal 6: Not define RI test.

- Reference

[1] R4-2508727 Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx