3GPP TSG-RAN WG4 Meeting #116bis	R4-2513434

Prague, Czech Republic, October 13 – 17, 2025

Title: 	Discussion on demodulation requirements for Ambient IoT Base station

Source: 	Huawei, HiSilicon

Agenda item:	6.15.5

Document for:	Discussion

Background

In last meeting, a WF[1] for A-IoT_demod was approved, where some open issues are left. This contribution provides our related issues.

- Discussions

Whether to define PDRCH demodulation requirement for Ambient-IoT

One issue is whether to define PDRCH demodulation requirements for Ambient-IoT, the candidate options are:

- Way forward - Option 1: Define PDRCH demodulation requirement. - Option 2: Not define PDRCH demodulation requirements

We prefer option 1, TS 38.191 has reserved one section for demodulation requirements and Ambient-Iot is a new Base station type and PDRCH is new physical channel, we should define limited number of tests to verify the performance.

Proposal 1: Define PDRCH demodulation requirements

D2R message structure for testing

One issue is D2R message structure for testing:

- Way forward - Option 1: Short D2R transmission with midamble inserted with 31 bits Preamble without additional midamble - Option 2: Long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble - Other options are not precluded

We prefer to configure 31bits preamble and additional midamble, i,e, option 2 to guarantee the time/frequency synchronization performance

Proposal 2: RAN4 to configure 31 bits preamble and additional midamble

Reader detection assumption

One issue is reader detection assumption, the candidate options are:

- Way forward - Option 1: Coherence detection - Other options are not precluded

PDRCH is based on OOK modulation, envelop detection is more reasonable, but it’s up to BS implementation for requirements definition.

Proposal 3: The reader detection assumption should be up to BS implementation

Modulation

One issue is modulation, the candidate options are:

- Way forward - Option 1: Both BPSK and OOK modulation scheme - Option 2: OOK modulation

We prefer to only consider one modulation scheme, OOK modulation is preferred.

Proposal 4:  Consider OOK modulation

Block level Repetition

One issue is block level repetition, the candidate options are:

- Way forward - Option 1: 1 - Option 2: 2

We prefer option 1.

Proposal 5:  Prefer option 1, consider no repetition.

SFO assumption & Device sampling rate

There are issues of SFO assumption and device sampling sate:

SFO assumption - Way forward - Option 1: 104 ppm~105 ppm - Other options Device sampling rate - Way forward - Option 1: 1.92MHz device sampling rate - Other options

10% SFO is assumed for RAN1 D2R evaluation and is also defined for RF requirements. SFO is a critical issue from device side due to hardware limitation and has big impact on the performance. We prefer 10% SFO, i,e, 105 ppm and 1.92MHz device sampling rate to align RAN1 assumption.

Proposal 6:  Prefer 10% SFO and 1.92MHz device sampling rate.

D2R message type for testing

One issue D2R message type for testing:

- Way forward - Option 1: Define PDRCH demodulation requirements with Message 1 reception - Option 2: Define PDRCH demodulation requirements with Message 3 reception

We prefer message 3, which is transmitted in connected mode and demodulation test is conducted in connected mode.

Proposal 7: Define PDRCH demodulation requirements with message 3 reception.

TBS

One issue is TBS, the candidate options are:

- Way forward - Option 1: 96 bits - Other options

RAN2 specifies the details of D2R upper layer data transfer message:

The fields in this message are defined as follows: - More Data Indication (MDI): This field indicates whether there are more upper layer data to be sent from the device (when set to 1) or not (when set to 0). This length of this field is 1 bit. - SDU Length: This field indicates the length of the Data SDU field in the unit of byte. The length of this field is 7 bits. - Data SDU: This field is of variable length and includes the upper layer data. - MAC Padding: This field includes padding bits. This field is optional.

We can see that the TBS mainly depend on variable upper layer data size, we think 96bits is reasonable.

Proposal 8: Use TBS 96 bits.

Channel model

One issue is channel model:

- Way forward - Option 1: TDL-A30-10 - Other options are precluded

We prefer option 1,i,e, TDLA30-10

Proposal 9: Use TDLA30-10

CBW

One issue is CBW, the options are:

- Way forward - Option 1: further discuss how to select the proper CBW - Option 2: postpone the channel bandwidth and (Tb, Tc, R) discussion until related core part is stable

One issue is CBW, which is the function of TchipD2R, TbitD2R and RSFS.

| Tb (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |
| 22.22 |  |  |  |  |  |  |  |  |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 |
| 11.11 |  |  |  |  |  |  |  |  |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 |
| 5.56 |  |  |  |  |  |  |  |  |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 |
| 2.78 |  |  |  |  |  |  |  |  |
| 2.08 |  |  |  |  |  |  |  | R=1 |
| 1.39 |  |  |  |  |  |  |  |  |
| 1.04 |  |  |  |  |  |  |  |  |

As starting point, we propose (TchipD2R, TbitD2R,R=1)= (133.33, 66.67, R=1)

Proposal 10: Use (TchipD2R, TbitD2R,R=1)= (133.33, 66.67, R=1) as start point.

Test methodology

One issue is test methodology, the agreement is:

- Agreement - 10% BLER with BLER=1- Number of responsed Message 2Number of transmitted Message 1 for testing - During countering the number of responded Message 2, the TE should check if the responded Message 2 is the correct response of Message 1.

The traditional way is that the specific test mode is applied to BS demod test so that BS directly report the results to the TE without any additional test methodology. RAN4 has never discussed the test methodology for BS demodulation test before, we think the logic can also be applied to A-Iot BS demodulation test.

Proposal 11: Don’t specify the test methodology for A-Iot BS demodulation test.

- Conclusion

In this contribution we provide our views on demodulation requirements for Ambient-Iot BS, the proposals are:

Proposal 1: Define PDRCH demodulation requirements

Proposal 2: RAN4 to configure 31 bits preamble and additional midamble

Proposal 3: The reader detection assumption should be up to BS implementation

Proposal 4:  Consider OOK modulation

Proposal 5:  Prefer option 1, consider no repetition.

Proposal 6:  Prefer 10% SFO and 1.92MHz device sampling rate.

Proposal 7: Define PDRCH demodulation requirements with message 3 reception.

Proposal 8: Use TBS 96 bits.

Proposal 9: Use TDLA30-10

Proposal 10: Use (TchipD2R, TbitD2R,R=1)= (133.33, 66.67, R=1) as start point.

Proposal 11: Don’t specify the test methodology for A-Iot BS demodulation test.

- Reference

[1] R4-2508116 WF on A-IoT_device, RAN4#115, CMCC