3GPP TSG-RAN WG4 Meeting #116bis				                                         R4-2513446

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.8.4

Title: 	Discussion on LB-CA RRM requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP#109, a new WI was agreed to enable LB-LB CA with swicthing in [1]. And the detailed WID is self-contained here for reference:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, we provided preliminary analyze on RRM impacts due to this new R20 LBCA WI below.

- RRM impacts:

  - Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC

  - SDL SCell related requirement.

- Discussion

As noted in the WID, the requirment of Rel19 LBCA can be reused as possible for these of Rel20 LBCA. And the switching pattern defined in Rel19 by RAN1 can be reused also. Thus

Observeration 1:  From RAN4 perspective, UE behavior for SDL Scell activation and measurements on SDL carrier (including L1 and L3 measurement) can be same as that for Rel19 LBCA.

Hereby, we firstly summarized the key difference between them.

Rel19 LBCA WID [] - Focus on FDD-SDL band with semi-static switching pattern - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier - Focus on collocated scenario with single TAG and aligned SCS 15kHz only

Observation 2: the key different scenario in the new Rel20 LBCA in comparison with Rel19 is when UE switching to SDL carrier, Rx on SDL carreir can be allowed as shown in Figure 1 below.

Figure 1.Rel20 LBCA scenario

Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC Agreement: RAN4 to confirm the agreement from the last RAN4 meeting - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

And as RAN4 agreed in Rel19 LBCA, there is not any interruption to the on the serving cells when switching between FDD and SDL carriers becasue

- No any scheduling data on FDD carrier when switching

- Only 1 FDD and 1 SDL carrier in Rel19 LBCA

Figure 2. Interruption in Rel19 LBCA

Observation 3：For LBCA in Rel19, there is no any DL in FDD when switching to SDL. That is there is no any data interruption during FDD switching to SDL.

Howerver, this argurment seems not valid for Rel20 LBCA anymore as illustrated in Figure 3 below. When UE switching to SDL DL, there are still DL from FDD. Thus, the interruption on these ongoing FDD DL is possbile.

Figure 3. Interruption in Rel20 LBCA

However, in Rel20 LBCA or 6G LBAC, the following scenario shall be studied.

Observation 4: When UE switching to SDL from FDD Pcell in Rel20/6G LBCA, there is interruption on PCell data reception, vice versa.

Proposal 1: The interruption requirements due to FDD/SDL switching shall be specified in TS38.133.

- Conclusion

In this contribution, serval issues related to RRM in Rel20 LB-CA are discussed. The observations and proposals can be summarized as:

Observeration 1:  From RAN4 perspective, UE behavior for SDL Scell activation and measurements on SDL carrier (including L1 and L3 measurement) can be same as that for Rel19 LBCA.

Observation 2: the key different scenario in the new Rel20 LBCA in comparison with Rel19 is when UE switching to SDL carrier, Rx on SDL carreir can be allowed as shown in Figure 1 below.

Observation 3：For LBCA in Rel19, there is no any DL in FDD when switching to SDL. That is there is no any data interruption during FDD switching to SDL.

Observation 4: When UE switching to SDL from FDD Pcell in Rel20/6G LBCA, there is interruption on PCell data reception, vice versa.

Proposal 1: The interruption requirements due to FDD/SDL switching shall be specified in TS38.133.

- References

- RP-252953

- TS38.133 v19.7.0