3GPP TSG-RAN WG4 Meeting #116bis	R4-2513452

Prague, Czech, Oct. 13-17, 2025

Source: 	Xiaomi

Title: 	Discussion on R20 AIoT co-existence simulation assumptions

Agenda item:	7.11.3

Document for:	Discussion

- Introduction

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

New SI on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices was approved in RAN#108 [1]. According to the SID, RAN4 objectives are as followings.

This is the first RAN4 meeting for this SI. This contribution provides our proposals for the co-existence assumptions based on SID and RAN1 agreements [2].

- Discussion

  - Co-existence scenarios

Deployment scenario 4 with topology 1 with no external carrier wave.

Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

Device types are device 2b and device C.

For the AIoT deployment scenarios, RAN1 assumes RMa, UMa and UMi in the discussion. For the detail deployment of each scenario, the following guidance from the SID is taken into account to consider the co-existence scenarios.

FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

According to the WID, there’s no deployment scenario difference for device 2b and device C. D4T1 is the only topology in the SID. For the spectrum, the following should be followed according to the SID.

There’s no clarification in the SID that if device 2b and C are dropped in the same cell, our understanding is that the two devices may be deployed separately because the output power capability of the two types are different thus lead to different cell coverage. However, it’s also feasible the coverage is derived from device 2b MOP capability then device C works in high data rate. But in the co-existence simulation, both of the device 2b and device C Tx emission/Rx selectivity performances are not fixed, it’s hard to drop the two types of UE in the same simulation to derive both of the requirements. For device C, only dropping device C is a worst case than mixing device 2b and device C case, so it’s safer to simulate the two device types separately.

Proposal 1: AIoT device 2b and C are simulated separately in the RAN4 co-existence simulation.

For the deployment scenario, SID assumes A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. Even AIoT reader has the same coverage with NR BS, there’re 6 possible scenarios for each device, as shown in Table 1, because AIoT can be operated in standalone and in-band modes.

Table 1: R20 AIoT co-existence simulation scenario for both device 2b/C

| Device | Operating mode | AIoT – NR deployment scenario | Priority |
| --- | --- | --- | --- |
| Device 2b | Standalone, In-band | RMa – RMa | Medium |
| Device 2b | Standalone, In-band | UMa – UMa | High |
| Device 2b | Standalone, In-band | UMi – UMi | Low |
| Device C | Standalone, In-band | RMa – RMa | Medium |
| Device C | Standalone, In-band | UMa – UMa | High |
| Device C | Standalone, In-band | UMi – UMi | Low |

In our understanding, UMa should be taken as high priority if the time is limited.

Observation 1: 12 possible co-existence scenarios are summarized in Table 1.

Proposal 2: UMa scenario is taken as high priority for RAN4 co-existence simulation.

Agreement

For Rel-20 study, add a new item of “Penetration margin (dB)” into link budget template and the value of X dB for evaluation purpose is from the followings:

- For both R2D and D2R

  - X = {0, 20, 80} dB

- Note: Example use cases for the values above:

  - 20 dB for a Device inside a car with metallized car windows

  - 80 dB for a Device inside a metallic shipping container

  - Companies can report additional example use cases

For AIoT device dropping, SID clearly clarifies they’re outdoor but no further details. In RAN1#122, some agreements on “Penetration margin” were agreed as following,

We’re not sure if these specific cases should be taken into account in the co-existence simulation. In theory, if there’re some devices are in cars or shipping containers, the interference will be decreased. For simplification, RAN4 can only simulate the worst case, i.e. AIoT devices are dropped randomly outdoor.

Proposal 3: RAN4 only consider AIoT devices dropped randomly outdoor in RAN4 co-existence simulation.

For NR UE, indoor ratio should be discussed for UMa and UMi. The ratio in TR 38.803 can be taken as the starting point as shown in Table 3.

Table 2: Topology

| Deployment scenario | Topology |
| --- | --- |
| RMa | D4T1-C, AIoT device outdoor, NR UE outdoor |
| UMa | D4T1-C, AIoT device outdoor, NR UE outdoor and indoor, indoor ratio [20%] |
| UMi | D4T1-C, AIoT device outdoor, NR UE outdoor and indoor, indoor ratio [80%] |

Proposal 4: Topologies in Table 2 can be taken as starting point for RAN4 co-existence simulation.

  - Cases for co-existence study

The co-existence cases for R20 AIoT are similar with the NR adjacent channel co-existence simulation when the DL/UL spectrum directions are aligned with NR deployment. The aggressor/victim cases can be derived based on the DL/UL spectrum usage as shown in Table 3.

Table 3: Co-existence cases

| Case No. | Aggressor | Victim |
| --- | --- | --- |
| A | Reader | NR UE |
| B | NR BS | Device |
| C | Device | NR BS |
| D | NR UE | Reader |

Proposal 5: Co-existence cases for all of the co-existence scenarios are summarized in Table 3.

  - Device architecture and RF performance assumption

One of the most important issue for RAN4 simulation assumption is how to assume the device architecture and the RF performance metric. In R19 AIoT study, RFED architecture is assumed for passive devices. RAN4 discussion assumes no RF BPF filter for device 1, therefore no Rx rejection of NR DL signal is taken of the simulation assumption. The conclusion for the R19 co-existence simulation is that indoor scenario is the workable scenario for passive devices. Even indoor scenarios, severe interference can be observed when NR UEs are dropped indoor. Based on the R19 study, R20 outdoor scenarios should take better Rx selectivity and Tx emission performance assumption for device 2b/C to solve the interference between NR and AIoT systems. These performances are highly related to UE RF architecture.

Figure 1 and Figure 2 for IF-ED receiver and ZIF receiver architectures are copied from TR 38.769.

Figure 1: Architecture of device 2b with IF-ED receiver (copied TR 38.769 Figure 5.2.2.1-1)

For the IF-ED architecture, the performance of RF BPF, IF filter and IF-ED needs to the analysed because the signal of IF-ED output plays an important role for the Rx selectivity performance. RAN4 needs to discuss and reach agreement to continue the co-existence simulation.

Observation 2: For IF-ED architecture, Rx selectivity performance needs to be discussed to continue the co-existence simulation.

Figure 2: Architecture of device 2b with ZIF receiver (copied TR 38.769 Figure 5.2.2.1-1)

For ZIF receiver, digital filter can be used if ADC/DAC sampling rate, dynamic range performance is good enough. Then ACLR/ACS metric can be assumed in co-existence simulation.

Observation 3: For ZIF receiver, ACLR/ACS performance metric can be used in the co-existence simulation if ADC/DAC sampling rate, dynamic range and digital filter for Tx/Rx performance is good enough.

Agreement

For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose:

- Device 2b: 15 dB

  - FFS: 40dB as additional assumption for Device 2b with IF-ED

- Device C: 13 dB

In RAN1 #122 discussion, both ZIF architecture for device C and ZIF/IF-ED architecture is assumed for device 2b/C.

Observation 4: In RAN1 evaluation, both ZIF and IF-ED architectures are assumed but with the same NF.

It is not clear why the two architectures assume the same NF. To simplify RAN4 simulation, only use ACLR/ACS metric can be a choice. We propose to use ACLR/ACS as the device 2b/C Tx emission/Rx selectivity performance metric for RAN4 co-existence simulation.

Proposal 6: Use ACLR/ACS as the device 2b/C Tx emission/Rx selectivity performance metric for RAN4 co-existence simulation.

  - Simulation methodology

Generally, R19 methodology can be reused with some change for device Tx/Rx Tx emission and Rx selectivity performance metric. Based on the analysis in 2.3, ZIF architecture can use ACLR/ACS performance metric, but IF-ED architecture may need more discussion.

Proposal 7: R19 simulation methodology for both AIoT and NR are reused for R20 AIoT with the modification that device ACLR/ACS performance is evaluated.

  - Other simulation assumptions

For R20 co-existence simulation assumptions, most of them can be aligned with NR or R19 AIoT co-existence simulation. Some of them may need more discussion and decision in RAN4.

2.5.1 AIoT Reader ISD

In the SID, it is assumed that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. But

For Rel-20 study, it is up to RAN4 to decide the evaluation assumption on ISD for the study in outdoor scenarios.

The reader ISD is another parameter to be discussed in RAN4. The following is copied from RAN1#122 meeting minutes,

One question is that if reader ISD is the same with NR BS. In our understanding, the target is to have the same coverage for AIoT BS. So as the starting point, AIoT reader ISD and NR BS ISD are assumed the same in RAN4 co-existence simulation.

Proposal 8: AIoT reader ISD and NR BS ISD are assumed the same in RAN4 co-existence simulation as the starting point.

For UMa scenario, reusing R19 assumption may be a good choice, i.e. ISD is 750m. For RMa, if we consider 500m maximum distance between Reader and Device, then ISD can consider 1.5 km. For UMi, reusing TS 38.803 assumption can be considered, the ISD is 200m.

Proposal 9: For UMa ISD reuse R19 AIoT assumption 750m. RMa ISD use 1.5 km. UMi can reuse TS 38.803 assumption 200m.

2.5.2 Reader Tx power

For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings:

- For standalone: 43 dBm

- For in-band:

  - 38 or 33 dBm over R2D transmission BW

  - Note1: any necessary PSD limitation can be applied by RAN4

  - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations

In the RAN1 LS [3], the power for standalone is decided, but for the in-band scenario, it needs decision in RAN4.

In the SID, there’s no clear assumption that if AIoT reader for in-band operation shares hardware with NR BS. If they share hardware, the same PSD should apply. However, if the same PSD assumption applies, the AIoT reader power is limited. For example, for 10MHz CBW, 1RB power is 17 dB lower than 52 RB total power. If NR BS Tx power is 46 dBm, AIoT reader power is only 29 dBm which is smaller than either of the choices in RAN1 LS.

Observation 5: Shared hardware assumption limits the AIoT reader power for small R2D CBW.

Considering the purpose of co-existence simulation, maybe it’s safe to simulate higher power for AIoT reader. If high interference is found for the higher power, it can be considered to choose the lower power level. But of course, both of the two choices can be simulated by companies.

Proposal 10: Both 38 dBm and 33 dBm can be Tx options for companies to conduct the simulation. 38 dBm is in high priority.

For this proposal, we also provide a draft LS in [4].

2.5.3 R2D and D2R CBW for simulation

Agreement

For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

Agreement

For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth

For R2D and D2R CBW, RAN1#122 has some starting point as following,

RAN4 can use RAN1 starting point to start the co-existence simulation.

Proposal 11: 180kHz for R2D TBW and 15kHz for D2R TBW is used for the co-existence simulation.

2.5.4 Device Tx power control model

It’s not decided in RAN4 that whether there’s Tx power control for device 2b and device C. For device output power, there’s some information in SID and RAN1 agreement.

- Maximum transmit power between -3 dBm and +5 dBm for device C

  - RAN1 to recommend feasible values within this range

- Maximum transmit power of -20 dBm and -10 dBm for device 2b

The following guidance is in the SID.

Agreement

For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {-3, 0, 5} dBm

RAN1 agreed the Tx power assumption for evaluation purpose.

Before RAN1 can have some agreement on the device power control, a choice for RAN4 is to reuse NR power control model as the starting point for co-existence simulation. If there’s agreement in RAN1, this assumption can be revisited.

Proposal 12: NR UE power control model can be used as the starting point for RAN4 co-existence simulation, i.e.

In order to use this power control model, the device MOP and minimum output power should be decided, and also CLx-ile should be derived. Usually the minimum output power is derived from system analysis and considering the device RF capability. From the very beginning of this SI, it may be difficult to derive this requirement, NR UE minimum output power -40 dBm can be taken as a starting pint if there’s no other good proposal. For CLx-ile, it may be a little difficult that no SNR input from RAN1 yet.

Proposal 13: For calibration purpose, the device output power can use the following values:

Device 2b: minimum output power: -40 dbm, MOP: -10 dBm

Device C: minimum output power: -40 dbm, MOP: 5 dBm

Observation 6: There’s no SNR threshold from RAN1 yet, a starting point for CLx-ile is needed.

- Conclusion

This contribution provides our analysis for co-existence simulation assumptions.

For Co-existence scenarios, we have the following proposals,

Proposal 1: AIoT device 2b and C are simulated separately in the RAN4 co-existence simulation.

Table 1: R20 AIoT co-existence simulation scenario for both device 2b/C

| Device | Operating mode | AIoT – NR deployment scenario | Priority |
| --- | --- | --- | --- |
| Device 2b | Standalone, In-band | RMa – RMa | Medium |
| Device 2b | Standalone, In-band | UMa – UMa | High |
| Device 2b | Standalone, In-band | UMi – UMi | Low |
| Device C | Standalone, In-band | RMa – RMa | Medium |
| Device C | Standalone, In-band | UMa – UMa | High |
| Device C | Standalone, In-band | UMi – UMi | Low |

Observation 1: 12 possible co-existence scenarios are summarized in Table 1.

Proposal 2: UMa scenario is taken as high priority for RAN4 co-existence simulation.

Proposal 3: RAN4 only consider AIoT devices dropped randomly outdoor in RAN4 co-existence simulation.

Table 2: Topology

| Deployment scenario | Topology |
| --- | --- |
| RMa | D4T1-C, AIoT device outdoor, NR UE outdoor |
| UMa | D4T1-C, AIoT device outdoor, NR UE outdoor and indoor, indoor ratio [20%] |
| UMi | D4T1-C, AIoT device outdoor, NR UE outdoor and indoor, indoor ratio [80%] |

Proposal 4: Topologies in Table 2 can be taken as starting point for RAN4 co-existence simulation.

For cases for co-existence study, we have the following proposals,

Table 3: Co-existence cases

| Case No. | Aggressor | Victim |
| --- | --- | --- |
| A | Reader | NR UE |
| B | NR BS | Device |
| C | Device | NR BS |
| D | NR UE | Reader |

Proposal 5: Co-existence cases for all of the co-existence scenarios are summarized in Table 3.

For Device architecture and RF performance assumption, we have the following proposals,

Observation 2: For IF-ED architecture, Rx selectivity performance needs to be discussed to continue the co-existence simulation.

Observation 3: For ZIF receiver, ACLR/ACS performance metric can be used in the co-existence simulation if ADC/DAC sampling rate, dynamic range and digital filter for Tx/Rx performance is good enough.

Observation 4: In RAN1 evaluation, both ZIF and IF-ED architectures are assumed but with the same NF.

Proposal 6: Use ACLR/ACS as the device 2b/C Tx emission/Rx selectivity performance metric for RAN4 co-existence simulation.

For simulation methodology, we have the following proposals,

Proposal 7: R19 simulation methodology for both AIoT and NR are reused for R20 AIoT with the modification that device ACLR/ACS is evaluated.

For other simulation assumptions, we have the following proposals,

Proposal 8: AIoT reader ISD and NR BS ISD are assumed the same in RAN4 co-existence simulation as the starting point.

Proposal 9: For UMa ISD reuse R19 AIoT assumption 750m. RMa ISD use 1.5 km. UMi can reuse TS 38.803 assumption 200m.

Observation 5: Shared hardware assumption limits the AIoT reader power for small R2D CBW.

Proposal 10: Both 38 dBm and 33 dBm can be Tx options for companies to conduct the simulation. 38 dBm is in high priority.

Proposal 11: 180kHz for R2D TBW and 15kHz for D2R TBW is used for the co-existence simulation.

Proposal 12: NR UE power control model can be used as the starting point for RAN4 co-existence simulation, i.e.

Proposal 13: For calibration purpose, the device output power can use the following values:

Device 2b: minimum output power: -40 dbm, MOP: -10 dBm

Device C: minimum output power: -40 dbm, MOP: 5 dBm

Observation 6: There’s no SNR threshold from RAN1 yet, a starting point for CLx-ile is needed.

References

[1] RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108

[2] R1-2506567, “Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN1#122, Ad-Hoc Chair (Huawei)

[3] R1-2506575, “LS on BS Tx power assumption for coverage evaluation in A-IoT”, RAN1#122

[4] R4-2513451, “Draft reply LS on BS Tx power assumption for coverage evaluation in A-IoT”, RAN4#116b, Xiaomi