3GPP TSG-RAN WG4 Meeting #116bis	R4-2513468

Prague, Czech Republic, 13-17 October 2025

Agenda item:	6.10.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor core part was closed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on performance part.

- Discussion

L3 measurement enhancement by optimising Rx BSF

During the last meeting, the key requirement regarding T1 has reached a consensus, with a fixed value of 90 seconds agreed upon.

Issue 1-1-1: Value for T1 Agreement: - A fixed value of 90 seconds.

After defining the core requirement, it is necessary to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

Proposal 1: Define TC to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

In the last meeting, about when the UE will re-evaluate after the duration of T1 and the BSF condition is still met, RAN4 has no consensus reached. If the re-evaluation is up to the UE implementation, it is necessary to define a TC to test when or whether the UE can re-evaluate the condition. If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

Observation 1: If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

Proposal 2: RAN4 to discuss whether or not to define a TC for UE to re-evacuate BSF condition after T1, if the BSF condition is still met.

- Conclusions

Proposal 1: Define TC to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

Observation 1: If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

Proposal 2: RAN4 to discuss whether or not to define a TC for UE to re-evacuate BSF condition after T1, if the BSF condition is still met.

4. References

[1] R4-2512125 WF on RRM requirements for NR_RRM_Ph5_Part2