3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513473

Prague, Czech Republic, 13-17 October 2025

Agenda item:		6.18.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on Performance requirements for XR Phase 3

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we provide our views on potential impact on performance part of XR.

- Discussion

Gap-related issues

Issue 2-2-3: Per UE and per FR gaps <Way forward> Discuss the following options - Option 1: Test cases are specified only for per-UE MGs - Option 2: Test cases are specified for both per-UE and per-FR MGs Issue 2-2-4: Type 1 and Type 2 gaps <Way forward> Discuss the following options - Option 1: Test cases are defined for both Type 1 and Type 2 gaps - Option 2: Test cases are defined only for Type 1 gaps

In terms of the XR scenario in FR1/FR2-1/FR2-2, the type of measurement gap is supposed to consider both per-UE and per-FR MGs.

Proposal 1: RAN4 defines the test cases for both per-UE and per-FR MGs.

The type 1 gap for XR is the default option, and the type 2 with certain conditions. The agreement of the type 2 is shown as follows:

<Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS

Based on this, the test case of XR should consider both Type 1 and Type 2 gaps.

Proposal 2: RAN4 define the test cases for both Type 1 and Type 2.

DRX issues

Issue 2-2-5: DRX proposals <Way forward> Discuss the following proposals: - Proposal 1: Define the test cases for the cases without DRX and with DRX. - Proposal 2: Test cases without DRX should have higher priority than with DRX. - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG. - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority. - Focus on non-DRX test cases for Type 2 MGs. - Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

For XR, the typical scenario is non-DRX, for example, the XR device will listen to the PDCCH or PDSCH when service is running on it. Therefore, the test case for XR is supposed to cover the non-DRX scenario with high priority.

Proposal 3: The test cases for XR should be defined with the non‑DRX cases as a high priority.

DCI aspects issue

Issue 2-5-1: Toffset between DCI and start of MG <Way forward> - Option 1: Define test cases for both 3ms offset and 5ms offset. - Option 2: Define test cases for 5ms offset

RAN4 also introduce a capability to differentiate different processing times for MG skipping (e.g. 3ms and 5ms). Thus, if UE supports 3ms processing capability, it should be verified in the test cases. In detail, the DCI for MG skipping shall be transmitted 3ms before MG occasion.

Proposal 4: The processing time of 3ms/5ms shall be verified in the test cases.

Cancellation model for XR test cases

Issue 2-4-1: Cancelation model for XR test cases <Way forward> - Proposal 1: Measurement gap cancelation is determined randomly by the test equipment. - Proposal 2: For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern. - Other options are not precluded.

The gap cancellation is random for the XR device, which is defined in the core requirement. For test cases, to verify the function and mechanism, it is possible to set a fixed pattern for needing cancellation gaps. RAN4 needs to discuss how to define this fixed cancellation gap pattern.

Proposal 5: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern.

- Conclusions

Proposal 1: RAN4 defines the test cases for both per-UE and per-FR MGs.

Proposal 2: RAN4 define the test cases for both Type 1 and Type 2.

Proposal 3: The test cases for XR should be defined with the non‑DRX cases as a high priority.

Proposal 4: The processing time of 3ms/5ms shall be verified in the test cases.

Proposal 5: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern.

4. References

[1] R4-2512144 WF on RRM requirements for XR_Ph3