3GPP TSG-RAN WG4 Meeting #116bis	R4-2513500

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Initial discussion on RRM for (e)RedCap UE enhancement

Agenda Item:	7.4.3.2

Document for:	Discussion

Introduction

In RAN Plenary #109 meeting, WI for “NR Radio Resource Management (RRM) Phase 6” was agreed [1].

The objectives of WI are duplicated as below:

- Specify the following requirements for (e)RedCap UEs [RAN4]

  - Relaxed RLM/BFD RRM requirements

    - Use R17 RLM/BFD relaxation for legacy UE as a baseline

  - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

    - RF requirements, considering example bands: n100, n106, n26, n28 and n85

    - RRM requirements including relaxed RLM/BFD RRM requirements

    - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline

  - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

- For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4]

  - Note 3: No new PDSCH demodulation requirement shall be defined.

In this contribution the initial consideration on the RRM part for this WI is provided.

Discussion

2.1 Relaxed RLM/BFD RRM requirements for (e)Redcap

The RLM/BFD relaxation feature was introduced in Rel-17, the same release as the Redcap feature was initially specified for 5G. According to [2], RLM/BFD relaxation specified in Rel-17 UE power saving enhancements WI shall also apply for RedCap UE

- Power saving enhancement solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) shall be assumed to be available also to RedCap UEs by default. [2]

And in Rel-17 RedCap WI, RAN4 discussed whether and how to specify RLM/BFD relaxations from Rel-17 UE power-saving enhancements WI for Redcap UEs however in the end the relaxed RLM/BFD requirements has not been specified for Redcap due to different understanding regarding the scope of the Redcap WI.

Checking Rel-17 RLM/BFD relaxation, the specification is copied as below using RLM as an example:

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB_Relax period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB_Relax evaluation period.

TEvaluate_out_SSB_Relax is defined in table 8.1.2.4-1 for FR1.

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

The requirements are the maximum between the lower bond with/without relaxation and corresponding evaluation period relaxed by K1.

For the relaxed criteria, the Rel-17 RLM/BFD is based on the low mobility and not at cell edge (good serving cell). RAN4’s corresponding requirements are defined based on these criteria. The same criteria will be reused in this WI.

For the Redcap UE with 2Rx, there is no difference compared with a normal UE and the relaxed requirements for both SSB based and CSI-RS based RLM can be reused for Redcap with 2Rx directly. Same principle can also apply for both SSB based and CSI-RS based Rel-17 BFD relaxation on Redcap UE with 2Rx.

- For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation will be reused and no new criteria will be considered.

- For the Redcap UE with 2Rx, for both the SSB based and CSI-RS based RLM/BFD RRM relaxation, requirements defined in Rel-17, i.e., K3 and K1 and the corresponding configurations (i.e., Max(TDRX,TSSB) ≤80 ms; 80 ms < Max(TDRX,TSSB) ≤160 ms) when they apply, can be reused for corresponding Redcap with 2Rx requirements directly.

For the Redcap with 1Rx, the following update was made for the hypothetical PDCCH parameters RLM Qin and Qout.

Enhancements to hypothetical PDCCH parameters for SSB based- and CSI-RS based RLM Qout

- For RedCap UE with 1 Rx, for coverage compensation:

- CCE is changed from 8 to 16 and

- BW to 48 PRBs

Enhancements to hypothetical PDCCH parameters for SSB based and CSI-RS based RLM Qin

- For RedCap UE with 1 Rx, for coverage compensation:

- CCE is changed from 4 to 8

- BW to 48 PRBs

The above agreements are captured in the specs as:

Table 8.1B.2.1-1: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| NOTE: SCS = 60kHz is not applicable for FR1. |  |

And the corresponding requirements for Redcap UE with 1rx are:

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Compared with Redcap requirement with 2Rx as below when DRX cycle≤320 ms, the difference is the lower bound and evaluation are extended for Redcap UE with 1Rx.

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

To our understanding, the impact on requirements due to Redcap UE with 1Rx is fully addressed in the current specs. For the RLM/BFD RRM relaxation, at least one option is that the same value of K1 and K3 and corresponding configuration, can also be reused for Redcap UE with 1Rx.

- For the Redcap UE with 1Rx, consider the following option:

Option 1: For both the SSB based and CSI-RS based RLM/BFD RRM relaxation, requirements defined in Rel-17, i.e., K3 and K1 and the corresponding configurations (i.e., Max(TDRX,TSSB) ≤80 ms; 80 ms < Max(TDRX,TSSB) ≤160 ms) when they apply, can be reused for corresponding Redcap with 1Rx requirements.

2.2 Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

For normal NR UE, the following new requirements are introduced for 15 PRB transmission bandwidth configuration and 12 PRB SSB:

Intra-frequency cell identification

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil(7 x Kp) x SMTC period) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(7 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Intra-frequency measurements with measurement gaps

Table 9.2.6.2-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, 7 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 7 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Void NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Inter-frequency measurement with measurement gaps

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120 ms, Ceil(6 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120ms, Ceil(6 M2 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(6* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. |  |

Inter-frequency measurements without measurement gaps

Table 9.3.9.1-5: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_inter_less_than_5 MHz |
| --- | --- |
| No DRX | max(120 ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320 ms | max(120 ms, ceil (6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320 ms | Ceil( 6 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. |  |

For the Redcap with 2Rx, it is likely that the formal requirements can be reused. For the Redcap UE with 1Rx, simulation campaign may be needed to identify the corresponding requirements. For the relaxed RLM/BFD RRM requirements for Redcap UE with 3M bandwidth, further evaluate whether the same value of K1 and K3 can be reused for Redcap UE with 3M bandwidth with 1Rx and 2 Rx.

- For the Redcap UE less than 5 MHz channel bandwidth, discuss whether the existing intra-frequency cell identification, intra-frequency measurement, inter-frequency measurement requirements can be reused for Redcap UE less than 5 MHz channel bandwidth with 2 Rx.

- For the Redcap UE less than 5 MHz channel bandwidth with 1Rx, simulation campaign on intra-frequency cell identification, intra-frequency measurement, inter-frequency measurement requirements may needed.

- For the RLM/BFD RRM relaxation, further evaluate whether the same value of K1 and K3 and corresponding configuration where K1 and K3 apply defined in Rel-17 RLM/BFD relaxation WI can be reused for Redcap UE less than 5 MHz channel bandwidth with 1Rx and 2Rx.

Conclusion

In this contribution, we provide our initial considerations on the Rel-20 RRM enhancement phase 6 WI and have the following observations and proposals.

- For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation will be reused and no new criteria will be considered.

- For the Redcap UE with 2Rx, for both the SSB based and CSI-RS based RLM/BFD RRM relaxation, requirements defined in Rel-17, i.e., K3 and K1 and the corresponding configurations (i.e., Max(TDRX,TSSB) ≤80 ms; 80 ms < Max(TDRX,TSSB) ≤160 ms) when they apply, can be reused for corresponding Redcap with 2Rx requirements directly.

- For the Redcap UE with 1Rx, consider the following option:

Option 1: For both the SSB based and CSI-RS based RLM/BFD RRM relaxation, requirements defined in Rel-17, i.e., K3 and K1 and the corresponding configurations (i.e., Max(TDRX,TSSB) ≤80 ms; 80 ms < Max(TDRX,TSSB) ≤160 ms) when they apply, can be reused for corresponding Redcap with 1Rx requirements.

- For the Redcap UE less than 5 MHz channel bandwidth, discuss whether the existing intra-frequency cell identification, intra-frequency measurement, inter-frequency measurement requirements can be reused for Redcap UE less than 5 MHz channel bandwidth with 2 Rx.

- For the Redcap UE less than 5 MHz channel bandwidth with 1Rx, simulation campaign on intra-frequency cell identification, intra-frequency measurement, inter-frequency measurement requirements may needed.

- For the RLM/BFD RRM relaxation, further evaluate whether the same value of K1 and K3 and corresponding configuration where K1 and K3 apply defined in Rel-17 RLM/BFD relaxation WI can be reused for Redcap UE less than 5 MHz channel bandwidth with 1Rx and 2Rx.

- References

[1] RP-252955, New WID: NR RRM enh Phase 6, Apple (moderator), RAN 109

[2] RP-220966, Revised WID on support of reduced capability NR devices, Ericsson, RAN 95e

[3] R4-2206950, WF on RedCap RRM requirements, Ericsson, RAN4 102e