3GPP TSG RAN4 Meeting #116bis	R4-2513506

Prague, Czech Republic, October 13 - 17, 2025

Agenda item:		5.5.2

Source:		Union Inter. Chemins de Fer, Nokia

Title:		TP to TR 38.795: HPUE PC2 and PC1.5 in TDD band n101

Release:		19

Work Item:	HPUE_NR_FR1_bands_R19-Core

Document for:		Discussion and approval

# Introduction

In RAN4#109 the revised WID RP-252606 has been approved, allowing completion of the TR 38.795 and TR 38.796 at TSG#110. The objectives of this WID are:

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC2, PC1.5 and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC2 and PC1 for FWVM

This enables the introduction of Power Class 2 and 1.5 in TDD band n101 for railway FRMCS cab-radios (i.e. FWVM UE). It is noted that based on EU regulation, railway FRMCS cab-radios for PC2 and PC1.5 must meet an ACLR requirement of 37 dB minimum. It is also noted that in TS 38.101-1 PC1 has already been defined for band n100.

The implementation of FRMCS PC2 and PC1.5 is assumed to be based on FRMCS PC1 devices making use of high power PAs external to the chipset. The implication of this is that for FRMCS cab-radios at PC2 and PC1.5 power levels the existing PC1 requirements apply as described in section 2 below.

For information the following pictures of currently developed FRMCS PC1 cab-radio devices are provided:

This contribution provides a text proposal for TR 38.795 to introduce PC2 and PC1.5 for FRMCS cab-radios in band n101.

# Text Proposal

<Start of change>

7	High power for fixed-wireless/vehicle-mounted use cases in NR TDD bands with power class 1 UE

### 7.1	NR Bands nZ

### 7.1.1	REFSENS exception

<Text will be added.>

### 7.1.2	A-MPR and MPR

<Text will be added.>

8 	High power for fixed-wireless/vehicle-mounted use cases in NR TDD bands with power class 2 and 1.5 UE

8.1 	NR band n101

8.1.1 	UE maximum output power

Table 8.1.1-1: UE output power for FRMCS PC2 and PC1.5

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n101 |  |  | 291 | +2/-3 | 261 | +2/-3 | 23 | ±2 |
| NOTE 1: PC2 and PC1.5 in Band n101 are allowed only for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

8.1.2 	MPR requirements

For FRMCS PC2 and PC1.5, the MPR is defined as per TS 38.101-1 Table 6.2.2-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14.

8.1.3 	A-MPR requirements

For FRMCS PC2 and PC1.5 no A-MPR values are defined as there are no additional spurious emission requirements for band n101.

8.1.4 	ACLR requirements

As FRMCS cab-radios are required by EU regulation to have an ACLR of minimum 37dB, a new ACLR table  for PC2 and PC1.5 in band n101 is defined:

Table 8.1.4-1: NR ACLR requirement for FRMCS PC2 and PC1.5 in band n101

|  | Power class 1.5 | Power class 2 |
| --- | --- | --- |
| NR ACLR | 37 dB | 37 dB |
| NOTE: PC2 and PC1.5 in Band n101 is allowed only for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, that also support PC1. |  |  |

<End of change>

# References

[1]	RP-252589 TR 38.795 v1.0.0

[2]	RP-252606 NR_bands_xFeature_R19-Core

[3]	TS 38.101-1