3GPP TSG-RAN WG4 Meeting # 116bis					       			       			R4-2513515

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.15.1

Source:	Moderator (Huawei)

Title:	Topic summary for [116bis][209] Netw_Energy_NR_enh_RRM_Part2

Document for:	Information

Introduction

This topic summary includes RRM performance requirements for Rel-19 network energy saving (AI 6.15.2.2 and 6.15.2.3).

Recommended Topics to be treated online (in order of decreasing priority):

Topic #1: Adaptation of common signal/channel transmission

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement

Issue 1-2: General principle

Issue 1-3: SMTC and SSB configuration

Issue 1-4: How to verify SSB adaption

Issue 1-5: Whether to define TC for applying SSB adaptation to NCD-SSB (if time allows)

Topic #2: OD-SIB1

Issue 2-1: Whether to define TC for OD-SIB1

Issue 2-2: Detail test case design

Issue 2-3: TC scope

Topic #1: Adaptation of common signal/channel transmission

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513106 | OPPO | Observation 1: For testing SSB adaptation, RAN4 to focus on the new aspects like DCI triggering and new SMTC mapping to SSB periodicity。 Proposal 1: To reduce the amount of test cases, - RAN4 to test SSB adaptation for either Scell activation or for L3 measurement, and - not to define new test case for L1 measurement, and - not to define TC for SSB adaptation for L1/L3 transition requirement. Proposal 2：For SSB adaptation for L3 measurement, RAN4 to define the test cases for SMTC and SSB periodicity adaptation based on DCI 2-9. |
| R4-2513193 | CATT | Proposal 1: RAN4 to define L1 measurement test cases including the following also need to be defined for SSB adaptation: - L1-RSRP measurement - L1-SINR measurement - CBD Proposal 2: Regarding general principles to design the SSB adaptation test cases, the following can be supported: - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - Without SSB time index detection. |
| R4-2513369 | China Telecom | Proposal 1: RAN4 to design the following SSB adaptation test cases - L3 intra-frequency measurement - L1-RSRP - L1-SINR - CBD Proposal 2: RAN4 to design the SSB adaptation test case with the following principles - Both FR1 and FR2 - Only test non-DRX mode - Without SSB time index detection - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9 |
| R4-2513485 | Huawei, HiSilicon | Proposal 1: No test cases are defined for requirements at transition for L1/L3 for SSB adaptation as legacy. |
| R4-2513549 | Qualcomm Technologies Ireland | Proposal 1: Introduce test cases for SSB adaptation for L1-RSRP, L1-SINR, CBD and L3 measurements. Proposal 2: RAN4 should adopt the general principle in Issue 3-3-2 from RAN4 #116 how to define test cases for SSB adaptation. Proposal 3: RAN4 should adopt Option 3 in Issue 3-3-3 from RAN4 #116: SSB and SMTC periodicity changes from 80 ms to 20 ms and 20 ms to 80 ms. |
| R4-2513556 | LG Electronics Inc. | Proposal 1: RAN4 to define L1-RSRP for SSB adaptation test case Proposal 2: Following principle can be defined to design the SSB adaptation test case - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection. |
| R4-2513620 | Apple | Proposal 1: adding L1-RSRP in test case list is enough. No need to consider L1-SINR, CBD and any other L1 procedures. Proposal 2: we are fine with the following proposals: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection. |
| R4-2513707 | vivo | Proposal 1: RAN4 to define one test for L3 Intra-frequency serving Cell measurement. Proposal 2: For L3 Intra-frequency serving Cell measurement, the details are proposed as follows: - UE is configured with Event A1 or A2. - Condition of serving Cell is changed to satisfy the Event threshold at the same time as DCI 2-9 - It is verified whether UE can trigger event within the delay based on the mapped additional SMTC. |
| R4-2513887 | ZTE Corporation, Sanechips | Proposal 1: The following cases should be verified around the SSB adaptation: - The L1-RSRP/L1-SINR measurement on the SCell after SSB adaption in FR1 and FR2; - The CBD measurement/evaluation on the SCell after SSB adaption in FR1 and FR2; - L3 measurement with the adapted SMTC corresponding to the SSB adaptation. |
| R4-2513983 | Ericsson | Proposal 1: For SSB adaptation, RAN4 to define new test cases to verify SSB(SMTC) adaptation for L3 measurement. Proposal 2: For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation for L1-RSRP, L1-SINR and CBD. Proposal 3: RAN4 to define the SSB adaptation test cases with the following general princinples: - Both FR1 and FR2 - Only test non-DRX mode - without SSB time index detection - Reuse legacy trigger events for measurement report - RAN4 not to define the SSB adaptation test case for L1/L3 transition period Proposal 4: RAN4 to verify both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase. - SMTC periodicity from 80ms to 20ms - SSB periodicity from 80ms to 20ms Proposal 6: RAN4 to define SSB adaptation test cae as follow. No.; Test category; Test purpose. 1; SSB based L1-RSRP measurement when DRX is not used for SSB adaptation; L1-RSRP measurement. 2; L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used for SSB adaptation; L1-SINR measurement. 3; Beam Failure Detection and Link Recovery Test for FR2 SCell configured with SSB-based BFD and LR in non-DRX mode; Candidate beam detection. 4; SA event triggered reporting tests without gap for SSB adaptation; L3-RSRP measurement(agreed). |
| R4-2514209 | Samsung | Proposal 1: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation, and to deprioritize L1-SINR - New TC is to verify the measurement procedure when SSB periodicity adaptation is activated by NW signalling DCI 2_9 Proposal 2: The following principles to design the SSB adaptation test case can be accepted - In RRC_CONNECT mode only. - SMTC and SSB periodicity change based on DCI 2-9. - Reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - Without SSB time index detection. Proposal 3: TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation - Typical default SSB periodicity: 20 ms - To larger SSB periodicity: 80/160 ms; To denser SSB periodicity: 5/10 ms |
| R4-2514487 | Nokia, Nokia Shanghai Bell | - Proposal 1: Test cases for SSB adaptation should be introduced also for L1-RSRP, L1-SINR and CBD. - Proposal 2: For general testing principle, include test case for verifying L1/L3 transition requirement and study the additional impact for including a test case with SSB time index detection. - Proposal 3: RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa, for test case design. - Proposal 4: RAN4 to continue work on introducing a test case for applying SSB adaptation to NCD-SSB. |

Open issues summary

Sub-topic 1 SSB adaptation

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement

Background:

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD

- Besides L3 intra-frequency measurement, whether to specify the following test case for SSB adaptation

  - L1-RSRP measurement

  - L1-SINR measurement

  - CBD

  - SCell activation

  - L1/L3 transition

- Recommended WF

  - Further discussion.

Issue 1-2: General principle

- RAN4 to discuss the following principles to design the SSB adaptation test case.

  - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only

  - Both FR1 and FR2

  - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement

  - Only test non-DRX mode

  - without SSB time index detection

  - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

  - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Recommended WF

    - Further discussion

Issue 1-3: SMTC and SSB configuration

- Proposals

  - Option 1 (QC, Ericsson, Nokia):

      - SMTC periodicity from 80ms to 20ms, and vice vase.

      - SSB periodicity from 80ms to 20ms, and vice vase.

  - Option 2 (Samsung):

    - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation.

      - Default SSB periodicity: 20ms

      - Larger SSB periodicity: 80/160ms

      - Denser SSB periodicity: 5/10ms

- Recommended WF

  - Further discussion.

Issue 1-4: How to verify SSB adaption

- Proposals

  - Option 1(vivo): For L3 Intra-frequency serving Cell measurement:

- UE is configured with Event A1 or A2.

- Condition of serving Cell is changed to satisfy the Event threshold at the same time as DCI 2-9

- It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

- Recommended WF:

  - Further discussion.

Issue 1-5: Whether to define TC for applying SSB adaptation to NCD-SSB

- Proposals

  - Option 1 (Nokia): RAN4 to continue work on introducing a test case for applying SSB adaptation to NCD-SSB.

- Recommended WF:

  - Further discussion

Topic #2: OD-SIB1

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513107 | OPPO | Proposal 1: As for no test case for paging interruption in current spec, RAN4 not to introduce any new test case due to OD-SIB1 acquisition procedure in IDLE mode. |
| R4-2513194 | CATT | Proposal 1: RAN4 need to define a new test case to verify the OD-SIB1 procedure in IDLE mode with longer TSI-NR. Proposal 2: For following test procedure can be a reference: - UE transmit UL-WUS transmission and using PRACH - UE receive and decoding RAR - UE receive OD-SIB1 |
| R4-2513370 | China Telecom | Proposal 1: RAN4 to define test case with longer TSI-NR with reasonable value in OD-SIB1 performance part. |
| R4-2513486 | Huawei, HiSilicon | Proposal 1: For OD-SIB1, define TC as below. TC; Remarks. Cell reselection FR1 to FR1 target Cell; UL-WUS timing are verified in the same TC and TSI-NR. Cell reselection FR2 to FR2 target Cell; UL-WUS timing are verified in the same TC and TSI-NR. |
| R4-2513550 | Qualcomm Technologies Ireland | Proposal 1: No RRM test cases for OD-SIB1 should be defined in RAN4. Rather this can be left to RAN5 protocol testing. Proposal 2: In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested. |
| R4-2513621 | Apple | Proposal 1: RAN4 not to define OD-SIB1 RRM performance test case. |
| R4-2513708 | vivo | Proposal 1: RAN4 to define one Cell re-selection test case for OD-SIB1. The details are proposed as follows: - Target Cell (Cell2) is a NES Cell. - The ending point of the TC is UL WUS transmission. - UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR. |
| R4-2513984 | Ericsson | Proposal 1: RAN4 to define a new test case to verify the OD-SIB1 procedure in IDLE mode. |
| R4-2514488 | Nokia, Nokia Shanghai Bell | - Proposal 1: RAN4 to define a test case for verifying longer interruption in case of OD-SIB1 support in NES cell, taking into account additional delay from following components: - Duration of UL WUS transmission to NES cell, - Duration of RAR reception and message decoding, - Duration of OD-SIB1 window. Observation 1: No paging interruption test for idle or inactive mode exists in TS 38.133. Observation 2: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1. - Proposal 2: RAN4 to specify a new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell, rather than specifying a test case for paging interruption. |

Open issues summary

Sub-topic 2- TC for OD-SIB1

Issue 2-1: Whether to define TC for OD-SIB1

Background

Agreements in RAN4#115

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

- Proposals

  - Option 1 (CATT, China Telecom, Huawei, vivo, Ericsson, Nokia):

    - RAN4 to define cell reselection TC with longer TSI-NR for OD-SIB1.

  - Option 2 (OPPO, Apple, Qualcomm):

    - RAN4 not to define OD-SIB1 RRM performance test case.

- Recommended WF

  - Further discussion.

Issue 2-2: Detail test case design

- Proposals

  - Option 1(CATT, Nokia, Ericsson): RAN4 to define a new test case to verify the cell reselection behaviour together with longer TSI-NR in case of OD-SIB1.

    - UE transmits UL-WUS transmission

    - UE receives and decodes RAR

    - UE receives OD-SIB1 within OD-SIB1 window

  - Option 1(Qualcomm): In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested.

  - Option 3(vivo):

    - Target Cell (Cell2) is a NES Cell.

    - The ending point of the TC is UL WUS transmission.

    - UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

- Recommended WF

  - Further discussion.

Issue 2-3: TC scope

- Proposals

  - Option 1(Huawei):

    - Cell reselection FR1 to FR1 target Cell

    - Cell reselection FR2 to FR2 target Cell

- Recommended WF

  - Further discussion.