3GPP TSG-RAN WG4 #116bis	R4-2513550

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.15.2.3		On-demand SIB1

Source:	Qualcomm Incorporated

Title:	On performance requirements for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. In RAN4 #116 the work on the core requirements has ended and the work on the performance requirements was started.

In this paper we provide our view for the open issues identified in the way forward in [2] in RAN4 #116.

# Discussion

Issue 3-2-1: Test case

- Background

In RAN4 #115 meeting, Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

In RAN4 #115 and RAN4 #116, the following proposals were made for the definition of performance test cases for on-demand SIB1 [1], [2].

In our view, it is sufficient to test UL WUS transmission and OD-SIB1 reception as part of cell reselection. No separate tests for UL WUS and OD-SIB1 reception are needed. We believe that OD-SIB1 reception is not related to RRM performance but rather to protocol or functional performance testing. This can be more adequately tested in RAN5 protocol testing rather than RRM RAN4 performance testing.

Proposal 1: No RRM test cases for OD-SIB1 should be defined in RAN4. Rather this can be left to RAN5 protocol testing.

If RAN4 believes that a RRM performance test case should be introduced, it should be tested that the cell re-selection delay to a newly detectable cell and to an already detected cell is within Δ1 and Δ2 seconds. Default assumption for TSI-NR could be 1280 ms for FR1 intra-frequency, perhaps adding an additional Δ. There is no need to specify more details on UL WUS transmission and OD-SIB1 reception.

Proposal 2: In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

Proposal 1: No RRM test cases for OD-SIB1 should be defined in RAN4. Rather this can be left to RAN5 protocol testing.

Proposal 2: In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1, RAN4 #116, August 2025