3GPP TSG-RAN4 Meeting #116bis	R4-2513574

Prague, Czech, Oct. 13-17, 2025

Agenda item:	 4.13.1.2

Source:	 vivo

Title:	Discussions on NTN OTA testing time reduction

Document for:	Approval

- Introduction

The CTIA Certification OTA Working Group recently finalized the inclusion of NB-IoT NTN OTA testing for NTN bands 255 and 256 as informative in Version 9 of the CTIA OTA Test Plan.

CTIA Certification has approved TRP and TIS OTA test cases for NB-IoT NTN using the 3GPP TS 36.521-4 test procedures for maximum output power and reference sensitivity as the baseline for TRP and TIS, respectively. To address excessive test time concerns, especially the sensitivity search for TIS, the approved OTA test procedures deviate from current 3GPP TS 36.521-4 test procedures as follows:

- For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms.

- For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power

- For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured.

The CTIA Certification OTA Working Group elected to include NB-IoT NTN OTA testing as only informative in Version 9 of the CTIA Certification OTA Test Plan to allow time for system provider validation of the deviations listed above. The CTIA Certification OTA Working Group expects to transition the NB-IoT NTN OTA testing to normative in Version 10 of the CTIA Certification OTA Test Plan pending successful authorization of test equipment; additionally, CTIA Certification plans to incorporate NR NTN OTA test cases in Version 10. Version 9 of the CTIA Certification OTA Test Plan is planned for release by the end of this year. Version 10 of the CTIA Certification OTA Test Plan is planned for release in the second half of 2026.

The CTIA Certification OTA Working Group kindly requests that RAN4 and RAN5 consider incorporation of the deviations listed above when considering the development of corresponding NB-IoT NTN OTA test cases.

RAN4 received the LS from CTIA on NB-IoT NTN testing time reduction and improved test configurations [1].

This contribution shares our views on NTN OTA testing time reduction and potential reply LS to CTIA OTA WG.

- Discussion

OTA testing time is the key discussion point when developing the OTA test methodology for requirements verification. NTN OTA is a new thing to the group, given less commercialized smartphones in the market, very limited measurement results and testing time estimation of this type of device is presented.

In CTIA, some companies perform conducted measurement and estimate testing time in [3], some observations are in table below:

Table 1: Estimate TIS test time using different Koffsets and UL padding settings [3]

| Case | Feeder Link | Service Link | UL Padding | Time for measuring 67 samples | Estimated TIS Test Time with Search Algorithm Optimization |
| --- | --- | --- | --- | --- | --- |
| #1 | 0ms | 258ms | ON | 54.6 s | 32.3 hr |
| #2 | 0ms | 258ms | OFF | 19.4 s | 11.5 hr |
| #3 | 0ms | 0ms | ON | 21.0 s | 12.4 hr |
| #4 | 0ms | 0ms | OFF | 2.2 s | 1.3 hr |
| Note: For case #1 and #2, we utilized the default satellite location used by Keysight UXM and the UE was located on the earth. For case #3 and #4, the settings used are the proposed locations where the UE and satellite are co-located. |  |  |  |  |  |

It shows that the testing time is much higher if the UE and satellite are non-collocated. In addition, some of the measurement results of conducted sensitivity are presented:

Table 2: The conducted sensitivity with different settings [3].

With that, the conclusion for NB-IoT NTN testing in CTIA OTA spec [4] is as following:

Unlike the non co-located configurations defined in 3GPP TS 36.508 [13], for OTA performance testing the LTE communication tester shall implement a co-located configuration where the UE location (defined in Table 4.7.1-1) is the same as the satellite location and k-Offset is set to 0ms. The UE location according to Table 4.7.1-1 is provided to the UE through any preconfigured means.

P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT's transmitter is at maximum output power during the sensitivity searches. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NB-IoT NTN sensitivity level. The minimum RF power level resulting in a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel shall be recorded as the downlink power level corresponding to the 95% throughput percentage (the data throughput rate is as defined in section 7.3B of 3GPP TS 36.521-4 [21]). C-TIS shall be based on the minimum RF power level resulting in a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement. Care must be taken to ensure that the duration of the throughput measurement is sufficient to achieve statistical significance according to Annex G.2 of 3GPP TS 36.521-4 [21]. The downlink signal level for each DUT test condition shall be recorded for integration pursuant to CTIA 01.20 [1] or CTIA 01.21 [2] to give a single figure of merit referred to as Combined Total Isotropic Sensitivity (C-TIS).

To reduce test time, TIS and EIS testing shall be performed with no uplink PUSCH allocation. That is, the uplink is control only with NRU (resource unit) = 0 (no user plane PUSCH data shall be configured).

Among the three configuration parameters, in our understanding one of the key issue might be the yellow highlighted part that whether all the UEs can be configured with k-Offset=0ms and whether additional test mode is needed. This should be checked and confirmed in RAN4.

Regarding higher P0(Nominal PUSCH) for UL output power, this is reasonable to ensure a stable connection during DL testing, which is similar approach also adopted for DL MIMO OTA.

For no user plane PUSCH data configured during DL measurements, this is also reasonable for OTA testing.

Therefore, our proposal is to adopt these three configurations as alternative for NB-IoT NTN OTA testing time reduction. Meanwhile, further study in RAN4 on whether k-Offset=0ms can be configured in any UE via pre-configuration or specific test mode. And further study in RAN4 on testing time reduction with/without using above alternative parameters.

Proposal 1: RAN4 adopt the improved configurations in LS as alternative for NB-IoT NTN OTA testing to reduce testing time.

Proposal 2: RAN4 further discuss the feasibility of setting k-Offset=0ms for all NB-IoT NTN UEs.

Proposal 3: RAN4 further discuss the testing time reduction with/without using above alternative parameters.

A reply LS to CTIA is also prepared in [5].

- Conclusion

In this contribution, we share the following proposals for NTN OTA testing time reduction:

Proposal 1: RAN4 adopt the improved configurations in LS as alternative for NB-IoT NTN OTA testing to reduce testing time.

Proposal 2: RAN4 further discuss the feasibility of setting k-Offset=0ms for all NB-IoT NTN UEs.

Proposal 3: RAN4 further discuss the testing time reduction with/without using above alternative parameters.

# References

- R4-2511714, LS on NB-IoT NTN OTA Testing, CTIA OTA WG, RAN4#116.

- R4-2512693, Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#116.

- OTA_2025_001_008 V6 Optimized NTN LTE Category NB1 OTA Testing

- CTIA 01.50 Wireless Technology 3GPP Radio Access Technologies V9.0.0 draft1_JF1

- R4-2513575, draft Reply LS to CTIA on NB-IoT NTN OTA Testing, vivo, RAN4#116bis.