3GPP TSG-RAN WG4 Meeting #116bis					R4-2513581

# Prague, Czech Republic, October 13-17, 2025

Agenda item:			5.10.3

Source:	SES, THALES, JSAT, Tejas Networks, Sharp

Title:	Ku Band Performance Part: An overview of the simulation work

Document for:	Discussion and Decision

# Introduction

In this contribution, we present an overview of the simulation work required for the Performance part of the Ku Band work.

To progress the simulation work we present several high-level assumptions for discussion.

# Discussion

There are four specifications that we need to fulfil as part of the Ku Band performance part and these are:

- TS 38.101-5

- TS 38.108

- TS 38.181

- TS 38.133

This document focus is on TS 38.101-5 and TS 38.108 specifications. The remaining specifications will be covered in future contributions.

In all above specifications, we focus our attention the Radiated part of the specification, namely Section 11.

To complete the work, we propose to conduct simulations on the FR1 and FR2 numerologies on the Shared and Control channels:

- PDSCH

- PDCCH

And using the following Sub-carrier spacings for FR1 and FR2 numerologies:

- 15 and 30 kHz SCS for FR1-NTN above 10 GHz

- 120 kHz SCS for FR2-NTN

Several tables would need to be updated to capture the simulation scenarios and parameters of the Ku-Band for FR1 and FR2 Numerologies.

The amended values and parameters in the tables are shown in Green.

# High Level Assumptions

To progress the simulation work, for the FR1 numerology, we propose the following parameters for the simulation:

FR1 Bandwidths: 50 MHz

FR1 Modulation Scheme: QPSK and 16 QAM

# Tables to be Updated

The following tables would have to be updated with new Ku Band Simulation Results.

## TS 38.101-5 Specification

Section 11.2.1.1.2- Applicability of requirements for optional UE features

Modified Table 11.2.1.1.2-1: Requirements applicability for optional UE features

| UE facture/capability [11] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR2-NTN | PDCCH | Clause 11.2.3.1.1.1 (Test 1-1, Test 1-2) |  |
|  | FR1-NTN | PDCCH | Clause 11.2.3.1.1.1 (Test 1-1, Test 1-2) | [TBD] FR1-NTN configuration in above 10 GHz |
| NR NTN scenario support (ntn-ScenarioSupport-r17) | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 2-1, Test 2-2, Test 2-3, Test 2-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “gso” |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 2-1, Test 2-2, Test 2-3, Test 2-4) | [TBD] FR1-NTN configuration in above 10 GHz |
|  | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “ngso” or is not configured. |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) | [TBD] FR1-NTN configuration in above 10 GHz |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR2-NTN | PDSCH | Clause 11.2.1.2.2.1 (Test 1-3, 2-3) |  |
|  | FR1-NTN | PDSCH | Clause 11.2.1.2.2.1 (Test 1-3, 2-3) | [TBD] FR1-NTN configuration in above 10 GHz |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR2-NTN | PDSCH | Clause 11.2.1.2.2.1 (Test 1-4, 2-4) |  |
|  | FR1-NTN | PDSCH | Clause 11.2.1.2.2.1 (Test 1-4, 2-4) | [TBD] FR1-NTN configuration in above 10 GHz |

Modified Table 11.2.2-1: Common test parameters

|  |  |  |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- | --- | --- |
| Parameter |  |  | Unit | Value |  |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |  |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |  |
|  | Subcarrier spacing |  | kHz | 120 | 15, 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |  |
|  | RB offset |  | RBs | 0 |  |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [15] for tested channel bandwidth and subcarrier spacing |  |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |  |
|  | SSB position in burst |  |  | First SSB in Slot #0 |  |
|  | SSB periodicity |  | ms | 20 |  |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |  |
|  | Symbols with PDCCH |  | Symbols | 0 |  |
|  | Number of PRBs in CORESET |  |  | Table 7.2-2 of 38.101-4 for tested channel bandwidth and subcarrier spacing |  |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |  |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |  |
|  | DCI format |  |  | 1_1 |  |
|  | TCI state |  |  | TCI state #1 |  |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | No precoding |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  | CDM Type |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS periodicity |  | Slots | 160 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS offset |  | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |  |
|  | QCL info |  |  | TCI state #0 |  |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 3 for 2 CSI-RS ports and 5 for 4 CSI-RS ports |  |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 160 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |  |
|  | QCL info |  |  | TCI state #1 |  |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 5 |  |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | 'FD-CDM2' |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 160 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |  |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 for CSI-RS resource 1,2 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |  |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 |  |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |  |
|  | CSI-RS periodicity |  | Slots | 160 for CSI-RS resource 1,2 |  |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |  |
|  | Repetition |  |  | ON |  |
|  | QCL info |  |  | TCI state #1 |  |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests |  |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |  |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 1 |  |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type D |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |  |
|  |  | QCL Type |  | Type D |  |
| PT-RS configuration |  |  |  | Not configured | TBD |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| HARQ ACK/NACK bundling |  |  |  | Not configured | Not used for GEO and MEO |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | No precoding |  |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 of 38.101-4 |  |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 of 38.101-4 |  |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [15] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |  |

Modified Table 11.2.2.1.1.1-2: Test parameters

|  |  |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- | --- |
| Parameter |  | Unit | Value |  |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 1 |  |
|  | Length (L) |  | 13 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4. |  |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4. |  |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-2 32 for Test 1-3 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes for Test 1-4 in which 4 disabled processes are randomly selected at test configuration | TBD: HARQ are disabled for GEO and MEO. |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 80 for Test 1-1, Test 1-2, Test 1-3 and Test 1-4 2080 for Test 2-1, Test 2-2, Test 2-3 and Test 2-4 |  |
| cellSpecificKoffset-r17 (Note 1) |  | Slots/15kHz | 64 for Test 1-1, Test 1-2, Test 1-3 and Test 1-4 2064 for Test 2-1, Test 2-2, Test 2-3 and Test 2-4 |  |
| Maximum number of HARQ transmission |  |  | 4 for Test 1-1, Test 1-2, Test 1-3 1 for Test 1-4 (re-Tx disabled for all HARQ processes) |  |
| Note 1: The number of slots between PDSCH and corresponding HARQ-ACK information contains the number of slots specified by cellSpecificKoffset-r17. |  |  |  |  |

Section 11.2.2.1.1-1 Minimum requirements for PDSCH Mapping Type A

Modified Table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel |  | Numerology | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.3-3.1 FDD |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 3-1 | [TBD] |  | FR1-NTN | 50 / 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 3-1 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 1-2 | R.PDSCH.3-4.1 FDD |  | FR2-NTN | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 3-2 | [TBD] |  | FR-NTN | 50 / 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 3-2 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 1-3 | R.PDSCH.3-3.1 FDD |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 3-3 | [TBD] |  | FR1-NTN | 50/ 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 3-3 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 1-4 | R.PDSCH.3-3.1 FDD (Note 1) |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| 3-4 | [TBD] |  | FR1-NTN | 50/ 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 3-4 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
|  |  | Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |
|  |  | Note 2 Tests for Ka Band covers FR2 Numerology and for Ku Band covers FR1 and FR2 Numerologies |  |  |  |  |  |  |  |
|  |  | Note 3 New Tests to be defined for FR1 Numerology and the Ku band |  |  |  |  |  |  |  |

Channel Model Parameters for FR1-NTN to be defined, see 7.1.

Modified Table 11.2.2.1.1.1-4: Minimum performance for Rank 1

| Test num. | Reference channel |  | Numerology | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.3-3.1 FDD |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 4-1 | [TBD] |  | FR1-NTN | 50 / 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 4-1 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 2-2 | R.PDSCH.3-4.1 FDD |  | FR2-NTN | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 4-2 | [TBD] |  | FR1-NTN | 50 / 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 4-2 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 2-3 | R.PDSCH.3-3.1 FDD |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 4-3 | [TBD] |  | FR1-NTN | 50 / 15 | TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 4-3 | [TBD] |  | FR1-NTN | 50 /30 | TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 2-4 | R.PDSCH.3-3.1 FDD (Note 1) |  | FR2-NTN | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| 4-4 | [TBD] |  | FR1-NTN | 50 / 15 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
| 4-4 | [TBD] |  | FR1-NTN | 50 / 30 | [TBD] | [TBD] | 1x1 | 70 | [TBD] |
|  |  | Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |
|  |  | Note 2 Tests for Ka Band covers FR2 Numerology and for Ku Band covers FR1 and FR2 Numerologies |  |  |  |  |  |  |  |
|  |  | Note 3 New Tests to be defined for FR1 Numerology and the Ku band |  |  |  |  |  |  |  |

Channel Model Parameters for FR1-NTN to be defined, see 7.1.

Modified Table 11.2.3-1: Common test Parameters

|  |  |  |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- | --- | --- |
| Parameter |  |  | Unit | Value | Value |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 | As in FR2 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |  |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |  |
|  | SSB position in burst |  |  | First SSB in Slot #0 |  |
|  | SSB periodicity |  | ms | 20 |  |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |  |
|  | Number of PDCCH candidates |  |  | 1 |  |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |  |
|  | TCI state |  |  | TCI state #1 |  |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | CSI-RS periodicity |  | Slots | 160 |  |
|  | CSI-RS offset |  | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |  |
|  | QCL info |  |  | TCI state #0 |  |
| NZP CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 8 CSI-RS resource 2: 9 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |  |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |  |
|  | Repetition |  |  | ON |  |
|  | QCL info |  |  | TCI state #1 |  |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |  |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type D |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |  |
|  |  | QCL Type |  | Type D |  |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |  |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211 [9] |  |  |  |  |  |

Modified Table 11.2.3.1.1-1: Minimum performance requirements with 1 Tx Antenna

| Test number | Numerology | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | FR2-NTN | 200 | 132 | 1 | 8 | R.PDCCH.1-1.1 FDD | NTN-TDLC5-1200 | 1x1 Low | 1 | 4.6 |
| 2-1 | FR1-NTN | 50 | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| 1-2 | FR2-NTN | 200 | 132 | 2 | 16 | R.PDCCH.1-1.2 FDD | NTN-TDLC5-1200 | 1x1 Low | 1 | 3.2 |
| 2-2 | FR1-NTN | 50 | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |

Channel Model Parameters for FR1-NTN to be defined, see 7.1.

## TS 38.108 Specification

In this section we present the tables that need to be modified in TS 38.108 specification to accommodate for Ku Band FR1 combinations.

Radiated performance requirements

Modified Table 11.2.2.1.1-1: Test parameters for testing PUSCH

| Parameter |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- |
|  |  | Value | Value |
| Transform precoding |  | Disabled | [TBD] |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS symbols | Pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0} |  |
|  | DM-RS sequence generation | NID=0, nSCID =0 |  |
| Time domain resource | PUSCH mapping type | B |  |
|  | Start symbol index | 0 |  |
|  | Allocation length | 10 |  |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |  |
|  | Time density (LPT-RS) | Disabled |  |

Modified Table 11.2.2.1.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 50 MHz channel bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Number of demodulation branches | Numerology | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | FR2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-1 | pos1 | 0.0 |
|  |  | FR1 | Normal | [TBD] | 70 % | [TBD] | [TBD] | [TBD] |
|  |  | FR2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A6-1 | pos1 | 8.9 |
|  |  | FRR1 | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 2 | FR2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-1 | pos1 | -3.4 |
|  |  | FR1 | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | FR2 | Normal | NTN-TDLC5-1200 Low | 70% | G-FR2-NTN-A6-1 | pos1 | 5.5 |
|  |  | FR1 | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |

Modified Table 11.2.2.2.1-1: Test parameters for testing PUSCH

|  |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- |
| Parameter |  | Value | Value |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 | [TBD] |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS symbols | Pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0} |  |
|  | DM-RS sequence generation | NID=0, nSCID =0 |  |
| Time domain resource | PUSCH mapping type | B |  |
|  | Start symbol index | 0 |  |
|  | Allocation length | 10 |  |
| Frequency domain resource | RB assignment | 30 PRBs in the middle of the test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |  |
|  | Time density (LPT-RS) | Disabled |  |

Modified Table 11.2.2.2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 50 MHz Channel Bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Numerology | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FR2 | 1 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-2 | pos1 | 0.1 |
|  | FR1 | [TBD] | Normal | [TBD] | 70% | [TBD] | [TBD] | [TBD] |
|  | FR2 | 2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-2 | pos1 | -3.2 |
|  | FR1 | [TBD] | Normal | [TBD] | 70% | [TBD] | [TBD] | [TBD] |

Modified Table 11.2.2.3.1-1: Test parameters for testing PUSCH repetition Type A

|  |  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- | --- |
| Parameter |  | Value |  |
| Transform precoding |  | Disabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 | [TBD] |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS symbols | Pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | 0 |  |
|  | DM-RS sequence generation | NID=0, nSCID =0 |  |
| Time domain resource | PUSCH mapping type | B |  |
|  | Start symbol index | 0 |  |
|  | Allocation length | 10 |  |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |  |
|  | Time density (LPT-RS) | Disabled |  |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |  |

Modified Table 11.2.2.3.2-1: Minimum requirements for PUSCH, TypeB, 50 MHz channel bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Numerology |  | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FR2 |  | 1 | Normal | NTN-TDLC5-1200 Low | 1% (Note1) | G-FR2-NTN-A3A-3 | Pos1 | -8.6 |
|  | FR1 |  | 1 | Normal | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  | FR2 |  | 2 | Normal | NTN-TDLC5-1200 Low | 1% (Note1) | G-FR2-NTN-A3A-3 | Pos1 | -12.0 |
|  | FR1 |  | 2 | Normal | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | NOTE 1: BLER is defined as residual BLER, i.e. ratio of incorrectly received transport blocks/sent transport blocks, independently of the number of HARQ transmission(s) for each transport block |  |  |  |  |  |  |  |

Modified Table 11.3.2.2.1-1: Test Parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Number of UCI information bits | 1 | [TBD] |
| Number of PRBs | 1 | [TBD] |
| First PRB prior to frequency hopping | 0 | [TBD] |
| Intra-slot frequency hopping | Enabled | [TBD] |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | [TBD] |
| Group and sequence hopping | neither | [TBD] |
| Hopping ID | 0 | [TBD] |
| Initial cyclic shift | 0 | [TBD] |
| First symbol | 12 for 2 symbols | [TBD] |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability | [TBD] |

Modified Table 11.3.2.2.2-1: Minimum requirements for PUCCH format 0 and 120 kHz SCS in FR2-NTN

| Number |  | Number of | Propagation conditions and |  |
| --- | --- | --- | --- | --- |
| of TX antennas | Numerology | demodulation branches | correlation matrix (Annex D) | 50 MHz |
| 1 | FR2-NTN | 1 | NTN-TDLC5-1200 Low | 6.3 |
|  | FR2-NTN | 2 | NTN-TDLC5-1200 Low | 1.3 |
|  | FR1-NTN | [TBD] | [TBD] | [TBD] |
|  | FR1-NTN | [TBD] | [TBD] | [TBD] |

Modified Table 11.3.2.3.1.1-1: Test Parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Number of information bits | 2 | [TBD] |
| Number of PRBs | 1 | [TBD] |
| Number of symbols | 14 | [TBD] |
| First PRB prior to frequency hopping | 0 | [TBD] |
| Intra-slot frequency hopping | enabled | [TBD] |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs– 1) | [TBD] |
| Group and sequence hopping | neither | [TBD] |
| Hopping ID | 0 | [TBD] |
| Initial cyclic shift | 0 | [TBD] |
| First symbol | 0 | [TBD] |
| Index of orthogonal cover code (timeDomainOCC) | 0 | [TBD] |

Modified Table 11.3.2.3.1.2-1: Minimum requirements for PUCCH format 1 with 120 kHz SCS in FR2-NTN and FR1-NTN in above 10 GHz

| Number |  | Number of | Cyclic | Propagation | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | Numerology | Demodulation Branches | Prefix | conditions and correlation matrix (Annex D) | 50 MHz |
| 1 | FR2-NTN | 1 | Normal | NTN-TDLC5-1200 Low | -0.2 |
|  | FR2-NTN | 2 | Normal | NTN-TDLC5-1200 Low | -5.3 |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |

Modified Table 11.3.2.4.1.1-1: Test Parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Modulation order | QSPK | [TBD] |
| Starting RB location | 0 | [TBD] |
| Intra-slot frequency hopping | N/A | [TBD] |
| Number of PRBs | 4 | [TBD] |
| Number of symbols | 1 | [TBD] |
| The number of UCI information bits | 4 | [TBD] |
| First symbol | 13 | [TBD] |
| DM-RS sequence generation | NID0=0 | [TBD] |

Modified Table 11.3.2.4.1.2-1: Minimum requirements for PUCCH format 2 with 120 kHz SCS in FR2-NTN and FR1-NTN in above 10 GHz

| Number |  | Number of | Cyclic | Propagation | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | Numerology | Demodulation Branches | Prefix | conditions and correlation matrix (Annex D) | 50 MHz |
| 1 | FR2-NTN | 1 | Normal | NTN-TDLC5-1200 Low | 5.0 |
|  | FR2-NTN | 2 | Normal | NTN-TDLC5-1200 Low | 0.3 |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |

Modified Table 11.3.2.4.2.1-1: Test Parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Modulation order | QSPK | [TBD] |
| First PRB prior to frequency hopping | 0 | [TBD] |
| Intra-slot frequency hopping | enabled | [TBD] |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) | [TBD] |
| Number of PRBs | 9 | [TBD] |
| Number of symbols | 2 | [TBD] |
| The number of UCI information bits | 22 | [TBD] |
| First symbol | 12 | [TBD] |
| DM-RS sequence generation | NID0=0 | [TBD] |

Modified Table 11.3.2.4.2.2-1: Minimum requirements for PUCCH format 2 with 120 kHz SCS in FR2-NTN and FR1-NTN in above 10 GHz

| Number |  | Number of | Cyclic | Propagation | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | Numerology | Demodulation Branches | Prefix | conditions and correlation matrix (Annex D) | 50 MHz |
| 1 | FR2-NTN | 1 | Normal | NTN-TDLC5-1200 Low | 4.2 |
|  | FR2-NTN | 2 | Normal | NTN-TDLC5-1200 Low | -1.6 |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |
|  | FR1-NTN | [TBD] | Normal | [TBD] | [TBD] |

Modified Table 11.3.2.5.1-1: Test parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Modulation order | QPSK | [TBD] |
| First PRB prior to frequency hopping | 0 | [TBD] |
| Intra-slot frequency hopping | enabled | [TBD] |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | [TBD] |
| Group and sequence hopping | neither | [TBD] |
| Hopping ID | 0 | [TBD] |
| Number of PRBs | 1 | [TBD] |
| Number of symbols | 14 | [TBD] |
| The number of UCI information bits | 16 | [TBD] |
| First symbol | 0 | [TBD] |

Modified Table 11.3.2.5.2-2: Required SNR for PUCCH format 3 with 120kHz SCS in FR2-NTN and FR1-NTN in above 10 GHz

| Number of TX |  | Number of demodulation | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel Bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | Numerology | branches |  | correlation matrix (Annex D) | configuration | 50 MHz |
| 1 | FR2-NTN | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 2.3 |
|  |  |  |  |  | Additional DM-RS | 1.7 |
|  | FR2-NTN | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -1.9 |
|  |  |  |  |  | Additional DM-RS | -2.5 |
| 1 | FR1-NTN | 1 | Normal | [TBD] | [TBD] | [TBD] |
|  | FR1-NTN | 2 | Normal | [TBD] | [TBD] | [TBD] |

Modified Table 11.3.2.6.1-1: Test parameters

|  | FR2 Numerology | FR1 Numerology |
| --- | --- | --- |
| Parameter | Value | Value |
| Modulation order | QPSK | [TBD] |
| First PRB prior to frequency hoppingstartingPRB | 0 | [TBD] |
| Number of PRBs | 1 | [TBD] |
| Intra-slot frequency hopping | enabled | [TBD] |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) | [TBD] |
| Group and sequence hopping | neither | [TBD] |
| Hopping ID | 0 | [TBD] |
| Number of symbols | 14 | [TBD] |
| The number of UCI information bits | 22 | [TBD] |
| First symbol | 0 | [TBD] |
| Length of the orthogonal cover code | n2 | [TBD] |
| Index of the orthogonal cover code | n0 | [TBD] |

Modified Table 11.3.2.6.2-1: Required SNR for PUCCH format 4 with 120 kHz SCS in FR2-NTN and FR1-NTN in above 10 GHz

| Number of TX | Numerology | Number of demodulation | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel Bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| antennas |  | branches |  | correlation matrix (Annex D) | configuration | 50 MHz |
| 1 | FR2-NTN | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 4.1 |
|  |  |  |  |  | Additional DM-RS | 5.8 |
|  | FR2-NTN | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -0.2 |
|  |  |  |  |  | Additional DM-RS | -0.6 |
| 1 | FR1-NTN | 1 | Normal | [TBD] | [TBD] | [TBD] |
|  | FR1-NTN | 2 | Normal | [TBD] | [TBD] | [TBD] |

Modified Table 11.4.2.2.1-1: Time error tolerance for NTN-TDLC5-1200

| PRACH preamble | Numerology | PRACH SCS (kHz) | Time error tolerance |
| --- | --- | --- | --- |
|  |  |  | NTN-TDLC5-1200 |
| B4, C2 | FR2-NTN | 120 | 0.13 us |
| [TBD] | FR1-NTN | 15 | [TBD] |
| [TBD] | FR1-NTN | 30 | [TBD] |

Modified Table 11.4.2.2.2-1: PRACH missed detection test requirements, 120 kHz SCS

| Number of TX antennas | Numerology | Number of demodulation branches | Propagation conditions and correlation matrix (annex D) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | FR2-NTN | 1 | NTN-TDLC5-1200 Low | 3000 Hz | -6.7 | -3.3 |
|  | FR2-NTN | 2 | NTN-TDLC5-1200 Low | 3000 Hz | -11.9 | -8.6 |
|  | FR1-NTN | 1 | [TBD] | [TBD] | [TBD] | [TBD] |
|  | FR1-NTN | 2 | [TBD] | [TBD] | [TBD] | [TBD] |

# Parameters to be Clarified

Proposal 1: RAN4 to provide clarification to the following questions

- HARQ is likely to be disabled for GEO or MEO orbits.  For Test 1-1, Test 1-2, Test 1-3, and Test 1-4 with HARQ disabled, what simulation can we configure?

- What Bandwidth and SCS should we assume for Ku Band?  50 MHz and 15kHz/30kHz?

- Are the modulation format and coding rate fixed and the same as Ka Band?

- Can the Ka Band results be re-used or do we need new set of results for Ku Band?

- What channel model to be used for Ku band FR1-NTN?  What doppler to be used for Ku band FR1-NTN?

- What BLER is required for the simulations?

- Presentation format of the simulated results

Proposal 2: Rel-19 Ku-band will reuse (as much as possible, and where applicable) FR2-NTN demodulation configurations from the Ka-band (from Rel-18).

Proposal 3: Rel-19 Ku-band will add specific FR1-NTN demodulation configurations for above 10 GHz with similar TDL channels, Doppler and Delay spread values as for Ka-band.

# Conclusion

To progress forward with Ku Band Performance part the Test conditions, scenarios and output from simulations need to be clarified and agreed. If Ka Band results cannot be reused for Ku Band, we need to provide simulation results based on updated parameters for the following tables:

TS38.101-5: Table 11.2.2.1.1.1-3, Table 11.2.2.1.1.1-4, Table 11.2.3.1.1-1

TS38.108: Table 11.2.2.1.2-1, Table 11.2.2.2.2-1, Table 11.2.2.3.2-1, Table 11.3.2.3.1.2-1, Table 11.3.2.3.2.2-1, Table 11.3.2.4.1.2-1, Table 11.3.2.4.2.2-1, Table 11.3.2.5.2-2, Table 11.3.2.6.2-1,Table 11.4.2.2.2-1

# References

## Combinations of channel model parameters [TS 38.101-5 section B.2.2]

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table B.2.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table B.2.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |
| NTN-TDLC5-1200 | NTN-TDLC5 | 1200Hz |

# Appendices

## Ku Band Specification Summary

| Category | FR1 (Ku Band) | FR2 (Ku Band) |
| --- | --- | --- |
| Bandwidths | 50 MHz, 100 MHz | 200 MHz |
| Subcarrier Spacing (SCS) | 15, 30 kHz | 120 kHz |
| Modulation Schemes | QPSK, 16QAM, 64QAM | QPSK, 16QAM, 64QAM |
| Duplex Mode | FDD | FDD |
| Cyclic Prefix | Normal | Normal |
| Reference Channels | R.PDSCH.3-3.1 (QPSK), R.PDSCH.3-4.1 (16/64QAM) | R.PDSCH.3-3.1 (QPSK), R.PDSCH.3-4.1 (16/64QAM) |
| Channel Model | NTN-TDLC5-1200 | NTN-TDLC5-1200 |
| Performance Metrics | TBD – Ku sim result | TBD – Ku sim result |