3GPP TSG-RAN WG4 Meeting #116bis														R4-2513640

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.6.4.2

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirements for NTN requirement enhancement phase 2

Document for:	Discussion

# Introduction

A new Work Item on Enhanced requirements for NR NTN and IoT NTN Phase 2 (NR_IoT_NTN_req_Ph2) has been approved in RAN Plenary #109 meeting [1]. In this contribution, we provide our initial view on the RRM impact.

The work plan on RF and RRM core part has been provided in our companion paper submitted in Agenda 7.6.2. Note that the completion date for Core Part is 2027.Q1 and for performance part is 2027.Q3 according to the WID.

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

The RRM discussion is planned to start with the objective of HD-FDD Phase 1 (Ku bands with FR1 numerology) [1]:

And the HD-FDD Phase 2 work (Ku band and Ka band with FR2 numerology) will commence upon completion of Phase 1 and RAN1/RAN2 availability. The DL intra-band intra-SAN CA objectives will commence upon completion of HPUE for NTN Core Part (PC1.5 for IoT-NTN) and RAN1/RAN2 availability.

# Potential RRM requirement impact

RRM impact of HD-FDD VSAT

The conclusions from the R19 NR NTN discussions on HD-FDD RedCap NTN can be leveraged. For example, the following clarifications apply:

- Requirements apply provided that at least one SSB/RS is available during a corresponding period.

This clarification would apply to the following VSAT RRM requirements as listed in Table 1.

Note that the RRM requirements for VSAT are covered in the section titled “FR2-NTN,” and the requirements defined for FR2-NTN apply for VSAT working operating in cells with FR1-NTN numerology, unless otherwise stated (see 3.6.12 in TS 38.133).

Table 1 RRM impact of HD-FDD VSAT

| Requirements | VSAT requirement in 38.133 | HD-FDD VSAT impact |
| --- | --- | --- |
| Cell reselection | 4.2C Cell Re-selection for NR UE for Satellite Access | Need clarification such as in 4.2E.2.5 Maximum interruption in paging reception - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration. |
| Handover | 6.1C.1.3 NR SAN FR2-NTN – NR SAN FR2-NTN Handover 6.1C.2.4 NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover | No impact - as no need UL during HO interruption |
| Satellite switching | 6.1C.3.3 NR SAN Satellite switching with re-synchronization for FR2-NTN | Need clarification |
| RRC Re-establishment | 6.2C.1.2.2 UE Re-establishment delay requirement for VSAT | Need clarification |
| UE UL timing accuracy | 7.1C UE transmit timing for Satellite Access | No impact. - As already condition of available SSB during 160 ms |
| RLM | 8.1C Radio Link Monitoring for Satellite Access | Need clarification such as in 8.1E.3 Requirements for CSI-RS based radio link monitoring - at least one RLM-RS sample must fall with DL occasion within an indication period. |
| inter-/intra-frequency | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | Need clarification |
| L1-RSRP | 9.5C L1-RSRP measurements for Reporting for satellite access | Need clarification |

Proposal 1: No impact of HD-FDD for the following VSAT RRM requirements

- Handover

- UE UL timing accuracy

Proposal 2: For HD-FDD, the following VSAT RRM requirements apply provided that at least one SSB/RS is available within the corresponding period:

- Cell reselection

- Satellite switching

- RRC Re-establishment

- RLM

- inter-/intra-frequency

- L1-RSRP

RRM impact of DL intra-band intra-SAN CA

Given that DL intra-band, intra-SAN CA will start after HPUE for the NTN Core Part (PC1.5 for IoT-NTN) and RAN1/RAN2 availability, Table 2 presents our initial RRM impact assessment for future discussion.

Table 2 RRM impact of DL intra-band intra-SAN CA

| Requirements | VSAT requirement in 38.133 | DL CA impact |
| --- | --- | --- |
| SCell Activation and Deactivation Delay | n.a. | To add 8.3C SCell Activation and Deactivation Delay for Satellite Access |
| inter-/intra-frequency | 9.2C NR intra-frequency measurements for SAN | To add intra-frequency measurement for deactivated SCell |

Observation 1: For DL intra-band, intra-SAN CA, the following VSAT RRM requirements are to be introduced:

- SCell Activation and Deactivation Delay

- intra-frequency measurement for deactivated SCell

# Summary

Our observations and proposals in this paper are summarized as below:

Proposal 1: No impact of HD-FDD for the following VSAT RRM requirements

- Handover

- UE UL timing accuracy

Proposal 2: For HD-FDD, the following VSAT RRM requirements apply provided that at least one SSB/RS is available within the corresponding period:

- Cell reselection

- Satellite switching

- RRC Re-establishment

- RLM

- inter-/intra-frequency

- L1-RSRP

Observation 1: For DL intra-band, intra-SAN CA, the following VSAT RRM requirements are to be introduced:

- SCell Activation and Deactivation Delay

- intra-frequency measurement for deactivated SCell

# References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated), RAN#109