# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513655

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.3

Source: 	CMCC

Title: 			Discussion on Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements was approved [1]. The objectives for RAN4 are duplicated as following.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models.

- Discussion

For inter-vendor training collaboration for two-sided AI/ML models, the target is to fully define/specify reference model. Direction C refer to fully standardized reference model (structure + parameters). Direction A, sub-option 3a-1 refer to specification of standardized encoder model structure plus parameter exchange.

As discussed in our contribution on general aspects, during Rel-19 RAN4 study, for the simulations for checking the feasibility of above testing options for CSI compression and prediction, AI/ML model backbone of CNN is in use. In RAN1 study phase, it was agreed to adopt Transformer as the backbone structure for temporal domain Case 0 in case of take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output. For the discussion on inter-vendor training collaboration, it is proposed to follow RAN1 assumption to use Transformer as the backbone structure.

Proposal 1: for discussion on reference model for inter-vendor training collaboration, it is proposed to use Transformer as the backbone structure.

According to the WID, RAN1 scalability study outcome need to be taken into account to define/specify reference model. For the model structure feasibility study of Case 0 with precoding matrix in spatial-frequency domain as input, following alternatives on scaling over token dimension (e.g., number of subbands, number of Tx ports, or combination thereof), feature dimension (e.g., number of Tx ports, number of subbands, or combination thereof), and payload configurations are studied in RAN1. It is proposed to take these aspects into account for RAN4 discussion.

Agreements in RAN1#120 meeting Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Proposal 2: for scalability, at least token dimension (e.g., number of subbands, number of Tx ports, or combination thereof), feature dimension (e.g., number of Tx ports, number of subbands, or combination thereof), and payload configurations are considered.

- Conclusion

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models. The proposals are:

Proposal 1: for discussion on reference model for inter-vendor training collaboration, it is proposed to use Transformer as the backbone structure.

Proposal 2: for scalability, at least token dimension (e.g., number of subbands, number of Tx ports, or combination thereof), feature dimension (e.g., number of Tx ports, number of subbands, or combination thereof), and payload configurations are considered.

- Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

[2] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface