3GPP TSG-RAN WG4 Meeting #116bis  		R4-2513685

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.7.4

Source: 	CMCC

Title: 			 Discussion on demodulation requirements for Rel-19 ATG

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed the ATG demodulation simulation assumption for DL CA, DL MIMO and UL MIMO feature. The WF has been approved in [1]. In this contribution, we further give our views on remaining test setups and test rules [2].

- ATG UE DL CA demodulation requirement

Issue 1-1: TDD pattern

- Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U.

Considering the popularity of 30D4S6U using in ATG scenario, we are fine to introduce 30D4S6U as one of the test parameter. Meanwhile, with the sprite of control the test burden, we propose only to test the 30D4S6U with the default HARQ mode, which is defined in Table A.1.2-2c for UE not supporting outOfOrderOperationDL-r16, and introduce the test applicability that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Proposal 1: Introduce TDD pattern 30D4S6U with following restrictions:

- Only test the default HARQ mode, which is applicable for UE not supporting outOfOrderOperationDL-r16.

- Introduce the applicability rule that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Issue 1-4: CA configuration selection rules

- Option 1: Define the CA configuration selection rules as

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Issue 1-5: Test applicability rules

- Option 1: Define the test rules as

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

In high level, we think the legacy Rel-16 CA configuration selection rule and test applicability rule can be reused for Rel-19 ATG CA, but some refinement need to be done with the consideration of ATG specific feature, including:

- Only intra-band contiguous CA and inter-band CA are included in WID

- For FDD 15kHz +TDD 30kHz case, (e.g., Example band combination: CA_n3A-n39A), FDD CC is configured as PCC

- Only FDD 15kHz and TDD 30kHz are supported for Rel-18 single carrier requirement

- In last meeting, RAN4 agreed only TDD 30 kHz + TDD 30 kHz and  FDD 15 kHz + TDD 30 kHz will be specified for ATG network.

- Rank 1 will be used when define the requirements

After the refinement, we propose following CA configuration selection rule and test applicability rule for Rel-19 ATG CA:

Proposal 2: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 3: Define the CA configuration selection rules for the UE with dual-polarization antenna as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- DL MIMO related UE demodulation requirement

Issue 2-1: TDD pattern

- Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U.

Considering the popularity of 30D4S6U using in ATG scenario, we are fine to introduce 30D4S6U as one of the test parameter. Meanwhile, with the sprite of control the test burden, we propose only to test the 30D4S6U with the default HARQ mode, which is defined in Table A.1.2-2c for UE not supporting outOfOrderOperationDL-r16, and introduce the test applicability that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Proposal 4: Introduce TDD pattern 30D4S6U with following restrictions:

- Only test the default HARQ mode, which is applicable for UE not supporting outOfOrderOperationDL-r16.

- Introduce the applicability rule that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Issue 2-2: MCS

Agreement:

- 64QAM MCS 22.

- FFS for 256QAM Table 2 MCS 24.

According to our simulation results, for the SNR of the 70% TP, both 64QAM MCS 22 and 256QAM MCS 24 are within the testable range, since only conducted test will be performed. Considering the DL throughput is the one of the most important target in ATG network, it is worthy test 256QAM with Rank2 to verify the maximum capacity. We propose to consider 256QAM Table 2 MCS 24 for DL-MIMO requirements.

Proposal 5: Introduce 256QAM Table 2 MCS 24 when define the DL-MIMO requirements.

- Conclusion

Based on the discussion above, the following proposals are concluded.

DL CA

Proposal 1: Introduce TDD pattern 30D4S6U with following restrictions:

- Only test the default HARQ mode, which is applicable for UE not supporting outOfOrderOperationDL-r16.

- Introduce the applicability rule that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Proposal 2: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 3: Define the CA configuration selection rules for the UE with dual-polarization antenna as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

DL MIMO

Proposal 4: Introduce TDD pattern 30D4S6U with following restrictions:

- Only test the default HARQ mode, which is applicable for UE not supporting outOfOrderOperationDL-r16.

- Introduce the applicability rule that if UE pass the legacy TDD pattern, the new TDD pattern test can be skipped, and vise versa.

Proposal 5: Introduce 256QAM Table 2 MCS 24 when define the DL-MIMO requirements.

- Reference

- R4-2512602, Way Forward for [116][326] NR_ATG_enh_demod, ZTE Corporation, CMCC

- R4-2513686 Simulation results for Rel-19 ATG, CMCC