3GPP TSG-RAN WG4 Meeting #116bis	R4-2513706

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 NES OD-SSB

Agenda Item:	6.15.2.1

Document for:	Discussion

- Introduction

The core part of Rel-19 NES was completed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on performance part for OD-SSB.

- Discussion

The performance part has been initially discussed in last meeting with following agreements.

Issue 3-1-1: Scenario - General - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 should not define fast SCell activation test cases for OD-SSB. - RAN4 only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB. - RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow. - Option 1: - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A. - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B. - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Option 2: RAN4 not to define test cases for scenario #3B. - Option 3: RAN4 to define the OD-SSB based test cases for - Case #1 and Scenario #2 - Case #2 and Scenario #2A - RAN4 to further discuss the test case scope for Case 2 as follow. - FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios. Issue 3-1-4: Test case list Agreement RAN4 to at least define the following OD-SSB test cases. - SCell activation in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 1 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Based on the discussion, it could be found that there are too many combination items of (case#1, case#2) and (Scenario 2/2A/3B). RAN4 should limited the number of introduced Test cases, and only the essential behaviour to be verified. As agreed in last meeting, the fast measurement window is a new behaviour introduced in Rel-19, and corresponding TC should be defined. In details, for case#2, only Alt Time-C1 needs to be verified since Alt time-C2 is not the typical scenario.

Proposal 1: For deactivated SCell measurement, define following test cases:

- TC1: Intra-frequency measurement (deactivated SCell) in Case 1

- TC2: Intra-frequency measurement (deactivated SCell) in Case 2 (Alt Time-C1)

For SCell activation, the enhancement is mainly for unknown SCell that UE can utilize OD-SSB to speed up the SCell activation procedure. For different SCell activation procedure, this part is common regardless of direct SCell/PUCCH SCell/Mulitple SCells.

Proposal 2: For SCell activation, only define TC for normal SCell activation, and do not define TC for other SCell activation procedure (e.g., direct SCell activation, PUCCH SCell activation, multiple SCells activations), since enhanced the UE behaviour is common.

Regarding whether to define known or unknown SCell activation, for case#1, there is no doubt that unknown case shall be defined. For case#2 Alt Time-C1, one particular UE behaviour may need verify is that UE can met known SCell requirements when the measurement is based on AO-SSB and the SCell activation is based on the union of AO-SSB and OD-SSB.

Proposal 3: For SCell activation, define following test cases:

TC1: Unknown SCell activation in case 1

TC2: known SCell activation in case 2 (Alt Time-C1). The measurement report is based on AO-SSB before SCell activation and the SCell activation is based on union of AO-SSB and OD-SSB.

For other test cases, we think they are not essential to be defined. For instance, UE use new SSB periodicity for L1-RSRP, or using new SMTC for L3 measurement.

Regarding the detailed test configurations, one critical issue is the SSB periodicity of OD-SSB/union of AO-SSB and OD-SSB. In legacy test cases, only 20 ms SSB is considered.

Proposal 4: For SSB periodicity configuration, the values are proposed as follows.

- Case 1: AO-SSB periodicity is [5 ms or 10ms].

- Case 2: AO-SSB periodicity 20ms, and the periodicity of union AO-SSB and OD-SSB is [5ms or 10ms].

- RAN4 to discuss and decide one value between 5 ms and 10 ms.

- Conclusion

Proposal 1: For deactivated SCell measurement, define following test cases:

- TC1: Intra-frequency measurement (deactivated SCell) in Case 1

- TC2: Intra-frequency measurement (deactivated SCell) in Case 2 (Alt Time-C1)

Proposal 2: For SCell activation, only define TC for normal SCell activation, and do not define TC for other SCell activation procedure (e.g., direct SCell activation, PUCCH SCell activation, multiple SCells activations), since enhanced the UE behaviour is common.

Proposal 3: For SCell activation, define following test cases:

TC1: Unknown SCell activation in case 1

TC2: known SCell activation in case 2 (Alt Time-C1). The measurement report is based on AO-SSB before SCell activation and the SCell activation is based on union of AO-SSB and OD-SSB.

Proposal 4: For SSB periodicity configuration, the values are proposed as follows.

- Case 1: AO-SSB periodicity is [5 ms or 10ms].

- Case 2: AO-SSB periodicity 20ms, and the periodicity of union AO-SSB and OD-SSB is [5ms or 10ms].

- RAN4 to discuss and decide one value between 5 ms and 10 ms.

References

[1] R4-2512327	WF on RRM requirements for Netw_Energy_NR_enh_Part1