3GPP TSG-RAN WG4 Meeting #116bis	R4-2513723

Prague, CZ, October 13rd – 17th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance requirements for OD-SSB SCell operation

Agenda item:	6.15.2.1

Document for:	Discussion

- 1. 	Introduction

Performance aspects for Rel-19 network energy saving [1] were discussed in last meeting. In this contribution we continue discussing the test cases for the respective features based on latest agreements and recommended WF [2]. Further discussion details can be found in the topic summary [3].

- OD-SSB SCell operation

- 2.1 Test scenarios

In last meeting, the following test scenarios are listed for discussion [2]. We will discuss the proposals which we have concerns.

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Test cases in scenario #3B

For OD-SSB SCell operation, RAN1 has identified different cases and scenarios as below, and made further down scoping or restriction on the details. Up to now, we understood the following cases and scenarios are agreed by RAN1 in this release:

- Case #1: No always-on SSB on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or “after SCell activation is completed and SCell is activated”

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or“After SCell activation is completed and SCell is activated”

In Case #1, since there is no always-on SSB on the cell, OD-SSB is at least needed in order to activate the SCell. Hence the first OD-SSB activation shall happen no later than when UE receives SCell activation command. Based on the latest RAN1 agreements, scenario #3B was also agreed but we understood this does not refer to the first OD-SSB activation as RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated. This means we can assume OD-SSB is always activated after UE receives SCell activation command. In summary, for Case 1 test cases design, we should consider scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command. There is no need to define test cases dedicated for scenario #3B.

Observation 1: In Case 1, RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated hence in Scenario #3B OD-SSB shall be always activated.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

In Case #2, there is always-on SSB on the cell hence OD-SSB is not a must for activating the SCell. Then all the scenarios listed above shall be considered.

Proposal #2: For Case 2, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

Test cases for T2

The UE behaviour for OD-SSB reactivation has been discussed for long time. Pending on if the OD-SSB activation command comes within T2 after it was deactivated, the UE is expected to perform OD-SSB based measurements in different ways. Hence it would be valuable to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2. This will also provide reference information to network implementation about OD-SSB activation/deactivation.

Proposal #3: For Case 1, RAN4 to define the test cases to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2.

Scenarios in Case 2

RAN1 has agreed on the different scenarios based on the time/frequency relation between AO-SSB and OD-SSB for Case 2, including Alt. Time C1, Alt. Time C2 and scenario 3. If RAN4 would agree to define the RRM requirements for all these scenarios, we should define the test cases according to verify the UE behaviour.

Proposal #4: For Case 2, RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if we would agree to define the RRM requirements for all these scenarios.

- 2.2 Test case list

The following test cases are being considered in last meeting.

Sub-issue 2: Test case list Issue 3-1-3: Test case list RAN4 to discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 1 - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

In our view, as RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated, the difference from legacy SCell activation is only OD-SSB periodicity is used instead of SMTC. Hence it is not necessary to verify all the SCell activation scenarios. In addition, multiple SCell scenarios are being discussed in [3]. Based on current agreement, we don’t see how activating multiple SCells could happen. We prefer to define the test cases only for single SCell activation.

Proposal #5: RAN4 to reduce the number of test cases for SCell activation and define the TCs only for single SCell activation.

Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving for the features OD-SSB SCell operation.

The following observations and proposals are made.

OD-SSB SCell operation

Observation 1: In Case 1, RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated hence in Scenario #3B OD-SSB shall be always activated.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

Proposal #2: For Case 2, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

Proposal #3: For Case 1, RAN4 to define the test cases to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2.

Proposal #4: For Case 2, RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if we would agree to define the RRM requirements for all these scenarios.

Proposal #5: RAN4 to reduce the number of test cases for SCell activation and define the TCs only for single SCell activation.

- References

[1] 	RP-251678, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, Apple, RAN#108

[2]	R4-2512136, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson, RAN4#115