# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513733

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.6.3

Source: 	LG Electronics

Title: 	Discussion on Enhanced requirements for NR NTN and IoT NTN Phase 2 in Rel-20

Document for:	Discussion

Introduction

In the RAN#109, the WID on “Enhanced requirements for NR NTN and IoT NTN Phase 2” was approved. In this discussion, we share our point of view for the HPUE requirements for IoT-NTN based on the approved WID [1].

Table 1 WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 [1].

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

Discussion

NB-IoT based IoT-NTN RF requirement for PC1.5

NB‑IoT‑based IoT‑NTN PC1 and PC2 were specified in Rel‑19, and PC1.5 is introduced in Rel‑20. Only half‑duplex operation is considered for NB‑IoT‑based IoT‑NTN, and similar to PC1 and PC2 in Rel‑19, both handheld and non‑handheld device types can be supported. For ACLR, it is expected that PC1.5 can reuse the PC3 ACLR values applied to PC1 and PC2 in Rel‑19. MPR and A‑MPR values for PC1.5 should be derived through simulation and incorporated into the RF requirements, considering regulatory constraints. Since only half‑duplex operation is assumed, defining RSD requirements for NB‑IoT‑based IoT‑NTN is not expected to be necessary.

Observation: Necessary considerations on NB-IoT based IoT-NTN RF requirement for PC1.5

- Duplexing: Only half-duplex operation is considered for NB-IoT-based IoT-NTN.

- Device types: Both handheld and non-handheld device types may be supported (consistent with PC1 and PC2 in Rel-19).

- ACLR: Reuse the PC3 ACLR values as in Rel-19 for PC1/PC2.

- MPR/A-MPR: Derive band-specific MPR and A-MPR values for PC1.5 by simulation and analysis considering regulatory constraints for the example bands; include the agreed values in the UE Tx requirements.

- Power boosting: Whether power boosting should be applicable to PC1.5 (similar to NB-IoT-based IoT-NTN PC2) is FFS.

- RSD: Since only half-duplex is considered, RSD requirements are not expected to be needed for NB-IoT-based IoT-NTN. If non-handheld implementations with lower filter rejection (< 60 dB) are envisaged in certain bands, RAN4 may revisit the need for defining RSD on a band-specific basis (see observations in Section 3).

Proposal 1: Based on the observation, RAN4 can update TS 36.102 as shown in the table below.

| Section | Expected changes/update |
| --- | --- |
| 6.2B Transmit power for category NB1 and NB2 6.2B.1 UE maximum output power for category NB1 and NB2 6.2B.2 UE maximum output power reduction for category NB1 and NB2 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE | For 6.2B.1: The MOP requirements should be introduced for the PC1.5. For 6.2B.2: The MPR requirement should be introduced based on the agreed simulation assumption on PC1.5. For 6.2B.3: The A-MPR requirement should be introduced for the PC1.5. |
| 6.5B Output RF spectrum emissions for category NB1 and NB2 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 | For 6.5B.3.3: Need to add the PC1.5 spectrum emission limit on “NS_02N” |

NR-NTN RF requirement for PC1.5

Table 2 WID on UE RF enhancements for NR FR1, Phase 5 [2].

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

The NR-NTN-related statement in WID[1] is as follows:

“The band-agnostic requirements for UE power classes specified for TN are expected to also apply to the corresponding NR-NTN power classes.”

As the TN requirements for PC1.5 FDD are scheduled for discussion in Release 20, as indicated in WID [2], the NR-NTN PC1.5 requirements are expected to be defined based on the outcome of the Release 20 discussions on TN PC1.5 FDD.

It was also noted during RAN#109 that there was a discussion on band-specific NR-NTN PC1.5 requirements, and it was suggested that such requirements could be addressed within the scope of the spectrum work item. Therefore, band-specific NR-NTN PC1.5 requirements, such as A-MPR, may be handled in the Release 20 spectrum work item.

Proposal 2: The band-agnostic NR-NTN PC1.5 requirements are expected to be defined based on the outcome of the Rel-20 discussion on TN PC1.5 FDD.

Proposal 3: The band-specific NR-NTN PC1.5 requirements, such as A-MPR, can be handled in the Rel-20 spectrum work item.

Conclusion

For NB-IoT based IoT NTN

Observation: Necessary considerations on NB-IoT based IoT-NTN RF requirement for PC1.5

- Duplexing: Only half-duplex operation is considered for NB-IoT-based IoT-NTN.

- Device types: Both handheld and non-handheld device types may be supported (consistent with PC1 and PC2 in Rel-19).

- ACLR: Reuse the PC3 ACLR values as in Rel-19 for PC1/PC2.

- MPR/A-MPR: Derive band-specific MPR and A-MPR values for PC1.5 by simulation and analysis considering regulatory constraints for the example bands; include the agreed values in the UE Tx requirements.

- Power boosting: Whether power boosting should be applicable to PC1.5 (similar to NB-IoT-based IoT-NTN PC2) is FFS.

- RSD: Since only half-duplex is considered, RSD requirements are not expected to be needed for NB-IoT-based IoT-NTN. If non-handheld implementations with lower filter rejection (< 60 dB) are envisaged in certain bands, RAN4 may revisit the need for defining RSD on a band-specific basis (see observations in Section 3).

Proposal 1: Based on the observation, RAN4 can update TS 36.102 as shown in the table below.

| Section | Expected changes/update |
| --- | --- |
| 6.2B Transmit power for category NB1 and NB2 6.2B.1 UE maximum output power for category NB1 and NB2 6.2B.2 UE maximum output power reduction for category NB1 and NB2 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE | For 6.2B.1: The MOP requirements should be introduced for the PC1.5. For 6.2B.2: The MPR requirement should be introduced based on the agreed simulation assumption on PC1.5. For 6.2B.3: The A-MPR requirement should be introduced for the PC1.5. |
| 6.5B Output RF spectrum emissions for category NB1 and NB2 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 | For 6.5B.3.3: Need to add the PC1.5 spectrum emission limit on “NS_02N” |

For NR-NTN

Proposal 2: The band-agnostic NR-NTN PC1.5 requirements are expected to be defined based on the outcome of the Rel-20 discussion on TN PC1.5 FDD.

Proposal 3: The band-specific NR-NTN PC1.5 requirements, such as A-MPR, can be handled in the Rel-20 spectrum work item.

- References

# [1] R4-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 (RAN4 VC, QC)

# [2] R4-252942, New WID: UE RF enhancements for NR FR1, Phase 5 (vivo)