3GPP TSG-RAN WG4 Meeting #116bis										R4-2513738

Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	5.11.2

Source: 	Fujitsu

Title: 	Discussion on system parameters for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

At the RAN#108 meeting, a spectrum-related work item [1] was approved to specify a new NR TDD 4.9GHz band for US operation. In this paper, we would like to provide our view on system parameters for US 4.9GHz new band.

# 2. Discussion

Firstly, the band numbering for this new band is discussed. The latest TS 36.101 and 38.101-1 states band 113 has already used as operating band for LTE based 5G terrestrial broadcast. Considering it, the next number, band n114, should be used for the new NR TDD 4.9GHz band.

Proposal 1: RAN4 to use n114 as the band number for new NR TDD 4.9GHz band.

Regarding the other system parameters, the contribution [2] brought at the August meeting has already analyzed them for new NR TDD 4.9GHz band. Those parameters can be used.

Proposal 2: RAN4 to technically endorse the system parameters proposed in R4-2509262.

For reference, those parameters are captured below.

Table 1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n112 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

Table 2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n112 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Table 3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n112 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Table 4  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n112 | 30 kHz | Case C | 8850 – <1> – 8877 |

# 3. Conclusions

In this contribution, we provided our view on system parameters for US 4.9GHz new band as follows.

Proposal 1: RAN4 to use n114 as the band number for new NR TDD 4.9GHz band.

Proposal 2: RAN4 to technically endorse the system parameters proposed in R4-2509262.

# 4. Reference

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- R4-2509262, System parameters for 4.9GHz new band, CATT