3GPP TSG-RAN WG4 Meeting #116bis										R4-2513740

Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	5.11.4

Source: 	Fujitsu

Title: 	Discussion on UE-RF requirements for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

At the RAN#108 meeting, a spectrum-related work item [1] was approved to specify a new NR TDD 4.9GHz band for US operation. In this paper, we would like to provide our view on UE-RF requirements for US 4.9GHz new band.

# 2. Discussion

Regarding the UE-RF requirements for US 4.9GHz, the emission mask M has been stated as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for high power transmitters (greater that 20 dBm).

Observation 1: Emission mask M has been stated as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for UE with its transmission power is greater than 20dBm.

(m) Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

The excerpt is as follows.

The above regulatory requirements need to be incorporated to the 3GPP spec. In the August meeting, some contributions [2][3] have compared the above emission mask and 3GPP SEM and concluded that the above emission mask is more stringent than the 3GPP SEM.

Observation 2: Emission mask M in the FCC Title 47 Part 90.210 (m) is more stringent than the 3GPP SEM.

Considering the above observations, A-MPR is necessary for satisfying the Emission mask M requirements. The following is our proposal.

Proposal 1: RAN4 to study the A-MPR values for satisfying the Emission mask M requirements.

Regarding the NS value, the latest TS 36.101 and 38.101-1 states NS_62 has already used. Cosidering it, the next number, NS_63 should be used as the network signaling for new A-MPR requirements for new NR TDD 4.9GHz band.

Proposal 2: RAN4 to use NS_63 as the network signaling for new A-MPR requirements for new NR TDD 4.9GHz band.

# 3. Conclusions

In this contribution, we provided our view on UE-RF requirements for US 4.9GHz new band as follows.

Observation 1: Emission mask M has been stated as the applicable emission masks for 4940-4990 MHz in the FCC Title 47 Part 90.210 (m) for UE with its transmission power is greater than 20dBm.

Observation 2: Emission mask M in the FCC Title 47 Part 90.210 (m) is more stringent than the 3GPP SEM.

Proposal 1: RAN4 to study the A-MPR values for satisfying the Emission mask M requirements.

Proposal 2: RAN4 to use NS_63 as the network signaling for new A-MPR requirements for new NR TDD 4.9GHz band.

# 4. Reference

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- R4-2510411, General aspects regarding the 4940–4990 MHz band, Qualcomm

- R4-2510429, Coexistence study for introduction of US 4.9GHz band, Huawei, HiSilicon