3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513758

# Prague, Czech Republic, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | xxxx | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS 38.101-3 to correct MSD test points for PC2 DC_12A_n78A and other configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct the MSD test points for PCell due to dual uplink operation for the following PC2 EN-DC configuration. - DC_12A_n78A - DC_5A_n41A - DC_26A_n41A - DC_71A_n41A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the MSD test points information with band and UL LCRB for PC2 DC_12A_n78A - Editorial corrections for MSD test points for PC2 DC_5A_n41A, DC_26A_n41A and DC_71A_n41A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MSD test points for the above mentioned PC2 EN-DC configurations will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1C_n3 | 1C | 1950 1970 | 20 20 | 1 (RBstart=0) 1 (RBstart=67) | 2140 2160 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 36 | IMD5 |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n71A DC_1A_n71B | 1 | 1958 | 5 | 25 | 2148 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | IMD3 |
| DC_1A_n77A DC_1A_SUL_n77A-n84A DC_1A_n77(2A) DC_1A_n77(3A) | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_1A_n78A DC_1A_SUL_n78A-n84A DC_1A_n78(2A) DC_1A_n78(A-C) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | IMD3 |
|  | n46 | 5720 | 20 | 100 | 5720 | N/A | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
| DC_2A_n66A DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n20A DC_3C_n20A DC_3A-3A_n20A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n26A | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3C_n26A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 | N/A |  |
|  | n26 | 841.5 | 15 | 25(RBSTART=54) | 886.5 | 18.9 | IMD3 |
| DC_3C_n28A | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 11 | IMD39 |
|  | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |
| DC_3A_n38A | 3 | 1712.8 | 5 | 25 | 1807.8 | 8.2 | IMD4 |
|  | n38 | 2616.7 | 10 | 50 | 2616.7 | N/A | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A DC_3A-3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n77A DC_3A_n77(2A) DC_3A_n77(3A) DC_3A_SUL_n77A-n80A DC_3A_n78A DC_3A_SUL_n78A-n80A DC_3A_n78(2A) DC_3A_n78(A-C) DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_4A_n2A | n2 | 1860 | 20 | 502 | 1940 | 5 | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A | N/A |
|  | n2 | 1868.3 | 5 | 25 | 1948.3 | N/A | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 | IMD5 |
| DC_4A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 4 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_4A_n7A | 4 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_5A_n3A | 5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | n3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_5_n38 | 5 | 844 | 5 | 25 | 889 | 12 | IMD33 |
|  | n38 | 2577 | 10 | 50 | 2577 | N/A | N/A |
| DC_5A_n41A | 5 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n77A8 DC_5A_n77(2A)8 DC_5A_n77(3A)8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78(A-C) DC_5A_n78C DC_5A_SUL_n78A-n89A | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n20A | 7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
| DC_7A_n26A DC_7C_n26A | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 12 | IMD33 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | N/A |
| DC_7A_n40A DC_7A-7A_n40A | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77A DC_7C_n77(2A) | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_7_n79 DC_7-7_n79 | 7 | 2510 | 5 | 25 | 2630 | [8] | IMD4 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |
| DC_8A_n1A DC_8B_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
| DC_8A_n3A DC_8B_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n38A | 8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_n77A DC_8B_n77A DC_8B_n77(2A) DC_8A_n78A DC_8B_n78A DC_8A_n78(2A) DC_8A_n77(3A) DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_8A_n79A, DC_8A_n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_11A_n28A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 10.4 | IMD4 |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_12A_n78A | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13A_n5A | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n5 | 828 | 5 | 25 | 873 | 25 | IMD3 |
| DC_13A_n7A DC_13A_n7(2A) | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n7 | 2520 | 40 | 216 | 2640 | 2.5 | IMD5 |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_14A_n5A | 14 | 791 | 5 | 25 | 761 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | 25 | IMD3 |
|  | 14 | 795.5 | 5 | 25 | 765.5 | 25 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_14A_n77A DC 14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |
| DC_18A_n77A10 | 18 | 827.5 | 5 | 25 | 872.5 | 8.4 | IMD410 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | 18 | 817.5 | 5 | 25 | 862.5 | 4.5 | IMD510 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| DC_18A_n78A | 18 | 827.5 | 5 | 25 | 872.5 | 8.4 | IMD411 |
|  | n78 | 3355 | 10 | 50 | 3355 | N/A | N/A |
| DC_19A_n77A | 19 | 836.5 | 5 | 25 | 881.5 | 13.6 | IMD43 |
|  | n77 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_19A_n78A | 19 | 836.5 | 5 | 25 | 881.5 | 13.6 | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | n8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | 20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_20A_n38A | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n41 | 20 | 851 | 5 | 25 | 810 | 12.1 | IMD3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 8.1 | IMD5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |
| DC_20A_n77A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n77 | 3359 | 10 | 50 | 3359 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_20A_n78A DC_20A_n78C7 DC_20A_n78(2A) DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 5 | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_26A_n77A DC_26A_n78A DC_26A_n78(2A) | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_28A_n77A DC_28C_n77A DC_28C_n77(2A) DC_28A_n78A DC_28A_n78(2A) DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_38A_n3A | n3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_38A_n8A | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |
| DC_40A_n7A | n7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | 2nd3 |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD4 |
| DC_42_n28 | 42 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n12A | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
| DC_66A_n2A, DC_66A_n2(2A) DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n46A | 66 | 1735 | 5 | 25 | 2135 | 12.0 | IMD3 |
|  | n46 | 5605 | 20 | 100 | 5605 | N/A | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A DC_66A-66A-66A_n77(2A) | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |
| DC_68A_n77A | 68 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_68A_n78A | 68 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_71A_n38A | 71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
|  | n38 | 2614 | 10 | 50 | 2614 | N/A | N/A |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| DC_71A_n77A8 DC_71A_n77(2A)8 | 71 | 671 | 5 | 25 | 625 | 5.5 | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | N/A |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
| DC_71A_n78(2A) | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |
| DC_106A_n41A DC_106A_n41C DC_106A_n41(2A) | 106 | 898.5 | 3 | 25 | 937.5 | [10] | IMD5 |
|  | n41 | 2656.5 | 10 | 50 | N/A | N/A | N/A |
| DC_106A_n48A DC_106A_n48C DC_106A_n48(2A) | 106 | 898.5 | 3 | 25 | 937.5 | [10] | IMD4 |
|  | n48 | 3633 | 10 | 50 | N/A | N/A | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This test configuration ensures the B3 self-interference would not interrupt the testing. NOTE 10: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 11: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |  |
| DC_1A_n77A DC_1A_n77(2A) DC_1A_n77(3A) | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 35.8 |  | IMD21 |  |
|  | n77 |  | 4090 |  | 10 |  | 50 |  | 4090 |  | N/A |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 17.8 |  | IMD41 |  |
|  | n77 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |  |
| DC_3A_n41A | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 18.4 |  | IMD4 |  |
|  | n41 |  | 2657.5 |  | 10 |  | 50 |  | 2657.5 |  | N/A |  | N/A |  |
| DC_3A_n78A | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 31.9 |  | IMD2 |  |
| DC_3A-3A_n78A | n78 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |  |
| DC_3A_n78A | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 18.5 |  | IMD4 |  |
| DC_3A-3A_n78A DC_3A_n78(2A) DC_3C_n78A DC_3C_n78(2A) | n78 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |  |
| DC_1A_n78A | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 17.8 |  | IMD4 |  |
|  | n78 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |  |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 32.10 |  | IMD2 |  |
|  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 19.10 |  | IMD41 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_2A_n78A DC_2A_n78(2A) | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 32.10 |  | IMD2 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 19.10 |  | IMD4 |  |
|  | n78 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_3A_n77A DC_3A_n77(2A) DC_3A_n77(3A) | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 31.9 |  | IMD21 |  |
|  | n77 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |  |
|  | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 18.5 |  | IMD41 |  |
|  | n77 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |  |
| DC_5A_n41A | 5 |  | 839 |  | 5 |  | 25 |  | 884 |  | 24.6 |  | IMD33 |  |
|  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |  |
| DC_5A_n77A3 DC_5A_n77C3 DC_5A_n77(2A)3 | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 18.60 |  | IMD41 |  |
|  | n77 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |  |
| DC_5A_n78A | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 17.5 |  | IMD4 |  |
|  | n78 |  | 3421 |  | 10 |  | 52 |  | 3421 |  | N/A |  | N/A |  |
| DC_8A_n78A DC_8B_n78A DC_8A_n78(2A) | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 15.5 |  | IMD4 |  |
|  | n78 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | N/A |  |
| DC_8A_n79A | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 21.5 |  | IMD5 |  |
|  | n79 |  | 4532.5 |  | 40 |  | 216 |  | 4532.5 |  | N/A |  | N/A |  |
| DC_13A_n77A DC_13A_n77C | 13 |  | 782 |  | 5 |  | 20 |  | 751 |  | 15.37 |  | IMD5 |  |
|  | n77 |  | 3879 |  | 10 |  | 50 |  | 3879 |  | N/A |  | N/A |  |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C DC_66A_n77(2A) DC_66A-66A_n77(2A) DC_66A-66A-66A_n77(2A) | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | 34.33 |  | IMD2 |  |
|  | n77 |  | 3950 |  | 10 |  | 50 |  | 3950 |  | N/A |  | N/A |  |
|  | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 11.27 |  | IMD5 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_8A_n41A | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 18.2 |  | IMD31 |  |
|  | n41 |  | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |
| DC_8A_n77A DC_8A_n77(2A) DC_8A_n77(3A) | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 15.5 |  | IMD4 |  |
|  | n77 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | N/A |  |
| DC_12A_n77A DC_12A_n77(2A) | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 11.7 |  | IMD5 |  |
|  | n77 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |
| DC_12A_n78A |  | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 11.7 |  | IMD5 |
|  |  | n78 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |
| DC_14A_n77A DC_14A_n77(2A) | 14 |  | 795.5 |  | 5 |  | 15 |  | 765.5 |  | 11.7 |  | IMD5 |  |
|  | n77 |  | 3947.5 |  | 10 |  | 50 |  | 3947.5 |  | N/A |  | N/A |  |
| DC_18A_n77A | 18 |  | 827.5 |  | 5 |  | 25 |  | 872.5 |  | 17.5 |  | IMD45 |  |
|  | n77 |  | 3355 |  | 10 |  | 50 |  | 3355 |  | N/A |  | N/A |  |
|  | 18 |  | 817.5 |  | 5 |  | 25 |  | 862.5 |  | 10.5 |  | IMD55 |  |
|  | n77 |  | 4130 |  | 10 |  | 50 |  | 4130 |  | N/A |  | N/A |  |
| DC_18A_n78A | 18 |  | 827.5 |  | 5 |  | 25 |  | 827.5 |  | 18.4 |  | IMD411 |  |
|  | n78 |  | 3355 |  | 10 |  | 50 |  | 3355 |  | N/A |  | N/A |  |
| DC_19A_n77A DC_19A_n77(2A) | 19 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 25.3 |  | IMD4 |  |
|  | n77 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
|  | 19 |  | 832.5 |  | 5 |  | 25 |  | 877.5 |  | 8.1 |  | IMD5 |  |
|  | n77 |  | 4195 |  | 10 |  | 50 |  | 4195 |  | N/A |  | N/A |  |
| DC_19A_n78A DC_19A_n78(2A) | 19 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 25.3 |  | IMD4 |  |
|  | n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_20A_n41 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 19.1 |  | IMD3 |  |
|  | n41 |  | 2512 |  | 10 |  | 50 |  | 2512 |  | N/A |  | N/A |  |
| DC_20A_n41 | 20 |  | 841 |  | 5 |  | 25 |  | 800 |  | 20.3 |  | IMD5 |  |
|  | n41 |  | 2564 |  | 10 |  | 50 |  | 2564 |  | N/A |  | N/A |  |
| DC_20A_n78A | 20 |  | 850 |  | 5 |  | 25 |  | 809 |  | 18.8 |  | IMD4 |  |
|  | n78 |  | 3359 |  | 10 |  | 50 |  | 3359 |  | N/A |  | N/A |  |
| DC_25A_n78A | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 32.1 |  | IMD2 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 19.1 |  | IMD4 |  |
|  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |  |
|  | 25 |  | 1875 |  | 5 |  | 25 |  | 1955 |  | 20 |  | IMD5 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
| DC_26A_n41A | 26 |  | 839 |  | 5 |  | 25 |  | 884 |  | 24.6 |  | IMD33 |  |
|  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |  |
| DC_26A_n78A | 26 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 23.8 |  | IMD4 |  |
|  | n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_28A_n77A | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 19.2 |  | IMD5 |  |
|  | n77 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |  |
| DC_30A_n77A DC_30A_n77(2A) | 30 |  | 2310 |  | 5 |  | 25 |  | 2355 |  | 17.6 |  | IMD4 |  |
|  | n77 |  | 3487.5 |  | 10 |  | 50 |  | 3487.5 |  | N/A |  | N/A |  |
| DC_28A_n78A | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 11.7 |  | IMD5 |  |
|  | n78 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |  |
| DC_21A_n79A | 21 |  | 1457.5 |  | 5 |  | 25 |  | 1505.5 |  | 33.4 |  | IMD3 |  |
|  | n79 |  | 4420.5 |  | 10 |  | 50 |  | 4420.5 |  | N/A |  | N/A |  |
| DC_66A_n78A | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 11.27 |  | IMD5 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_71A_n41A | 71 |  | 666 |  | 5 |  | 25 |  | 620 |  | 16.3 |  | IMD4 |  |
|  | n41 |  | 2618 |  | 10 |  | 50 |  | 2618 |  | N/A |  | N/A |  |
| DC_71A_n77A3 | 71 |  | 681.5 |  | 5 |  | 25 |  | 635.5 |  | 11.4 |  | IMD5 |  |
|  | n77 |  | 3361.5 |  | 10 |  | 50 |  | 3361.5 |  | N/A |  | N/A |  |
| DC_71A_n78A | 71 |  | 681.5 |  | 5 |  | 25 |  | 635.5 |  | 11.4 |  | IMD5 |  |
| DC_71A_n78(2A) | n78 |  | 3361.5 |  | 10 |  | 50 |  | 3361.5 |  | N/A |  | N/A |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Void NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>