3GPP TSG-RAN WG4 Meeting #116bis			R4-2513793
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	on 6MHz UE RF requirement

Source:	OPPO

Agenda item:	7.1.5.2

Document for:	Approval

Introduction

In Rel-20, the UE RF topic on 6MHz has been introduced in RANP[1]. This paper shares our view on this aspect.

Discussion

Spectrum Utilization

During the 7MHz study, the spectrum utilization has been studied. The 90% SU has been agreed to give more guard band for UE RF requirements. Hence for 6MHz, it is assumed still 90% SU can be reused. In such case, the SU for 6MHz has been shown as below.

Table 1 SU for 6MHz

| SCS | RB | CBW | SU |
| --- | --- | --- | --- |
| 15 | 30 | 6 | 0.9 |
| 15 | 31 | 6 | 0.93 |

For 7MHz, 35RB with 90% SU has been agreed.

For 6MHz, apply 90% SU and 30 RB.

UE REFSENS

The scaling factor method has been used for 7MHz and to define the REFSENS requirement for different bands. For 6MHz, in the WID, it has required to define at least the RF requirements for band n5, n8, n12, n85, n29. Below has shown the REFSENS requirement for these bands.

Table 2 NR REFSENS requirement for band n5 n8 n12 and n85

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n5 | 15 | -100.2 | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n8 | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
|  | 30 |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |  |  |  |
| n12 | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  | -66.3 |  |  |
| n85 | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |

The scaling factor of number of RBs has been listed below table 3.

‘Table 3 Scaling factor of n5, n8, n12 and n85

|  | Scaling of 10MHz | REFSENS scaling | Scaling of 6MHz |
| --- | --- | --- | --- |
| n5 | 3.2 | 3.2 | 2.4 |
| n8 | 3.2 | 3.2 | 2.4 |
| n12 | 3.2 | 3.2 | 2.4 |
| n85 | 3.2 | 3.2 | 2.4 |

It can be seen that for these bands, the REFSENS scaling factor are all 3.2dB and they are the same for the RB number scaling factor from 10MHz as 52RB to 5MHz as 25RB.

The REFSENS scaling factor of band n5, n8, n12 and n85 are all 3.2 which is the same as RB number of scaling.

In such case, the same RB number of scaling is proposed for 6MHz. In previous discussion, it has been proposed 30RB for 6MHz and hence the scaling factor of 30RB to 25RB is proposed and also shown above in table 3.

The REFSENS scaling factor of band n5, n8, n12 and n85 for 6MHz are all 2.4dB.

Below table 4 has shown the REFSENS requirement for band n29. Since the band n29 is an SDL band and no UL-DL interference is considered. The 7MHz can be directly added into the table.

Table 4 NR REFSENS requirement for band n29

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n29 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |

For 6MHz for band n29, directly add 6MHz in the REFSENS table for band n29.

Conclusion

This paper discussed xxxx

Spectrum Utilization:

Observation 1: 	For 7MHz, 35RB with 90% SU has been agreed.

Proposal 1: 	For 6MHz, apply 90% SU and 30 RB.

UE REFSENS

Observation 2: 	The REFSENS scaling factor of band n5, n8, n12 and n85 are all 3.2 which is the same as RB number of scaling.

Proposal 2: 	The REFSENS scaling factor of band n5, n8, n12 and n85 for 6MHz are all 2.4dB.

Proposal 3: 	For 6MHz for band n29, directly add 6MHz in the REFSENS table for band n29.

Reference

RP-252942 New WID: UE RF enhancements for NR FR1, Phase 5, Moderator (vivo)