3GPP TSG-RAN WG4 Meeting #116bis                                                           R4- 2513794

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on RRM impacts on Rel-20 MIMO Phase 6

Agenda item:			7.10.3

Document for:	Discussion

- Introduction

In RAN#108, RAN plenary approved a new MIMO work item of Rel-20 MIMO Phase 6 in [1], which has been further revised in [2] in RAN#109.

The objectives of Rel-20 MIMO Phase 6 are described in [1] including:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we analyze the overall impacts to RRM requirements related to the above objectives.

- Discussion

RAN1 started discussion in last RAN1#122 meeting. Although RAN1 discussion of Rel-20 MIMO Phase 6 is still ongoing, we can start high level analysis of the potential impact to RRM requirements as well as other WGs progresses are ongoing.

2.1 SRS further enhancement

For objective#1 in WID, the SRS further enhancement are primarily focus on two dimensions: 1) frequency domain enhancement; 2) time domain enhancement.

- Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

For frequency domain enhancement, it will further enhance multiple frequency domain starting positions during repetition. It changes the SRS resource mapping in frequency domain including multiple frequency-domain starting positions for SRS repetition symbols for hopping. Typically, for key RRM core requirements including SRS carrier based switching and SRS antenna port switching, no impacts by this SRS frequency mapping. Even in several RRM test cases by using SRS, it is just use SRS to verify such transmission which means simple SRS configuration is used. Thus, we think there is no RRM impacts by introducing the frequency domain enhancement for SRS.

Proposal 1: No RRM impacts by SRS enhancement in frequency domain “Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding”.

For time domain enhancement, the purpose it to introduce the “cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”. There can be different time domain resource mapping in this enhancement including: 1) different SRS resources are in two adjacent slots. 2) one SRS resource is in two adjacent slots.

From RRM perspective, the only potential RRM impact is for the requirements of Interruptions at NR SRS antenna port switching. In legacy RAN4 requirements for it, the common understanding is the components of the interruption time of SRS antenna port switching are: antenna switching time before and after SRS transmission; SRS transmission time of X symbols. And in the requirements, it is separated to 1 SRS symbol is configured or others (X=6 SRS symbols).

In existing RRM requirements, the SRS antenna port switching requirements are defined. However, the requirements are applicable by conditions.

The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.

After Rel-16, the SRS are further enhanced in time domain which more symbols can be used without the limitation of “last 6 symbols”.

However, in RRM requirements, there is no further enhancement for “SRS antenna port switching” to other than “FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.”

In previous MIMO WI, NR_ENDC_RF_Ph4, and RAN-P RRM enh proposal, there were discussion on whether to extend existing SRS antenna port switching RRM requirements in MIMO WI or other common RRM enhancement WI.

Observation 1: In SRS antenna port switching requirements, the applicability of the requirement is “on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot”.

Proposal 2: FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot.

2.2 DL CSI acquisition enhancement

For objective#2 in WID, the DL CSI acquisition enhancement are primarily focus on two dimensions: 1) Early SRS/CSI/CSI-RS triggering; 2) CSI-RS freq. density reduction.

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

For 1) Early SRS/CSI/CSI-RS triggering, from the purpose, RAN1 is intended to introduce an early SRS/CSI/CSI_RS trigger mechanism for the UE transitioning from IDLE/INACTIVE to CONNECTED mode. It may work together with SCell activation/deactivation or SCell dormancy procedures.

In existing RRM requirements, RAN4 specified many RRM requirements related to such procedures, such as: the SCell activation delay or SCell deactivation delay or Interruption due to SCell dormancy. In RAN4 discussion and final requirements, RAN4 should specify the timeline of the procedures and conditions. In previous releases, RAN4 specified the Scell activation/deactivation delay in many WIs. For example, in Rel-16, the measurement in based on SSB. In Rel-17, Scell activation procedures can be enhanced by A-TRS (temporary RS) for intra-band CA. After that, Scell activation were also enhance in NES and for EMR and further enhanced in RRM phase 5.

In RAN1#122 meeting, RAN1 initially discussed for early triggering for both SRS-AS and CSI/CSI-RS in SCell activation procedure, the early agreements are shown as below:

Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of SRS-AS/CSI-RS/CSI when UE transition from IDLE to CONNECTED mode, study the following at least three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability.

It can be observed that the early triggering of SRS-AS/CSI-RS/CSI may work together or with in the SCell activation procedure which mean there are RRM impacts by early SRS/CSI/CSI-RS triggering especially for SCell activation/deactivation delay/SCell dormancy.

Proposal 3: For early triggering of SRS-AS/CSI-RS/CSI, there are RRM impacts especially for SCell activation/deactivation delay/SCell dormancy. FFS on detailed RRM requirements after further RAN1 progress.

For 2) CSI-RS freq. density reduction, it may have demod impacts which we would not discuss and conclude here. It belongs to the demod discussion. But it has no RRM impacts obviously.

Proposal 4: No RRM impacts by CSI-RS frequency domain density reduction.

- Conclusion

In this contribution, we provide our proposals for overall RRM impacts of Rel-20 MIMO Phase 6:

Proposal 1: No RRM impacts by SRS enhancement in frequency domain “Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding”.

Observation 1: In SRS antenna port switching requirements, the applicability of the requirement is “on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot”.

Proposal 2: FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot.

Proposal 3: For early triggering of SRS-AS/CSI-RS/CSI, there are RRM impacts especially for SCell activation/deactivation delay/SCell dormancy. FFS on detailed RRM requirements after further RAN1 progress.

Proposal 4: No RRM impacts by CSI-RS frequency domain density reduction.

- Reference

[1] RP-251856, New WID: NR MIMO Phase 6, Samsung (Moderator)

[2] RP-252936, Revised WI: NR MIMO Phase 6, MediaTek