3GPP TSG-RAN WG4 Meeting #116bis		     R4-2513806

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	6.11.4

Source: 	OPPO

Title: 	RRM core requirement and testing framework for beam management

Document for:	Approval

Discussion

OTA tests for FR2 BM

Regarding the testing framework on BM, from our understanding, following aspects need to be considered.

- The number of FR2 beams that RAN4 tests can support remain unclear. It is necessary to clarify the limitations regarding the FR2 beams or probes that TE vendors could support. Based on this limitation(if any), RAN4 can further consider how to reach a consensus on designing/conducting tests for BM use cases and how to set/form beam set A and B of different sizes for different BM cases.

- A potential test setup could be (as shown in Figure 1)

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

As agreed in RAN4 #114 meeting, we can start with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Figure 1 A potential test setup for BM tests

- Besides, it's also crucial to further consider that, as a data/scenario-driven solution, the chamber and beam patterns provided by different TEs may impact the test results of BM (e.g., due to variations in beam settings, different considerations on multipath, and other implementations). Besides, even for a same TE, different DUT hardware implementations may also lead to variations in receiving behavior and beam measurement results. Attention must be paid to ensure that the BM model constructed on the DUT side(e.g. by corresponding training dataset) can match(or approximate match) and be utilized in the chamber environment on the TE side. Otherwise, it becomes challenging for a DUT to pass the test under unknown chamber conditions/scenarios. This may require a close collaboration between DUT and TE to ensure that the model under test is compatible with the testing environment.

- In RAN4 #116 meeting, RAN4 agreed to use CDL-C Umi as starting point. Following issues should be taken into account when identifying the BM test settings,

  - to model correlation between the measurements of different beams to ensure that the beams remain predictable,

  - the predictability of the beams must not be too strong, as an extreme case where there is no distinction between different beams would undermine the necessity for BM testing,

  - the test should not be overly complex and should optimally utilize the existing test environment.

For BM testability, RAN4 needs to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Further consider how to ensure that the BM model constructed on the DUT side can match and be utilized in the testing environment on the TE side.

New approach for FR2 BM test

In R18 and R19, a lot of effort is spent on generating signals in a chamber environment that can serve as input for AI use case models. However, key questions still lack clear answers, e.g., how to effectively simulate beam-sweeping scenarios in a chamber with limited probes, how to model channels, and whether testing model performance under simplified CDL UMi conditions is truly meaningful. From our understanding, in current BM FR2 test discussions, the over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

Another approach FR2 BM test could be RAN4 may explore whether/how to decouple the signal acquisition testing(e.g., or model inputs) from model performance evaluation(e.g., for model performance) for BM cases.

- Signal acquisition could continue to be tested via OTA, e.g., evaluating BM Set B measurement accuracy.

- Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets).

For instance, RAN4 study and specify test dataset for models under test. TE would provide DUT with a predefined BM test dataset. The DUT would then return inference results to the TE for performance evaluation. This approach would reduce the need for complex OTA tests, accelerate model validation, and bring testing closer to real-world conditions depending on the quality of the test dataset.

The over-simplified OTA assumptions being used for BM may offer little practical guidance for field deployment.

For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Signal acquisition could continue to be tested via OTA, e.g., evaluating BM Set B measurement accuracy.

Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets).

Conclusion

In this contribution, we have discussed the testing framework for beam management, and got the following proposals:

Proposal 1: 	For BM testability, RAN4 needs to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 2: 	Further consider how to ensure that the BM model constructed on the DUT side can match and be utilized in the testing environment on the TE side.

Observation 1: 	The over-simplified OTA assumptions being used for BM may offer little practical guidance for field deployment.

Proposal 3: 	For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Proposal 4: 	Signal acquisition could continue to be tested via OTA, e.g., evaluating BM Set B measurement accuracy.

Proposal 5: 	Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets).

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2