3GPP TSG-RAN4 Meeting #116bis	R4-2513834

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			6 MHz channel bandwidth – system and UE RF impacts

Agenda item:		7.1.5.2

Document for:		Approval

# Introduction

In last RAN#109, a new WI ([1]) has been approved to enhance UE RF requirements for the deployment in NR FR1. One of the objectives of this WI is to specify 6 MHz channel bandwidth for NR bands n5, n8, n12, n85 and n29.

In this contribution, we are analyzing the system parameters (BS and UE) and UE RF impacts when introducing 6 MHz channel bandwidth.

# Discussion

## Background

The new WI ([1]) approved in last RAN#109 meeting, embraces several enhancements which are UE related, except the following one which would impact both UE and BS RF, adding 6 MHz channel BW support in RAN4 specifications.

## Transmission bandwidth configuration

### System utilization

One of the main issues with the introduction of 6 MHz channel bandwidth is the specification of the maximum transmission bandwidth configuration, also called the spectrum utilization, for this 7 MHz channel bandwidth.

Considering 15 kHz sub-carrier spacing, from preliminary calculation, the maximum transmission bandwidth configuration for 6 MHz should be a value between 30, 31, 32 and 33 RBs. The Table 1 gives the corresponding spectrum utilization and minimum guard band for each of those 3 options, comparing with 3, 5, 7 and 10 MHz channel bandwidth.

Table 1: Spectrum utilization for 3, 5, 7 and 10 MHz channel bandwidth

|  | Channel BW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6 |  |  |  | 7 | 10 |
| Number of RBs | 15 | 25 | 30 | 31 | 32 | 33 | 35 | 52 |
| Spectrum utilization | 90.0% | 90.0% | 90.0% | 93.0% | 96.0% | 99.0% | 90.0% | 93.6% |
| Minimum guard band (kHz) | 142.5 | 242.5 | 292.5 | 202.5 | 120.0 | 22.5 | 342.5 | 312.5 |

It should be noted that the decision for the 7 MHz spectrum utilization (35 RBs) was motivated by the fact that UE supporting 5 MHz (and not 7 MHz) and not supporting the 10kHz enhanced channel raster shall also be supported within the 7 MHz.

When deciding on the spectrum utilization for a channel bandwidth, RAN4 usually considers a linear progression of the spectrum utilization over the different channel bandwidths. Following this approach, not considering 7 MHz for the reason mentioned above the 6 MHz spectrum utilization should then be around 90-93% as shown in Table 1.

The minimum guard band for 6 MHz channel bandwidth is another aspect to consider. It should not be much smaller than the 5 MHz minimum guard band. Again, using a linear progression between 5 and 10 MHz, the minimum guard band for 6 MHz should then be around 250 kHz. As shown in Table 1, only a maximum transmission bandwidth configuration with 30 RBs would guarantee this, while 31RBs would give a lower value.

Also, another criterion which could influence the value of the spectrum utilization is the support (or not) of NB-IoT in-band in the 7 MHz channel bandwidth. Indeed, if NB-IoT in-band must be supported, the BS shall then also support the mandatory 6dB power boosting feature for the NB-IoT RB. Considering this NB-IoT RB could be located at channel bandwidth edge, depending then on the spectrum utilization, this could be a challenge for filters design. Nevertheless, from the first investigations we have done, we haven’t seen any issue for a spectrum utilization of 30 or even 31 RBs.

Observation 1: Based on the spectrum utilization and the minimum guard band analysis, the maximum transmission bandwidth configuration for 6 MHz could be 30 RBs or 31 RBs. This should allow supporting NB-IoT operating in-band as well.

### Legacy UE

As discussed for the introduction of 7 MHz channel BW ([2]), another aspect that should be considered when specifying the maximum transmission bandwidth configuration for 6 MHz channel bandwidth is if legacy UEs supporting 5 MHz channel bandwidth but not supporting the 10 kHz enhanced channel raster should be supported within the 6 MHz.

For 5 MHz channel bandwidth, the maximum transmission bandwidth configuration is equal to 25 RBs, which is an odd number. So, to support a legacy UE supporting 5 MHz channel BW within the 6 MHz and keep the RBs alignment:

- The legacy UE shall support the 10kHz enhanced channel raster. Its channel raster could then be shifted by 90 kHz (or 270 kHz) from the 100kHz channel raster, keeping PRBs aligned.

- If not, the maximum transmission bandwidth configuration for 6 MHz shall also be specified with an odd number of PRBs.

Nevertheless, in the case of 6 MHz, as 5 and 6 MHz channel BWs are very close, the 5 MHz signal would have to be centred in the 6 MHz grid, meaning the PDCCH CCEs will be misaligned, which leads to PDCCH blocking. To solve this problem, UE supporting only 5 MHz channel BW would still have to support the enhanced channel raster.

Observation 2: UEs supporting 5 MHz channel BW only will still have to support the 10kHz enhanced channel raster when scheduled in the 6 MHz channel BW, even if the maximum transmission BW for 6 MHz is an odd number of RBs (e.g. 31).

### Proposal

Based on the previous observations, to ensure a flexible usage of the 6 MHz spectrum and optimized spectrum utilization, we would make the following proposal:

Proposal 1: Specify the maximum transmission bandwidth configuration with 31 RBs.

## Minimum channel BW in a NR band

Similar to 7 MHz channel BW, we would make the following proposal.

Proposal 2: 6 MHz can not be the minimum channel BW in a NR band.

## System parameters impact overview

Focusing on the system parameters, the following Table 2 gives an overview of the system impacts in the NR specifications (TS 38.101-1 and TS 38.104) when introducing 7 MHz channel bandwidth.

Table 2: System impacts overview

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 5.1 General | No |  |
| 5.2 Operating bands | No |  |
| 5.3 Channel bandwidth |  |  |
| 5.3.1 General | No |  |
| 5.3.2 Transmission bandwidth configuration | Yes | See section 2.1 |
| 5.3.3 Minimum guardband and transmission bandwidth configuration | Yes | See section 2.1 |
| 5.3.4 RB Alignment | No |  |
| 5.3.5 BS channel bandwidth per operating band | Yes | A 6 MHz column shall be added to the table 6 MHz shall be added as supported channel BW for bands n5, n8, n12, n85 and n29 |
| 5.3A Channel bandwidth for CA (UE only) 5.3E Channel bandwidth for V2X (UE only) 5.3I Channel bandwidth for RedCap (UE only) | No |  |
| 5.4 Channel arrangement | No |  |
| 5.4A Channel arrangement for CA (UE only) 5.4E Channel arrangement for V2X (UE only) | No |  |

Refer to our other contributions for an overview of the BS specification impacts ([2]).

Proposal 3: Agree with the system impacts listed in Table 2.

## UE specific issues

### Spectrum emission mask

The spectrum emission mask shall be specified for 6 MHz channel bandwidth. We would propose to specify it according to below table.

Proposal 4: Specify 6 MHz channel BW SEM according to the following table:

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 6, 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -13 |  |  |  |
| ± 6-10 | -25 |  |  |  |
| ± 5-BWChannel |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |  |

## UE specification impact overview

Focusing on the UE RF parameters, the following Table 2 gives an overview of the impacts in the NR specifications (TS 38.101-1) when introducing 6 MHz channel bandwidth.

Table 3: UE specifications impacts overview

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 6. Transmitter characteristics |  |  |
| 6.1 General | No |  |
| 6.2 Transmitter power |  |  |
| 6.2.1 UE maximum output power | No |  |
| 6.2.2 UE maximum output power reduction | TBC | To be checked based on the spectrum utilization decision |
| 6.2.3 UE additional maximum output power reduction | Yes | The need for A-MPR should be checked 6 MHz channel BW and accordingly added to NS_43 and NS_06 in Table 6.2.3.1-1 (to be further discussed if NS_43U should also be considered) Corresponding A-MPR shall be evaluated for 6MHz for NS_43 (6.2.3.6). Note that for NS_06, no A-MPR is needed for PC3 and A-MOR is channel BW agnostic for PC2. |
| 6.2.4 Configured transmitted power | No |  |
| 6.3 Output power dynamics |  |  |
| 6.3.1 Minimum output power | Yes | 6 MHz shall be added in Table 6.3.1-1 |
| 6.3.2 Transmit OFF power | Yes | 6 MHz shall be added in Table 6.3.2-1 |
| 6.3.3 Transmit ON/OFF time mask | No |  |
| 6.3.4 Power control | No |  |
| 6.4 Transmit signal quality | No |  |
| 6.5 Output RF spectrum emissions |  |  |
| 6.5.1 Occupied bandwidth | Yes | 6 MHz shall be added in Table 6.5.1-1 |
| 6.5.2 Out of band emission | Yes | 6 MHz shall be added in Table 6.5.2.2-1 6 MHz shall be added in Table 6.5.2.4.1-1 6 MHz should be added in Table 6.5.2.3.4-1 NS_06 |
| 6.5.3 Spurious emissions | Yes | 6 MHz should be added in Table 6.5.3.3.5-1 NS_43 |
| 6.5.4 Transmit intermodulation | No |  |
| 7. Receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Diversity characteristics | No |  |
| 7.3 Reference sensitivity | Yes | A column for 6 MHz CBW shall be added in Table 7.3.2-1a and corresponding REFSENS for n5, n8, n12, n85 and n29 shall be evaluated A column for 6 MHz CBW shall be added in Table 7.3.2-3 and, based on the agreed spectrum utilization, the uplink configuration shall be updated for n5, n8, n12, n85 and n29. |
| 7.4 Maximum input level | Yes | 6 MHz shall be added to Table 7.4-1 |
| 7.5 Adjacent channel selectivity | Yes | 6 MHz shall be added to Table 7.5-1, Table 7.5-3, Table 7.5-4 |
| 7.6 Blocking characteristics |  |  |
| 7.6.1 General | No |  |
| 7.6.2 In-band blocking | Yes | 6 MHz shall be added to Table 7.6.2-1 |
| 7.6.3 Out-of-band blocking | Yes | 6 MHz shall be added to Table 7.6.3-1 |
| 7.6.4 Narrow band blocking | Yes | 6 MHz shall be added to Table 7.6.4-1 |
| 7.7 Spurious response | Yes | 6 MHz shall be added to Table 7.7-1 |
| 7.8 Intermodulation characteristics | Yes | 6 MHz shall be added to Table 7.8.2-1 |

Proposal 5: Agree with the system impacts listed in Table 3.

Observation 3: A-MPR evaluation would only be needed for NS_43 (and eventually NS_43U).

# Conclusion

In this contribution, we analyzed the system parameters impacts and gave an overview of the UE RF impacts when introducing 6 MHz channel bandwidth. We made the following observations and proposals:

Observation 1: Based on the spectrum utilization and the minimum guard band analysis, the maximum transmission bandwidth configuration for 6 MHz could be 30 RBs or 31 RBs. This should allow supporting NB-IoT operating in-band as well.

Observation 2: UEs supporting 5 MHz channel BW only will still have to support the 10kHz enhanced channel raster when scheduled in the 6 MHz channel BW, even if the maximum transmission BW for 6 MHz is an odd number of RBs (e.g. 31).

Proposal 1: Specify the maximum transmission bandwidth configuration with 31 RBs.

Proposal 2: 6 MHz can not be the minimum channel BW in a NR band.

Proposal 3: Agree with the system impacts listed in Table 2.

Proposal 4: Specify 6 MHz channel BW SEM according to the following table:

Proposal 5: Agree with the system impacts listed in Table 3.

Observation 3: A-MPR evaluation would only be needed for NS_43 (and eventually NS_43U).

# References

- RP-252901, UE RF enhancements for NR FR1, Phase 5

- R4-2501361, 7 MHz channel bandwidth – system impacts, Ericsson

- R4-2513833, 7 MHz channel bandwidth – BS impacts, Ericsson