3GPP TSG-RAN5 Meeting #116bis	R4-2513840

# Prague, Czech Republic, 13th Oct 2025 – 17th Oct 2025

Agenda item:	7.6.3

Source: 	Viasat, Space 42, Aalyria

Title: 	Assumptions, Proposals and Work Plan for IoT-NTN PC1.5 single Tx related work

Document for:	Approval

## 1.	Introduction

During the RAN4#116 meeting, as part of NTN WI [1], IoT-NTN single Tx PC1 UE for both handheld and non-handheld UEs was introduced in [2]. During the RAN#109 meeting, as part of the Rel-20 package for NTN, development of specifications for IoT-NTN PC1.5 single Tx UE was approved [3].

This document presents aspects of [1] that can be leveraged including assumptions related to MPR and A-MPR evaluations. Based on the proposed re-use of the studies/work undertaken as part of [1], a work plan is proposed that would allow for efficient use of the NTN TUs across the topics of PC1.5 single Tx IoT NTN and DL CA topics that are part of the Rel-20 NTN package [3].

## 2.	Overview

For the IoT NTN single Tx PC1.5 UE, the following scope [3] was approved as part of the Rel-20 NTN package at the RAN#109 meeting:

Figure 1: WID Objectives for IoT NTN PC1.5 single Tx UE

## 3.	UE Types to Consider for PC1.5 Single Tx IoT NTN

As part of Release-19, RAN4 spent several meetings evaluating feasibility of various UE architecture options for NR NTN and IoT NTN HPUE for inclusion in the NTN UE specifications – TS 38.101-5 and TS 36.102. Based on contributions submitted by various companies, RAN4 verified the feasibility of PC1 single Tx IoT NTN UE for both handheld and non-handhelds. Due to the feasibility of PC1, RAN4 agreed to specify PC2 and PC1 single Tx UE for IoT NTN for both handhelds and non-handhelds. Furthermore, due to lack of operator interest, the specification work for Cat-M1 IoT NTN UE power classes other than PC2 was not undertaken in Rel-19. As a result, per the scope of the IoT NTN PC1.5 with 1Tx, RAN4 is to focus only on NB-IoT IoT NTN, and Cat-M1 PC1.5 1 Tx Iot NTN UE is out of scope.

Observation 1: PC1 Single Tx NB-IoT NTN has been specified for both handheld and non-handheld.

Observation 2: Due to lack of operator interest, RAN4 decided to not work on higher power classes other than PC2 for Cat-M1 IoT NTN UEs.

Observation 3: Top level bullet in Rel-20 WID objectives mention that the scope of Rel-20 PC1.5 IoT NTN work will be associated with the NB-IoT based IoT NTN.

Proposal 1: As part of Rel-20, RAN4 shall only specify PC1.5 single Tx specifications for NB-IoT NTN handhelds and non-handhelds.

## 4.	Co-existence Studies

As part of deliberation during the RAN#109 meeting, it was agreed to leverage much of the evaluations performed as part of the Rel-19 IoT NTN HPUE. This included re-using the coexistence studies performed for NB-IoT PC2 and PC1 submitted during Rel-19 deliberations [4].

Based on the coexistence studies, RAN4 has specified the same value of 37 dBc for the ACLR for both PC2 and PC1 NB-IoT NTN HPUEs in Rel-19. Given the conclusion from the coexistence studies resulted in the same ACLR value for PC2 and PC1 NB-IoT NTN UE, during the deliberations at RAN#109 it was tentatively agreed that additional coexistence studies are not necessary for specifying PC1.5 for NB-IoT NTN and were not included in the objectives.

Proposal 2: Additional coexistence studies are not required for specifying PC1.5 single Tx NB-IoT NTN UE since coexistence studies performed for PC1 and PC2 NB-IoT NTN UE resulted in ACLR specification of 37 dBc.

Proposal 3: Specify ACLR value of 37 dBc for PC1.5 single Tx NB-IoT NTN UE.

## 5.	SEM for PC1.5 single Tx NB-IoT NTN UE

Given SEM for PC2 and PC1 NB-IoT NTN UEs are the same, it is also proposed that the existing SEM also apply to PC1.5 single Tx NB-IoT NTN UE.

Proposal 4: Apply existing SEM for PC1.5 single Tx NB-IoT NTN UE.

## 6.	MPR/A-MPR Evaluation Assumptions

Furthermore, significant discussions were held as part of the Rel-19 HPUE NTN work prior to reaching agreements on the assumptions and parameters for MPR/A-MPR evaluation for PC2 and PC1 NB-IoT NTN UE [5, 6]. The agreements are summarized below in Figures 2 and 3:

Figure 2 – Agreement from R4-2505114

Figure 3 – Agreement from R4-2508007

It is proposed that the PC1 NB-IoT NTN UE agreements on assumptions for MPR and A-MPR evaluation be used for PC1.5 MPR/A-MPR evaluation as well.

Proposal 5: Re-use the assumptions agreed to and used for PC1 NB-IoT NTN UE MPR/A-MPR evaluation in [5, 6] for PC1.5 NB-IoT NTN UE MPR/A-MPR evaluation.

## 7.	RSD Evaluation

Since the scope is limited to NB-IoT NTN PC1.5 UE, given its operation mode being half-duplex, like PC1 and PC2 NB-IoT NTN UEs, there is no need to specify RSD requirements.

Proposal 6: Do not specify RSD for PC1.5 NB-IoT NTN UE.

## 8.	Feasibility Study and Re-use of PC2/PC1 SEM and ACLR

Based on contributions submitted by various companies during Rel-19 NTN HPUE discussions, RAN4 verified the feasibility of PC1 single Tx IoT NTN UE [7]. Due to the feasibility of PC1, RAN4 agreed to specify PC2 and PC1 single Tx UE for IoT NTN. Given that PC1 single Tx NB-IoT NTN UE is feasible and already specified, further feasibility assessment is not required for PC1.5 single Tx NB-IoT NTN UE.

Proposal 7: Given the feasibility of PC1 single Tx NB-IoT NTN, PC 1.5 single Tx NB-IoT NTN UE is feasible and additional feasibility assessment is unnecessary and not required.

Based on the above proposals, one can conclude to limit the scope to evaluating the MOP tolerance, MPR and A-MPR for PC1.5 NB-IoT NTN UEs for bands 255and 256.

Proposal 8: Re-use SEM, ACLR for PC1 single Tx NB-IoT NTN and focus the work on evaluating and specifying MOP tolerance, MPR and A-MPR for PC1.5 single Tx NB-IoT NTN UE for bands 255 and 256.

## 9.	Work Plan

Based on above observations and proposals, it is proposed to adopt the following work plan for IoT NTN:

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4#116bis | - Agree on the assumptions for MPR and A-MPR simulation/measurements for example bands for PC1.5 NB-IoT NTN UE - Agree to additional emission requirements for bands 256 and 255 A-MPR evaluation |
| RAN4#117 | - Agree to MOP tolerances - Reach tentative agreement on MPR and A-MPR for bands 256 and 255 - Endorse draft CR(s) based on tentative agreements |
| RAN4#118 | - Finalize CR(s) |

Table 1: Proposed Work Plan

Proposal 9: Approve the proposed work plan in Table 1 above.

## 10.	Conclusion

Observation 1: PC1 Single Tx NB-IoT NTN has been specified for both handheld and non-handheld.

Observation 2: Due to lack of operator interest, RAN4 decided to not work on higher power classes other than PC2 for Cat-M1 IoT NTN UEs.

Observation 3: Top level bullet in Rel-20 WID objectives mention that the scope of Rel-20 PC1.5 IoT NTN work will be associated with the NB-IoT based IoT NTN.

Proposal 1: As part of Rel-20, RAN4 shall only specify PC1.5 single Tx specifications for NB-IoT NTN handhelds and non-handhelds.

Proposal 2: Additional coexistence studies are not required for specifying PC1.5 single Tx NB-IoT NTN UE since coexistence studies performed for PC1 and PC2 NB-IoT NTN UE resulted in ACLR specification of 37 dBc.

Proposal 3: Specify ACLR value of 37 dBc for PC1.5 single Tx NB-IoT NTN UE.

Proposal 4: Apply existing SEM for PC1.5 single Tx NB-IoT NTN UE.

Proposal 5: Re-use the assumptions agreed for PC1 NB-IoT NTN UE MPR/A-MPR evaluation in [5, 6] for PC1.5 NB-IoT NTN UE MPR/A-MPR evaluation.

Proposal 6: Do not specify RSD for PC1.5 NB-IoT NTN UE.

Proposal 7: Given the feasibility of PC1 single Tx NB-IoT NTN, PC 1.5 single Tx NB-IoT NTN UE is feasible and additional feasibility assessment is unnecessary and not required.

Proposal 8: Re-use SEM, ACLR for PC1 single Tx NB-IoT NTN and focus the work on evaluating and specifying MOP tolerance, MPR and A-MPR for PC1.5 single Tx NB-IoT NTN UE for bands 255 and 256.

Proposal 9: Approve the proposed work plan in Table 1 above.

## 11.		References

[1]	RP-242901, "Revised WID on Enhance requirements and conductive test methodology for NR NTN and IoT NTN ", Apple, Thales, Inmarsat, Viasat, Globalstar

[2]	TS 36.102, ver 19.1.0

[3]	RP-252962, Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[4]	R4-2510859, Annex X, BigCR for TR 38.863

[5]	R4-2505114, RAN4#114bis, WF on HPUE RF requirements for IoT NTN, vivo

[6]	R4-2508007, RAN4#115, WF on IoT-NTN HPUE, vivo

[7]	RP-2419533, Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)