# 3GPP TSG-RAN WG4 Meeting #116bis		R4-2513865

Prague, Czech Republic, 13th to 18th October 2025

Agenda Item:	7.7.3

Source:	Ericsson

Title:	On Uplink co-existence considerations for UAV

Document for:	Discussion

# 1	Introduction

During RAN#109, a RAN4 WI [1] was agreed on Enhanced NR requirements for UAV operations to address issues related to UL throughput and UL coverage with higher transmit power. The UAVs will be served by ground based UAV BSs by adjusting the tilt angle upwards. It has been highlighted in the WID to take reference from NR ATG study [2], which supported two ATG UE antenna types – omni-directional antenna and antenna array. The objectives for core part from the WID [1], has been highlighted below –

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

This contribution highlights the network modelling assumptions and uplink co-existence scenarios for the UAV co-existence work.

# 2	Discussion

## 2.1	Network modelling assumptions

The WID objective states that the ISDs for UAV ground BSs in UAV network deployment is around 2-4 kms. In the co-existence between UAV network deployment and a terrestrial network deployment, this would mean the UAV network BS density would be comparatively lower and hence would cover a greater number of TN cells per UAV ground BS.

##### Deployment scenario

The application scenarios of UAV from WID include low altitude logistics, travel, tourism, control, supervision and medical rescue. Since the network of UAV BS is dedicated for UAV operation and with UAV ISD of 2-4 km, it is reasonable to consider UAV BS in Rural Macro scenarios and TN BS in Urban Macro scenarios with ISD of 450m.

RAN4 to clarify whether UAV BS are deployed in Macro Urban or Macro Rural locations.

##### ISD

In TN networks, the ISD is the distance between two BS sites. For the TN UEs, the cell radius is less than the ISD as coverage areas overlap and TN BS has a down-tilt. However, for UAV networks, the geometry changes as the BS antenna beam will have a higher uptilt that should consider both vertical and horizontal coverage. This means the UAVs can only be served at certain horizontal ranges and the coverage doesn’t start near the site. So, the cell range must extend at least as far as the ISD to ensure the vertical coverage lobes overlap and there is no coverage holes.

UAV BS cell range should at least be equal to ISD to ensure vertical coverage lobes overlap and there is no coverage holes.

##### Network layout

For the TN network, RAN4 assumes a hexagonal grid of 19 macro sites and 3 sites per sector. However, the network layout will be highly dependent on the studied scenario. We have briefly explained in our companion paper [3] on why DL scenario should be also studied. Considering aggregate interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interferences. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to full model the UL interferences.

Considering aggregate interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interferences. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to full model the UL interferences.

RAN4 to clarify on how to model the TN and UAV network based on the DL and UL scenarios.

##### Colocation

From a co-existence perspective, to ensure robust specifications, in our view it makes sense to consider the worst-case among collocated and non-collocated. This assumption is also dependent on whether UAV UE is capable of beamforming or not.

The TN and UAV BS are collocated or non-colocated, depends on the UAV antenna assumption. However, non-colocated would be the worst-case scenario from co-existence perspective.

##### No. of cell per UAV BS

In the ATG study, the assumption was one cell per ATG BS. This assumption was considering vertical beam geometry towards the flight corridor path rather than a sectorized azimuth coverage. The UAV WID mentions that UAV will be in specific routes. A busier corridor or when in populated areas, many UAVs may be within a few kms. Such localized concentration of UAV UEs can aggregate interference towards the TN network. Another view on this could be how far from the UAVs can be and still contribute significant interference towards TN BS.

RAN4 to clarify the assumption of number of cells per UAV BS.

##### No. of active users in DL and UL per UAV-TN network

A further consideration is the number of UAV UE that are served from the UAV BS per cell. Usually in RAN4, the assumptions are either 1 active UEs per cell in DL and 1-3 active UEs per cell in UL.

RAN4 to clarify assumption on the number of DL and UL active users per cell in TN- UAV network within the simulation area.

##### UAV vertical – horizontal distribution

In the Rel-18 ATG study [2], Ericsson provided additional simulation results to study the impact of ATG UE height distribution on the TN DL. The common assumption to derive the ATG UE ACS was with uniform ATG UE distribution between 3-10 km altitude. However, from the additional results it was observed that the ATG UE suffers higher degradation than the acceptable 5% during take-off and landing i.e. at 3 km as compared to two other cases of uniform distribution and fixed 10 km. It was not concerning as aircrafts are usually at altitudes close to 10km or higher when cruising.

In DL case when ATG UEs are victim, ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

RAN4 should further discuss what shall be the UAV UE distribution both vertically and horizontally.

##### UAV density

Here UAV density means the number of UAV UEs that are simultaneously active within a coverage area or a flight route. It is unclear from an aviation safety perspective, what shall be the maximum number of UAVs at a particular coverage area to avoid collision maintaining minimum separation distance. This is an important assumption to model the UAV density, particularly in cases where aggregated interference from multiple UAVs can impact the TN network.

RAN4 to clarify from an aviation safety perspective, on the number of maximum UAV UEs that can be modelled at a particular coverage area maintaining minimum separation distances.

##### UAV UE power control

The motivation for this work item is to enhance the UL throughput and UL coverage. Since we are examining HPUE UAVs, the presence should be justified by scenarios in which 23 dBm transmit power would be insufficient. It could be the case that the UAV UEs is a near the serving BS and may therefore achieve a high UL SNR without the high transmit power, particularly if the UAV BS main lobe faces the HPUE UAV. Therefore, the study should investigate the actual required transmit power of the UAV UEs and ensure that a situation is simulated where these HPUEs are always at high power at 33 dBm.

The study should investigate the actual required transmit power of the UAV UEs and ensure that a situation is simulated where these HPUEs are always at high power

## 2.2	BS antenna assumptions

### 2.2.1	UAV BS antenna configuration

In the Rel-18 Study on Air-to-ground network for NR documented in TR 38.876 [2], there were two models agreed for ATG BS considering Rural scenario for the co-existence evaluations –

- Non-subarray model 8x8 configuration with a mechanical uptilt of 14 degrees

- Subarray model 4x8x3 configuration with a mechanical uptilt of 6.5 degrees

The UAV AAS BS antenna parameters can be adapted from the Rel-19 Study on IMT parameters documented in TR 38.922 [4] from Clause 4.4 for 1710 to 4990 MHz, with the array antenna model described in Clause 7.1.1 [4]. The agreed parameters have been copied below for convenience -

Table 4.4.1.2-1: IMT parameters relevant for 1710 to 4990 MHz

| Parameter | Macro Rural | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 4 | 4 | 4 | 8 |
| N | 8 | 8 | 8 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 46 dBm | 37 dBm |
| mech | TBD | 6 deg. | 6 deg. | N/A |

Adopt the 4400 to 4800 MHz sub-array model with 4x8x3 antenna configuration of Macro Rural scenario for UAV AAS BS antenna configuration, as captured in TR 38.922 Clause 4.4.1.

The mech (mechanical uptilt) parameter needs to be discussed further. It is not clear from the WID [1] whether the UAV BS antennas will point towards the sky or towards the horizon with some uptilt. In our understanding, the UAV BS uptilt has a direct implication on the BS coverage and the maximum-minimum distance of the UAV UEs from the UAV BS. If we recall from the Rel-18 ATG co-existence study, it was agreed that the ATG BS antennas will point towards the horizon with an uptilt, as depicted in the Figure 1 below -

Figure 1: UAV BS pointing towards the horizon with overlapping coverage areas

The UAV BS pointing at the horizon with some uptilt will serve the UAV UE that are at a distance, but not the UAV UE that are hovering directly above or close to the UAV BS.

RAN4 to clarify whether the UAV BSs will point towards the horizon with an uptilt (same as ATG study assumptions) or towards the sky.

For co-existence simulations, it is important to identify which antenna uptilt should be used and the maximum-minimum horizontal coverage that the UAV UE can be served by the UAV BS. Figure 2 below provides an illustration.

Figure 2: UAV BS pointing towards the horizon with overlapping coverage areas

It is not wrong to argue that for the UAV case, the maximum horizontal service distance depends on the antenna pattern of the UAV BS and not the ISD. The UAV BS can beamform without interfering with one another, even if the coverage range overlaps.

The upper end of the coverage range is determined by the angle of the UAV UE at the lowest possible altitude at the edge of the coverage. The lower end of the coverage range is determined by the angle of the UAV UE that is at the maximum possible altitude when it is at the minimum distance to the BS. The WID states that ISD is around 2-4 kms. Traditionally for TN UEs, cell radius is less than the ISD based on the hexagonal geometry. However, based on Figure 1 with UAV BS pointing at the horizon, the cell range will have to be at-least equal or higher than ISD to avoid coverage holes for UAV case.

Assuming, minimum vertical UAV UE-BS distance as 10m, the UAV UE with respect to the horizon will have an –

- An elevation angle of 0.143 degrees when maximum horizontal distance is 4km

- An elevation angle of 0.286 degrees when maximum horizontal distance is 2km

A BS with 4x8x3 antenna configuration and 65 degrees single element vertical 3 dB beamwidth will result in ~18 degrees effective vertical 3 dB beamwidth. The upper edge of the 3dB beamwidth will then be at 18.143/ 18.286 degrees with respect to the horizon. The WID states that the coverage height is upto 600m ~1 km. Based on the upper edge 3 dB beamwidth of ~18 degrees (rounding off), the minimum horizontal distance between the UAV BS and UAV UE will be –

- ~1.85 km when the maximum vertical coverage height is 600 m

- ~3.08 km when the maximum vertical coverage height is 1 km

If the UAV UEs horizontal distance is less than the minimum distance as stated above, then the closer UAVs will sit outside the main lobe of the closest UAV BS.

For the stated ISD and coverage height in the WID for co-existence evaluations, the UAV BS uptilt comes to be ~9 degrees.

RAN4 needs to further discuss what shall be the assumptions of minimum horizontal distance between UAV BS-UE, minimum vertical UAV BS-UE distance and UAV BS antenna uptilt for the simulation assumptions.

### 2.2.2	TN BS antenna configuration

The listed bands in WID for TDD operation are n41, n77, n78 and n79. The TN AAS BS antenna parameters can be adapted from the Rel-19 Study on IMT parameters documented in TR 38.922 [4] from Clause 4.4 for 1710 to 4990 MHz, with the array antenna model described in Clause 7.1.1 [4]. The relevant deployment scenario is Urban Macro.

Adopt the 4400 to 4800 MHz sub-array model with 4x8x3 antenna configuration of Urban Macro scenario for TN AAS BS antenna configuration, as captured in TR 38.922 Clause 4.4.1.

## 2.3	UE antenna assumptions

### 2.3.1	UAV UE antenna configuration

The WID states that in Rel-18 ATG study [2], two ATG UE antennas were introduced – one is omnidirectional antenna and the other is an antenna array. However, it is worthy to highlight that the antenna model were highly specific to the deployment scenario and different frequency in the analysis. Based on our understanding, the choice of positioning of UAV UEs whether the UEs are hovering directly over/ near to a TN BS or are at far at the horizon will play a significant role in evaluating the uplink performance degradation.

An omni-directional UE antenna with a large UE transmit power would be a significant co-existence scenario to considering the worst-case. A UE with adaptive beamforming is more realistic, however it will reduce the interference. The assumption for the UE antenna could impact both co-existence analysis and RF requirements.

RAN4 should perform co-existence analysis considering omni-directional UE antenna, as it would reflect the worst-case scenario.

### 2.3.2	TN UE antenna configuration

The TN UE can be considered an omni-directional UE antenna configuration with 0 dBi gain.

The TN UE can be considered an omni-directional UE antenna configuration with 0 dBi gain.

## 2.4	Uplink co-existence simulation considerations

### 2.4.1	UAV uplink to TN uplink co-existence

In the Rel-15 Study on Enhanced LTE support for Aerial Vehicles documented in TR 36.777 [5], the deployment assumptions were considering both aerial UEs and terrestrial UEs in a single network with IoT (Interference over Thermal) as a performance metric among others. However, there were some general observations made on the UL interference in scenarios involving aerial UEs with key take aways summarized below –

  - Increasing the ratio of aerial UEs caused more UL interference to multiple cells.

  - Increased UL interference causes a potential risk for eNB to experience too strong reception power.

It should be noted that the assumptions for this study can be taken as a good starting point, however all may not be relevant considering the current study.

The co-existence scenarios concerning this study shall assume two separate networks. Here, the uplink interference at terrestrial base stations (TN BSs) originates from transmissions by UAV UEs. The positioning of UAV UEs and their UE antenna type may play a significant role in evaluating the UL performance in this scenario.

- When the UAV UEs are hovering directly above or near to a TN BS:

The pathloss is low because of the short distance and LoS. However, since the TN BS antennas are down tilted to serve the TN UEs, the UAV UE may be received through TN BS antenna sidelobes or null regions. With the assumption of omni-directional UAV UE antenna type and high transmit power, the received interference power at the TN BS can be significant, even through sidelobes. Now, if we assume beamforming capable UAV UEs, the impact to the nearby TN BS may be limited if the beam is steered towards the serving UAV BS. This would imply assuming an omni-directional antenna for UAV UE antenna with a high number of active UAVs near the TN BS could be the worst case. It is important to note that the impact may be localized among the affected TN BSs.

- When the UAV UEs are far at the horizon from TN BS

The UAVs which are far will be at horizon from the TN BS point of view. They will have LoS conditions to many TN BSs across a wide area. Hence, the uplink interference will be spatially distributed resulting in degradation across a cluster of TN cells. The aggregated impact scales up with UAV density, as many UAVs can appear to be at the horizon at once.

For the UAV UL to TN UL co-existence analysis, RAN4 should clarify assumptions on the UAV UEs positioning, UAV UEs antenna type and number of active UAVs in a certain time, which will play a significant role in modelling the co-existence conditions.

### 2.4.2	TN uplink to UAV uplink co-existence

The uplink interference at UAV BSs originates from transmission by TN UEs. It is most likely that maximum interference towards the UAV BSs will arise from the TN UEs that are close to the UAV BS. However, it is reasonable to assume that the interference would be less severe as UAV BS points upwards whereas the current ACLR/ ACS requirements are derived assuming victim BS antenna points downwards towards the aggressor TN UEs.

For the TN UL to UAV UL co-existence analysis, the interference would be less severe as UAV BS points upwards whereas the current ACLR/ ACS requirements are derived assuming victim BS antenna points downwards towards the aggressor TN UEs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	UAV BS cell range should at least be equal to ISD to ensure vertical coverage lobes overlap and there is no coverage holes.

Observation 2	Considering aggregate interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interferences. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to full model the UL interferences.

Observation 3	The TN and UAV BS are collocated or non-colocated, depends on the UAV antenna assumption. However, non-colocated would be the worst-case scenario from co-existence perspective.

Observation 4	In DL case when ATG UEs are victim, ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

Observation 5	The study should investigate the actual required transmit power of the UAV UEs and ensure that a situation is simulated where these HPUEs are always at high power

Observation 6	For the stated ISD and coverage height in the WID for co-existence evaluations, the UAV BS uptilt comes to be ~9 degrees.

Observation 7	For the TN UL to UAV UL co-existence analysis, the interference would be less severe as UAV BS points upwards whereas the current ACLR/ ACS requirements are derived assuming victim BS antenna points downwards towards the aggressor TN UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to clarify whether UAV BS are deployed in Macro Urban or Macro Rural locations.

Proposal 2	RAN4 to clarify on how to model the TN and UAV network based on the DL and UL scenarios.

Proposal 3	RAN4 to clarify the assumption of number of cells per UAV BS.

Proposal 4	RAN4 to clarify assumption on the number of DL and UL active users per cell in TN- UAV network within the simulation area.

Proposal 5	RAN4 should further discuss what shall be the UAV UE distribution both vertically and horizontally.

Proposal 6	RAN4 to clarify from an aviation safety perspective, on the number of maximum UAV UEs that can be modelled at a particular coverage area maintaining minimum separation distances.

Proposal 7	Adopt the 4400 to 4800 MHz sub-array model with 4x8x3 antenna configuration of Macro Rural scenario for UAV AAS BS antenna configuration, as captured in TR 38.922 Clause 4.4.1.

Proposal 8	RAN4 to clarify whether the UAV BSs will point towards the horizon with an uptilt (same as ATG study assumptions) or towards the sky.

Proposal 9	RAN4 needs to further discuss what shall be the assumptions of minimum horizontal distance between UAV BS-UE, minimum vertical UAV BS-UE distance and UAV BS antenna uptilt for the simulation assumptions.

Proposal 10	Adopt the 4400 to 4800 MHz sub-array model with 4x8x3 antenna configuration of Urban Macro scenario for TN AAS BS antenna configuration, as captured in TR 38.922 Clause 4.4.1.

Proposal 11	RAN4 should perform co-existence analysis considering omni-directional UE antenna, as it would reflect the worst-case scenario.

Proposal 12	The TN UE can be considered an omni-directional UE antenna configuration with 0 dBi gain.

Proposal 13	For the UAV UL to TN UL co-existence analysis, RAN4 should clarify assumptions on the UAV UEs positioning, UAV UEs antenna type and number of active UAVs in a certain time, which will play a significant role in modelling the co-existence conditions.

# References

[1]	RP-252908, “Enhancement of NR RF and RRM requirements for UAV”, 3GPP RAN#109, September 2025

[2]	3GPP TR 38.876 v18.2.0, “Air-to-ground network for NR”, July 2024

[3]	R4-2513864, “On General Aspects and DL co-existence considerations for UAV”, RAN4#116bis, Ericsson

[4]	3GPP TR 38.922 v19.1.0, “Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14800-15350 MHz”, June 2025

[5]	3GPP TR 26.777 v15.0.0, “Study on Enhanced LTE support for Aerial Vehicles”, December 2017