3GPP TSG-RAN4 #116-bis	R4-2513922

Prague, Czech Republic, 13th Oct. – 17th Oct. 2025

Title:	Assumption of eccentricity as orbital element in Keplerian model for NGSO

Source:			Anritsu Corporation

Agenda Item:			6.6.3.1

Document for:			Approval

# 1.	Introduction

RAN4 demodulation group continued a discussion on the time varying channel model for NGSO, and we agreed on the methodologies to estimate the satellite orbit by solving Keplerian model by Newton-Raphson method.

In this contribution, we share our view on one condition with one of the orbital elements, Eccentricity (e).

# 2.	Discussion

# 2.1 Relationship between Eccentricity and Satellite orbit for NGSO

As already captured in the appendix of the way forward [1], we use six orbital elements to estimate the satellite orbit such as Semi-major Axis, Eccentricity, Argument of Periapsis, etc. In these elements, Eccentricity is a measure of how much an orbit deviates from being circular. It defines the shape of the orbit, ranging from 0 (a perfect circle) to values approaching 1 (a highly elongated ellipse). In satellite dynamics, eccentricity determines how far the satellite's path stretches away from a circular trajectory, affecting its speed and distance from the central body at different points in the orbit.

Observation 1: One of the six orbital elements “Eccentricity” is a measure of how much an orbit deviates from being circular and it defines the shape of the orbit.

2.2 Relationship between Eccentricity and other orbital elements

If we see the formulas in the WF [1] to derive other elements such as “Argument of Periapsis” or “Mean Anomaly at time 0”, we can find that Eccentricity (e) is included in their formulas especially at denominator of fractions.

As one of the examples, we extract the formula to derive Argument of Periapsis as follows.

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

As mentioned in the sub-clause 2.1, in a case zero Eccentricity (e) is applied, which denotes that the orbit is a perfect circle, division by zero occurs within the relevant formulas and it causes w and M0 to become indeterminate.

Observation 2: “Argument of Periapsis” and “Mean Anomaly at time 0” become indeterminate when Eccentricity (e) is zero.

One of the expected solutions to avoid this issue is that we assume satellite orbits to be ellipse and exclude the case that Eccentricity becomes zero as the test condition in a case of NGSO.

Proposal 1: Assume Eccentricity (e) as more than zero, which means the satellite orbit is ellipse in a case of NGSO.

To capture that assumption on Eccentricity, we recommend that we add a sentence within descriptions to derive orbital elements in the associated specification, i.e. TS 36.102 and TS 38.101-5.

Proposal 2: Capture the following sentence to clarify the condition of Eccentricity in descriptions to derive orbital elements in TS 36.102 and TS 38.101-5.

“The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero.”

Associated draft CRs [2, 3] are also submitted in this meeting.

- Conclusion

In this contribution, we shared our view on one condition with one of the orbital elements, Eccentricity (e).

Observation 1: One of the six orbital elements “Eccentricity” is a measure of how much an orbit deviates from being circular and it defines the shape of the orbit.

Observation 2: “Argument of Periapsis” and “Mean Anomaly at time 0” become indeterminate when Eccentricity (e) is zero.

Proposal 1: Assume Eccentricity (e) as more than zero, which means the satellite orbit is ellipse in a case of NGSO.

Proposal 2: Capture the following sentence to clarify the condition of Eccentricity in descriptions to derive orbital elements in TS 36.102 and TS 38.101-5.

“The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero.”

Associated draft CRs [2, 3] are also submitted in this meeting.

# 4.	References

[1] R4-2512525, “Way Forward for [116][323] NTN_testing_NGSO_channel_model_and_demod”, Samsung, RAN4 #116, Bengaluru, India

[2] R4-2513920, “Draft CR to introduce a new annex to 36.102 for NGSO time varying doppler shift and propagation delay modelling”, Anritsu Corp., RAN4 #116-bis, Prague, Czech Republic

[3] R4-2513921, “Draft CR to introduce a new annex to 38.101-5 for NGSO time varying doppler shift and propagation delay modelling”, Anritsu Corp., RAN4 #116-bis, Prague, Czech Republic