# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513928

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on 6MHz CBW BS RF

# Agenda Item:   7.1.5.1

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WI to introduce 6MHz CBW for bands n5, n8, n12, n85, n29 was approved [1]. The objective of this WID is:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

Specifying BS RF requirements is needed to support 6MHz CBW, including both conducted and OTA requirements. In this contribution, we provide our views on the impacts on BS RF requirements.

- Discussion

## 2.1 Scope

In our opinion, most of RAN4’s works on 7MHz CBW are good reference and we can follow those conclusions when introducing 6MHz CBW to reduce workload. Before specifying the BS RF requirements, the scope of this WID should be clarified.

NB-IoT

NB-IoT operation in NR in-band is supported for 7MHz CBW because NB-IoT support is specified per NR band, not per channel BW. Considering all channel bandwidths specified in a band were assumed being able to support NB-IoT, 6MHz should not be an exception.

Proposal 1: NB-IoT operation in NR in-band should be supported for 6MHz CBW.

Multi-carrier and CA

Muti-carrier and CA related requirements for 6MHz should be specified within this WI, same as 7MHz CBW.

Proposal 2: Muti-carrier (including MSR) and CA related requirements should be specified for 6MHz WID but no new BCS is specified.

IAB and NCR

For IAB, it has been agreed in RAN4#115 meeting in CR R4-2507239[2] that channel bandwidths below 10MHz are not supported. Thus 6MHz is precluded naturally.

For NCR, 7MHz CBW is not supported. We prefer to preclude 6MHz as well to avoid too much work on NCR ACLR requirements since the intention of 6MHz CBW is not to support NCR nodes.

Proposal 3: 6MHz CBW should not be supported by IAB and NCR.

## 2.2 FRC

For both conducted and OTA BS RF requirements for 7MHz CBW, RAN4 determined to reuse the legacy fixed reference channels (FRCs) defined in Annex in TS38.104. The direct starting point was to consider the convenience of introducing new bandwidth such as 6MHz in the future and avoid too much simulation workload. Therefore, for 6MHz CBW, of course reusing the legacy FRCs is reasonable.

Proposal 4: Reuse the legacy FRCs to specify the conducted and OTA requirements for 6MHz CBW.

## 2.3 BS RF requirements

Based on NRB=30 for 6MHz CBW with 15kHz SCS we proposed in another contribution [3], the spec impact on TS38.104 and our views on the BS RF requirements are listed in Table 1.

Table 1  Spec impact on BS RF to introduce 6MHz CBW

| Clause | Requirements | ZTE’s views on the BS RF requirements |
| --- | --- | --- |
| 6.3.3 | Total power dynamic range | Total power dynamic range can be derived according to transmission bandwidth configuration of 6 MHz CBW. Proposal that the total power dynamic range for 6MHz CBW with 15kHz SCS is 14.7 dB. |
| 6.3.4 | NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | Add 6MHz CBW to Table 6.2.4.2-1, reuse legacy NB-IoT RB power dynamic range for 3, 5, 7, 10MHz CBW. |
| 6.6.3 | Adjacent Channel Leakage power Ratio | Add 6MHz CBW to Table 6.6.3.2-1, 6.6.3.2-2a, 6.6.3.2-3, reuse legacy ACLR requirements. |
| 7.2 | Reference sensitivity level | Add 6MHz to Table 7.2.2-1, 7.2.2-2 and 7.2.2-3, reuse legacy REFSENS for 5, 7, 10, 15MHz CBW. |
| 7.3 | Dynamic range | ▪ Reference measurement channel and wanted signal mean power defined for 5MHz CBW with 15kHz SCS can be reused for 6MHz CBW with 15kHz SCS. ▪ The interfering signal mean power could be -81.7dBm for WA, -76.7dBm for MR and -73.7dBm for LA. - ▪ For NB-IoT operation in NR in-band, the interfering signal mean power is -81.7dBm for WA, -76.7dBm for MR and -73.7dBm for LA. |
| 7.4.1 | Adjacent Channel Selectivity (ACS) | - ▪ Add 6MHz CBW to Table 7.4.1.2-1, reuse wanted signal mean power as PREFSENS + 6 dB. - ▪ Add 6MHz CBW to Table 7.4.1.2-2, the interfering signal centre frequency offset could be ±2.5125. |
| 7.4.2 | In-band blocking | ▪ Add 6MHz CBW to Table 7.4.2.2-1, reuse legacy Base station general blocking requirement for 5~20MHz CBW. ▪ Add 6MHz CBW to Table 7.4.2.2-2a, reuse legacy Base Station narrowband blocking requirement for NB-IoT operation in NR in-band for 3~50MHz CBW. ▪ Add 6MHz CBW to Table 7.4.2.2-3, reuse 5MHz interfering signal and the interfering RB centre frequency offset could be ±(360+m*180). |
| 7.7 | Receiver intermodulation | ▪ Add 6MHz CBW to Table 7.7.2-2, reuse legacy 5MHz interfering signal ans the interfering signal centre frequency offset could be ±7.475 for CW and ±17.5 for 5MHz interfering signal. ▪ Add 6MHz CBW to Table 7.7.2-4, reuse legacy 5MHz interfering signal ans the interfering signal centre frequency offset could be ±425 for CW and ±1240 for 5MHz interfering signal. |
| 7.8 | In-channel selectivity | ▪ Reuse the reference measurement channel and interfering signal of 5MHz CBW and 7MHz CBW. ▪ Propose the type of interfering signal for ICS of 6MHz is same as 7MHz CBW, i.e., DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs. ▪ The interfering signal mean power could be -79.6dBm for WA, -74.6dBm for MR and -71.6dBm for LA. ▪ For NB-IoT operation in NR in-band, propose interfering signal mean power is -79.6dBm for WA, -74.6dBm for MR and -71.6dBm for LA. |
| 10.3 | ### OTA reference sensitivity level | Add 6MHz to Table 10.3.2-1, 10.3.2-2 and 10.3.2-3, reuse legacy REFSENS for 5, 7, 10, 15MHz CBW. |
| 10.4 | OTA dynamic range | ▪ Reference measurement channel and wanted signal mean power defined for 5MHz CBW with 15kHz SCS can be reused for 6MHz CBW with 15kHz SCS. ▪ The interfering signal mean power could be -81.7-ΔOTAREFSENS dBm for WA, -76.7-ΔOTAREFSENS dBm for MR and -73.7-ΔOTAREFSENS dBm for LA. ▪ For NB-IoT operation in NR in-band, propose interfering signal mean power is -81.7-ΔOTAREFSENS dBm for WA, -76.7-ΔOTAREFSENS dBm for MR and -73.7- ΔOTAREFSENS dBm for LA. |
| 10.5.1 | OTA adjacent channel selectivity | - ▪ Add 6MHz CBW to Table 10.5.1.2-1, reuse wanted signal mean power as EISminSENS + 6 dB. ▪ Add 6MHz CBW to Table 10.5.1.2-2, the interfering signal centre frequency offset could be ±2.5125. |
| 10.5.2 | OTA in-band blocking | ▪ Add 6MHz CBW to Table 10.5.2.2-1, reuse legacy General OTA blocking requirement for BS type 1-O for 5~20MHz CBW. ▪ Add 6MHz CBW to Table 10.5.2.2-2, reuse legacy OTA narrowband blocking requirement for BS type 1-O for 5~20MHz CBW. ▪ Add 6MHz CBW to Table 10.5.2.2-3, reuse 5MHz interfering signal and the interfering RB centre frequency offset could be ±(360+m*180). |
| 10.8 | OTA receiver intermodulation | ▪ Add 6MHz CBW to Table10.8.2-2, reuse legacy 5MHz interfering signal ans the interfering signal centre frequency offset could be ±7.475 for CW and ±17.5 for 5MHz interfering signal. ▪ Add 6MHz CBW to Table 10.8.2-4, reuse legacy 5MHz interfering signal ans the interfering signal centre frequency offset could be ±425 for CW and ±1240 for 5MHz interfering signal. |
| 10.9 | OTA in-channel selectivity | ▪ Reuse the reference measurement channel and interfering signal of 5MHz CBW and 7MHz CBW. ▪ Propose the type of interfering signal for ICS of 6MHz is same as 7MHz CBW, i.e., DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs. ▪ The interfering signal mean power could be -79.6-ΔminSENS dBm for WA, -74.6-ΔminSENS dBm for MR and -71.6-ΔminSENS dBm for LA. ▪ For NB-IoT operation in NR in-band, propose interfering signal mean power is -79.6-ΔminSENS dBm for WA, -74.6-ΔminSENS dBm for MR and -71.6-ΔminSENS dBm for LA. |

Proposal 5: Agree with the BS RF requirements for 6MHz CBW as listed in Table 1.

- Conclusion

Proposal 1: NB-IoT operation in NR in-band should be supported for 6MHz CBW.

Proposal 2: Muti-carrier (including MSR) and CA related requirements should be specified for 6MHz WID but no new BCS is specified.

Proposal 3: 6MHz CBW should not be supported by IAB and NCR.

Proposal 4: Reuse the legacy FRCs to specify the conducted and OTA requirements for 6MHz CBW.

Proposal 5: Agree with the BS RF requirements for 6MHz CBW as listed in Table 1.

- Reference

- RP-252952 New WID: UE RF enhancements for NR FR1, Phase 5.

- R4-2507239 CR to 38.174 with clarification for channel bandwidths below 10 MHz, Nokia, RAN4#115, Malta, May, 2025.

- R4-2513929 Discussion on 6MHz CBW UE RF, ZTE, RAN4#116bis, Prague, CZ, Oct. 2025.