3GPP TSG-RAN WG4 Meeting #116bis	R4-2513960

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.19.4

Source:	Ericsson

Title:	Discussion on Rel-19 NTN phase 3 demodulation requirement

Document for:	Discussion

# 1	Introduction

In RAN#104, a revised WID [1] on NR NTN enhancements was endorsed for Release 19. The objectives for UE demodulation are mainly on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2	UE demodulation requirements

In this section, we discussed the UE demodulation requirements for adding the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

## 2.1	RedCap demodulation for NTN

According to the previous agreements, the following test cases will be specified:

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 |

We have provided our simulation results in our companion paper [3] for alignment.

For the FRC for HD-FDD RedCap demodulation, there’s a proposal to define a new HD-FDD reference channel for MCS4/52PRB test. The main purpose is to configure 12 DM-RS REs (1+1) instead of 18 REs (1+1+1).

Issue 2-1-2: FRC for HD-FDD RedCap demodulation Way forward: - Reuse “R.PDSCH.1-1.2 HD-FDD” in TS38.101-4 for MCS13/52PRB test. - FFS whether to define a new HD-FDD reference channel for MCS4/52PRB tests. - Note: R.PDSCH.1-1.1 HD-FDD in TS 38.101-3 configures 18 DM-RS REs. MediaTek proposes to configure 12 DM-RS REs (1+1) instead of 18 REs (1+1+1).

As we browsed the spec. 38.101-4, we found the following facts:

- For the tests with QPSK + 52PRBs, the number of DMRS in the existing FRC can be configured with either 18 or 12

  - For those tests that DMRS configured with 18, we also found the propagation condition is configured with TDLB100-400

  - For those tests that DMRS configured with 12, we also found the propagation condition is configured with TDLA30-10

The reason we chose 18 DMRS REs for those cases with QPSK+52PRBs+TDLB100-400 is mainly because the TDL-B channel model with delay spread between 100 ns and 400 ns introduces more severe frequency selectivity, which means the channel response varies significantly across subcarriers. Under this condition, if we keep the DMRS density too low, the channel interpolation error becomes dominant and directly increases the demodulation error rate. Furthermore, with QPSK modulation and a relatively large allocation size (52 PRBs), the system is already operating with a low SINR margin and is thus more sensitive to any channel estimation inaccuracy. By selecting 18 DMRS REs, we provide a denser pilot grid that captures these rapid channel variations more precisely, ensuring robust equalization and preventing BLER from exceeding the 10 % target.

While we looked at the 38.101-5, we found that the number of DMRS is always configured as 12 for both 
NTN-TDLA100-200 and NTN-TDLC5-200 test cases, mainly because the Doppler value for these two NTN channels is not as high as TDLB100-400 channel model.

Therefore, we think it is reasonable to reuse the NTN test configuration to redefine the reference channel to configure 12 DMRS REs instead of 18 REs.

Proposal 1: Define a new HD-FDD reference channel for MCS4/52PRB tests, e.g. configure 12 DM-RS REs

## 2.2	eRedCap demodulation for NTN

According to the previous agreements[2], the following test cases have been specified:

Table 2.2-1 Test cases without baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 |

Table 2.2-2 Test case with baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 |

As for the FRC, we are of the opinion that to reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth.

Proposal 2: Reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth

Regarding the applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirements, we insist that RAN4 should apply such time-varying channel model if there is a chance, since it provides a more precise/realistic transmission condition. For now, the WI NR_IoT_NTN_req_test_enh has completed the DL Doppler shift and propagation delay modeling with NGSO satellite and RAN4 is discussing the performance impact to the existing Rel-17 NTN PDSCH demodulation requirements if the model is applied. Considering the realistic scenario, RAN4 should discuss whether the NGSO channel model is applied for (e)RedCap UE in NTN later. We are open to consider it if it finally concludes with no additional margin to be added.

Proposal 3: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

# 3	SAN demodulation requirements

Issue 1-2-3: Frequency offset of two multiplexed UEs.

- Consider setting the same frequency offset for two UEs.

- Evaluate two frequency offsets: 0 Hz and 200 Hz. Down select to one value according to the simulation results.

- If there is no performance difference, select 200Hz.

Issue 1-2-4: Channel model

- Apply the same channel model to the multiplex UEs.

- Consider NTN-TDLC5-200. Interested companies can provide the simulation results with NTN-TDLA100-200.

- Down select one channel model according to the simulation results.

Issue 1-2-8: Antenna configuration

- FFS whether to apply the existing SAN PUSCH demodulation requirements applicability between 1Rx and 2Rx requirements or not.

In RAN4#116 meeting, companies agreed on defining SAN PUSCH requirements with orthogonal cover codes (OCC) length-2. Further, companies had agreement on many simulation assumptions. As per the WF [2], remaining open issues copies below are discussed in this contribution.

We have provided simulation results in our other contribution [3]. We provide our view in this contribution based on our results.

- Frequency offset

As per RAN4 assumptions, we typically consider 0.1ppm carrier frequency offset (CFO) in the RF requirements, while ideally as per the definition of OCC group there should not be any CFO between the multiplexed UEs. CFO will result in phase rotation in time domain which would break the orthogonality of the OCC code. Therefore, in the last meeting, companies prefer to consider both 0 Hz and 200 Hz CFO for initial evaluation. Based on our simulations [3], we do not observe any performance difference between 0 and 200 Hz.

Observation 1: There is no performance difference by setting CFO of 0 and 200Hz for both multiplexed UEs.

In RAN4#116 meeting companies agreed to consider the same frequency offset for the two multiplexed UEs. As per our understanding, UE pre-compensates the frequency error that occurs due to high speed (or Doppler) of the satellite movement in low earth orbit (LEO). However, some residual frequency errors are expected even after pre-compensation, and it seems highly unlikely that both OCC multiplexed UEs in such scenario would have the same residual frequency error. Therefore, considering the same CFO for both multiplexed UEs may not be feasible, and we prefer to revisit and discuss this further with other companies.

In our opinion, different frequency offsets should be considered for both multiplexed UEs in one OCC group and the worst-case scenario would be when the difference of CFO between the two UEs is 200 Hz, i.e., either [0, 200 Hz] or [-100 Hz,100 Hz] for [UE1, UE2] respectively. Based on our understanding, even in the worst-case scenario, we don’t expect performance variation between 0 and 200 Hz, as seen in Table 1.

Observation 2: It is highly unlikely that two multiplexed UEs in one OCC group would have the same CFO.

Observation 3: It would be feasible to consider the worst-case scenario for CFO setting of the two UEs, i.e., the difference between the CFO of two UEs should be 200Hz. However, even in the worst case no significant performance difference (<0.5dB) is observed from our simulations.

Proposal 4: RAN4 SAN Demod to revisit and discuss the frequency offset assumption for two multiplexed UEs in one OCC group.

Proposal 5: As no performance difference in the worst case for two OCC multiplexed UEs, CFO of 200 Hz can be considered. However, RAN4 needs to evaluate how to highlight in the specification on the differential CFO between the UEs.

For illustration and understanding, we have simulated by setting CFO as [700, 700 Hz] and [0, 700 Hz] for [UE1, UE2] respectively. As seen from our results in Table 3.1, there is significant performance degradation in worst case scenario.

Table 3.1: Frequency offset and Channel model comparison

| TX | RX | Propagation conditions and correlation matrix | Frequency offset UE1 (Hz) | Frequency offset UE2 (Hz) | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] per UE |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-200 Low | 0 | 0 | 5 | -6.21 |
|  |  |  | 200 | 200 |  | -6.23 |
|  |  |  | 700 | 700 |  | -4.67 |
|  |  |  | 0 | 200 |  | -6.05 |
|  |  |  | 0 | 700 |  | 2.11 |
| 1 | 1 | NTN-TDLC5-200 Low | 0 | 0 |  | -8.22 |
|  |  |  | 200 | 200 |  | -8.26 |
|  |  |  | 700 | 700 |  | -8.25 |
|  |  |  | 0 | 200 |  | -8.03 |
|  |  |  | 0 | 700 |  | -7.58 |

- Channel Model

As per the WF [2], it is reasonable to consider the same channel model for two UEs multiplexed with OCC for simulations. Regarding the propagation condition, NTN-TDLA100-200 channel model is used for non-LoS (line of sight) and NTN-TDLC5-200 for LoS scenarios for Rel-17 FR1-NTN requirements.

Based on our evaluation given in Figure 3.1, we observe that with LoS channel model peak throughput can be achieved but not in the case for non-LoS channel.

Figure 3.1: Comparison for NTN-TDLA and NTN-TDLC channel

In a typical real-network configuration, peak throughput is expected to be achieved. Since the companies have agreed on MCS5, if such a low MCS level cannot deliver the required peak throughput, then non-LoS channel may not be feasible.

Further, Table 3.1 shows that the LoS channel is significantly more robust than the non-LoS channel, as increasing CFO results in minimal performance degradation.

Observation 4: Peak throughput can be achieved under the NTN-TDLC channel model; however, the same performance is not attainable under the NTN-TDLA channel model. Also, NTN-TDLC is more robust to frequency errors.

Proposal 6: Consider NTN-TDLC5-200 channel model SAN demodulation performance requirements with OCC.

- Applicability rule

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.48 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported connectors, the tests shall be done by using performance requirement for the closest number of connectors lower than this highest number of supported connectors.

In TS 38.181, following applicability rules exist:

Based on this, it may not be needed to define new applicability rules for 1Rx and 2Rx requirements.

Proposal 7: The current applicability rule apply for 1Rx and 2Rx SAN PUSCH demodulation requirements.

# 3	Summary

Based on the discussion, following observations are captured:

Observation 1: There is no performance difference by setting CFO of 0 and 200Hz for both multiplexed UEs.

Observation 2: It is highly unlikely that two multiplexed UEs in one OCC group would have the same CFO.

Observation 3: It would be feasible to consider the worst-case scenario for CFO setting of the two UEs, i.e., the difference between the CFO of two UEs should be 200Hz. However, even in the worst case no significant performance difference (<0.5dB) is observed from our simulations.

Observation 4: Peak throughput can be achieved under the NTN-TDLC channel model; however, the same performance is not attainable under the NTN-TDLA channel model. Also, NTN-TDLC is more robust to frequency errors.

Based on the discussion, following proposals are captured:

Proposal 1: Define a new HD-FDD reference channel for MCS4/52PRB tests, e.g. configure 12 DM-RS REs

Proposal 2: Reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth

Proposal 3: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

Proposal 4: RAN4 SAN Demod to revisit and discuss the frequency offset assumption for two multiplexed UEs in one OCC group.

Proposal 5: As no performance difference in the worst case for two OCC multiplexed UEs, CFO of 200 Hz can be considered. However, RAN4 needs to evaluate how to highlight in the specification on the differential CFO between the UEs.

Proposal 6: Consider NTN-TDLC5-200 channel model SAN demodulation performance requirements with OCC.

Proposal 7: The current applicability rule apply for 1Rx and 2Rx SAN PUSCH demodulation requirements.

# References

[1]	RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

[2]	R4-2512644, “Way Forward for [116][331] NR_NTN_Ph3_demod”, Ericsson, RAN4#116, Bangalore.

[3]	R4-2513961, “Simulation results NTN enhancement”, Ericsson, RAN4#116bis, Prague, October 2025.