3GPP TSG-RAN WG4 Meeting #116bis		R4-2513990
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on R20 4Tx and 2Tx HPUE

Source:	OPPO

Agenda item:	7.1.4

Document for:	Approval

Introduction

In Rel-20, the UE RF enhancements focus on several HPUEs, among them 4x26dBm and 2x29dBm for FWA is one of the topics [1]. This paper shares our view on this aspect.

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

Discussion

Power class

For 4x26dBm and 2x29dBm FWA, the max power that can transmit is 32dBm. In RAN#109, when the WID was approved, there were some discussions on which power class is targeting for this kind of UE. There were two views, one is to define PC1bis for 32dBm to fully utilize the PA capabilities, the other is to reuse PC1 and limit this kind of UE to 31dBm or using power boosting approaches to reach 32dBm.

In our view, Tx power is key for UL coverage, and in the past releases many efforts have been put on increase the Tx by even 0.5dB. There is no reason to give up 1dB here just because of keeping alignment with the current PC1 public safety UE.

In the past, RAN4 spent whole release to enhance Tx power with 0.5dB, there is no reason to give up 1dB in Rel-20.

The PC1 defined as 31dBm was based on public safety UE probably with 1Tx where the max achievable power is 31dBm. Now for multi-Tx UE, its power can easily exceed PC1, therefore, we suggest to enable the full power of multi-Tx UE and no bounding to PC1.

PC1 was defined for public safety UE with 1Tx 31dBm which is not same as current multi-Tx FWA whose Tx power can exceed PC1.

For 4x26dBm and 2x29dBm FWA, introduce a new power class, e.g., PC1bis, with 32dBm for it.

Coexistence simulations

In current spec, the ACLR for PC1 was specified as 37dB which is much higher than PC1.5. This may need to be reviewed if the above 4x26dBm and 2x29dBm with PC1 is introduced considering the UE type difference and also the usage scenario differences.

Current ACLR for PC1 is for public safety which is much higher than PC1.5 and may not be applicable to other UE types like handheld UE and FWA.

If PC1 is used as the power class for 4x26dBm and 2x29dBm, the ACLR needs to be re-evaluated.

Conclusion

This paper discussed the 4x26dBm and 2x29dBm HPUE, and got below observations and proposals.

Observation 1: 	In the past, RAN4 spent whole release to enhance Tx power with 0.5dB, there is no reason to give up 1dB in Rel-20.

Observation 2: 	PC1 was defined for public safety UE with 1Tx 31dBm which is not same as current multi-Tx FWA whose Tx power can exceed PC1.

Proposal 1: 	For 4x26dBm and 2x29dBm FWA, introduce a new power class, e.g., PC1bis, with 32dBm for it.

Observation 3: 	Current ACLR for PC1 is for public safety which is much higher than PC1.5 and may not be applicable to other UE types like handheld UE and FWA.

Proposal 2: 	If PC1 is used as the power class for 4x26dBm and 2x29dBm, the ACLR needs to be re-evaluated.

Reference

RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, RAN #109.