3GPP TSG-RAN WG4 Meeting #116bis		R4-2513991
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on R20 PC1.5 with 2Tx for FDD HPUE

Source:	OPPO

Agenda item:	7.1.4

Document for:	Approval

Introduction

In Rel-20, the UE RF enhancements focus on several HPUEs, PC1.5 with 2Tx for FDD is one of the topics[1]. This paper shares our view on this aspect.

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

Discussion

PC2 HPUE for FDD bands had been introduced in Rel-17. During the study, the primary concerns regarding FDD HPUE were UE heat dissipation and the power handling capability of filters. With advancements in both PA and filter technologies, supporting PC1.5 for FDD bands with 2Tx has now become a technically viable option.

PC1.5 with 2Tx for TDD band has been introduced in Rel-16, so the co-existence study for FDD bands with PC1.5 don’t need do.

PC1.5 with 2Tx for TDD band has been introduced in Rel-16.

In Rel-20, for PC1.5 with 2Tx for FDD bands, the Tx and Rx requirements which is related with power class need discuss and define, as list in following Table 1:

Table 1 Tx and Rx requirements related with power class

| Tx requirements | Power class and tolerance |
| --- | --- |
|  | MPR |
|  | AMPR |
|  | ACLR |
| Rx requirements | Reference Sensitivity Degradation from PC3 to PC1.5 |

Actually, PC1.5 with 2Tx for NTN FDD bands HPUE had been discussed in Rel-19, finally, it was removed from WID due to AMPR for NTN bands wasn’t defined with less operators’ attention.

In the discussion for PC1.5 with 2Tx for NTN FDD bands HPUE, it was agreed that PC1.5 with 2Tx for NTN FDD bands HPUE can reuse

- NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1.5 for NR-NTN

- TN PC1.5 ACLR

- TS 38.101-1 Table 6.2D.2-3 applies to NR-NTN PC1.5 HPUE MPR with 2Tx

PC1.5 with 2Tx for NTN FDD bands HPUE had been discussed in Rel-19 and some requirements had been agreed to reuse TN TDD PC1.5 with 2Tx.

Therefore, PC1.5 with 2Tx for FDD bands can also reuse the Tx requirements for PC1.5 with 2Tx for TDD bands.

For PC1.5 with 2Tx for NTN FDD bands HPUE, consider reuse following Tx requirements has been defined for PC1.5 with 2Tx for TDD bands:

- Nominal MOP and +2/-3 tolerance

- 31dBc ACLR

- MPR with 2Tx in Table 6.2D.2-3 of TS 38.101-1

APMR is band specifically, two example bands n1 and n25 are given in the WID, RAN 4 can first define AMPR based on these two bands NS values as list in Table 2:

Table 2 NS values for band n1 and n25

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |

Define AMPR for example bands n1 and n25 based on their NS values request, separately.

Since there is no impact of Tx to Rx for TDD bands, PC1.5 with 2Tx for FDD bands need study the impact of Tx to Rx for TDD bands based on the feasible duplexers for handheld and FWA separately.

RSD for PC1.5 with 2Tx FDD bands need study based on the feasible duplexers for handheld and FWA separately

Conclusion

This paper discussed the PC1.5 with 2Tx for FDD bands HPUE, and got below observations and proposals.

Observation 1: 	PC1.5 with 2Tx for TDD band has been introduced in Rel-16.

Observation 2: 	PC1.5 with 2Tx for NTN FDD bands HPUE had been discussed in Rel-19 and some requirements had been agreed to reuse TN TDD PC1.5 with 2Tx.

Proposal 1: 	For PC1.5 with 2Tx for NTN FDD bands HPUE, consider reuse following Tx requirements has been defined for PC1.5 with 2Tx for TDD bands:

•	Nominal MOP and +2/-3 tolerance

•	31dBc ACLR

•	MPR with 2Tx in Table 6.2D.2-3 of TS 38.101-1

Proposal 2: 	Define AMPR for example bands n1 and n25 based on their NS values request, separately.

Proposal 3: 	RSD for PC1.5 with 2Tx FDD bands need study based on the feasible duplexers for handheld and FWA separately

Reference

RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, RAN #109.