3GPP TSG-RAN4 Meeting # 116bis	R4-2513997

Prague, CZ, 13th – 17th, Oct. 2025

# Title: 	Discussion on R20 low-low band CA via switching

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.3

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, a new WID on enhancement of low NR band carrier aggregation via switching was approved.

The objectives are shown below:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

The reasons are similar with Rel-19 LBCA switching, which is there exists some challenges to implement the low-low band CA band combination due to poor antenna implementation when sharing one common antenna, or cost/size issue when separated antennas are used.

In this contribution, we gives some initial discussions on enhancement of low-low band CA via switching

# 2	Discussion

Comparing with the Rel-19 LBCA switching, additional switching case is introduced. Thus there are three cases up to Rel-20 for LBCA switching.

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

For Rel-19 LBCA switching, it is the switching case between Case 1 and Case 2, which is switchedDL option, and For Rel-20 enhanced LBCA switching, it is the switching case between Case 1 and Case 3, which is dual DL option. It should be noted that there is no switching happened between Case 2 and Case 3.

The switching scenarios for Rel-19 and Rel-20 LBCA switching are illustrated in Fig 1.

Fig 1. Rel-19 and Rel-20 LBCA switching scenario

The different switching scenarios are applied to the following FDD+SDL LBCA combination:

1. Rel-19 LBCA switching mechanistic (i.e. single DL switched) applies to CA_n5-n29, CA_n12-n29, CA_n14-n29, CA_n28-n67 and CA_n29-n71 according to the CR[2].

2. Rel-20 LBCA switching mechanistic (i.e. dual DL switched) applies to CA_n12-n29, CA_n28-n67 according to the WID

It should be noted that more LBCA combination will be introduced via basket WID. All band combination are comprised of single carrier in each band.

It can be seen that for CA_n12A-n29A and CA_n28A-n67A can support both single DL switched and dual DL switched options. However, we think it may not support both switching options for all FDD+SDL due to large Rx frequency separation between the two band such as CA_n5A-n29A.

In addition, in terms of the objectives, we can find that:

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

  - Reuse Rel-19 LB-CA RAN4 requirements as much as possible

Meanwhile, the deployment constraints are the same for Rel-19 and Rel-20 LBCA switching, which are：

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Observation 1. Some FDD+SDL low band CA can support both single DL switched and dual DL switched options

Observation 2. Rel-19 LBCA switching outcomes are reused as much as possible.

Observation 3. Only single carrier in each band for FDD-SDL LBCA band combination switching.

For switching period, there are 35us, 70us and 140us values are supported for Rel-19 LBCA single DL switching. For dual DL switching, the RF components involved in switching would be the same, and when we see the Tx switching feature, no matter switchedUL and dualUL, the switching period share the same sets of switching values. Thus for dual DL switching, we think the existing values should be reused.

Accordingly, the UL transient period (10us) is not overlapping with switching period. This is the common understanding for switching feature.

In our understanding, the R20 enhanced LBCA switching WID should not impact RAN1, which means the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching are reused as stated in the WID. Thus the switching location for dualUL switching should be in the switching from carrier.

Considering the above, we have the following proposal:

Proposal 1. For dual DL switching:

- reuse the same sets of switching values as single DL switching, i.e. 35us, 70us and 140us for dual DL switching.

- the UL transient period (10us) is not overlapping with switching period.

- the switching location for dualUL switching is in the switching from carrier.

Comparing with Rel-19 case 1<->case 2 and Rel-20 case 1 <->case 3, the common point is no Tx on FDD carrier 1. In the CR[2], there is only Tx on-off time mask requirements for switching defined in the specification, and there are no requirements defined for Rx switching mask instead of discussing the UE DL behaviour following the switch operation in the RRM session. We think the same approach should be adopted for dual DL switching, i.e. only define Tx on-off time mask requirements, also it seems Rel-19 Tx on-off time mask requirements can be adopted where minor modifications maybe needed to describe the dual DL switching in the specification.

Proposal 2. Reuse the Tx on-off time mask requirements of Rel-19 for defined, where minor modifications maybe needed to describe the dual DL switching in the specification.

In the feature list [3], the switching cases are included:

Thus for R20 dual DL switching, per and pair per band combination signalling is needed, the new switching cases should be informed to RAN2 to update the signalling (existing one or introduce new signalling) if necessary, which depends on RAN2 decision.

Proposal 3. Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating.

# 3 Conclusion

In this contribution, we give some discussions on the R20 enhancement FDD-SDL LBCA downlink carrier switching.

The observations, proposals and conclusions are:

Observation 1. Some FDD+SDL low band CA can support both single DL switched and dual DL switched options

Observation 2. Rel-19 LBCA switching outcomes are reused as much as possible.

Observation 3. Only single carrier in each band for FDD-SDL LBCA band combination switching.

Proposal 1. For dual DL switching:

- reuse the same sets of switching values as single DL switching, i.e. 35us, 70us and 140us for dual DL switching.

- the UL transient period (10us) is not overlapping with switching period.

- the switching location for dualUL switching is in the switching from carrier.

Proposal 2. Reuse the Tx on-off time mask requirements of Rel-19 for defined, where minor modifications maybe needed to describe the dual DL switching in the specification.

Proposal 3. Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating.

# 4	Reference

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei).

[2] R4-2511894, (NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1, Apple, TELUS.

[3] R4-2511883, Rel-19 RAN4 UE feature list for NR (version 2), CMCC