3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514015

Prague, Czech Republic, 13th – 17th October 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR MIMO Phase 5 demodulation requirements

Agenda item:	6.12.3

Document for:	Discussion

# Introduction

In RAN#116 meeting, companies discussed on 3-antenna-port codebook-based transmissions for BS demodulation requirements. In this paper, we present our view on the open issues, along with UE demodulation requirements.

# BS Demodulation requirements

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

The current BS demodulation requirements cover UEs with 1, 2, and 4 Tx antennas. Looking ahead, as hardware and design technologies advance, the introduction of UEs with 3 Tx antennas is a plausible next step, as indicated in WID [1]. To support this evolution, demodulation requirements for 3-port PUSCH with 3 transmission layers must be defined; otherwise, there will be no performance criteria to test them. Furthermore, as per the RAN1 agreement [2], a new precoder was adopted at the RAN116 meeting, and RAN4 should establish corresponding performance requirements for its testing.

Observation 1: If BS demodulation does not create a requirement for 3Tx UE, there is no performance requirement to test it and the new precoder agreed in RAN1.

Proposal 1: RAN4 should specify PUSCH demodulation requirements for 3-antenna-port non-coherent codebook-based UL transmissions.

2.1 Discussion

In the last meeting, companies agreed on most of the simulation assumptions. The general agreement was to follow the same assumptions as 4Tx requirements as a reference. The open issue is on the channel bandwidth selection per sub-carrier spacing as follows:

Issue 2-2-3: SCS and CHBW for FDD

-	15kHz SCS, 5MHz, 20MHz, 50MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

Issue 2-2-4: SCS and CHBW for TDD

-	30kHz SCS, 10MHz, 40MHz, 100MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

Based on our simulation results in [3], we have the following observations:

Observation 1: No performance difference was observed between 5, 20 and 50MHz channel bandwidth (CBW) for 15KHz sub-carrier spacing (SCS). For 30KHz SCS, we expect similar behavior for different CBWs.

Observation 2: Considering the test coverage and test effort, we prefer to define performance requirement for minimum and one additional CBW (preferably, the maximum one) per SCS.

Proposal 1: Define 3Tx BS demodulation performance requirement as follows:

- 15KHz SCS with 5 and 50MHz CBW

- 30KHz SCS with 10 and 100MHz CBW

# UE Demodulation requirements

3.1	CSI reporting requirements

3.1.1	Refined Type I Single-Panel Codebook (typeI-SinglePanel-r19)

RAN4#116 agreed to define PMI reporting requirements with the refined Type I Single-Panel codebook, but RAN4 has not concluded the detailed simulation parameters. Table 1 is the summary of test parameters for TypeI reporting requirements. Note RAN4 has not concluded whether to define TypeI PMI reporting requirements with 128 ports.

Table 1	Configurations of PMI reporting requirements with Rel-19 refined Type-I single panel codebook.

| Test case | CSI-RS ports (N1, N2) | Port mapping method (portMappingMethod) | Codebook mode (typeI-codebookMode-r19) | CMR configuration (K and nrofPorts per CSI-RS resource) | Number of slots for CSI-RS transmission | CSI-RS configuration and density |
| --- | --- | --- | --- | --- | --- | --- |
| 64 ports | 64 (8, 4) | Method 1 | Mode A | K=2 32 CSI-RS ports | One slot | Row: 17 Density: 1 |
| 128 ports (Not decided yet) | 128 (16, 4) | Method 1 | Mode A | K=4 32 CSI-RS ports | One slot | Row: 18 Density: 0.5 |

Scheduling

Issue 1-2-6: NZP-CSI-RS configurations for 64 CSI-RS ports NZP CSI-RS; CSI-RS resource 1 and 2; CSI-RS resource Type; Aperiodic. Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 17,(2, 4, 6, 8). CSI-RS resource 1; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (1, 7). CSI-RS resource 2; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (5, 12). CSI-RS resource 1 and 2; CSI-RS interval and offset; slot; Not configured. aperiodicTriggeringOffset; 0. Issue 1-2-8: CQI/RI/PMI delay for FDD 64CSI-RS ports - Apply slot n+4, PMI delay (from the PMI estimation slot to the PMI apply slot) 11ms, Report slot offset = [7], for FDD SCS=15kHz - Interested companies are welcomed to provide results when the reported PMI is applied at slot n+7 Issue 1-2-9: CQI/RI/PMI delay for TDD 64CSI-RS ports - Apply slot n+6, PMI delay (from the PMI estimation slot to the PMI apply slot) 11.5ms, Report slot offset = 17, for TDD SCS=30kHz

RAN4#116 agreed to configure the scheduling based on CSI reporting Capability 2. Table 2 summarizes the CSI computation time according to TS 38.214 5.4 for both 64Tx ports and 128 Tx ports. According to the agreement in RAN4#116, the delays to apply the reported PMI are same as Rel-15 32 Tx requirements.

Table 2	CSI computation time and timing where TE can apply the reported PMI.

|  | UE CSI computation time (64 ports Capability 2) | UE CSI computation time (128 ports Capability 2) | Slot number that TE applies PMI at slot n |
| --- | --- | --- | --- |
| FDD SCS=15kHz | 6 slots | 12 slots | n+4 |
| TDD SCS=30kHz | 11 slots | 21 slots | n+6 |

For FDD SCS=15kHz, since UL slots are always available, we can confirm the agreement on RAN4#116, where CSI report slot offset and PMI delay are 7 slots and 11ms, respectively.

For TDD SCS=30kHz with 64 Tx ports, on the other hand, we propose that TE schedules AP-CSI-RS in slot 6 to reduce the delay from CSI request to the PDSCH scheduling. With this configuration, UE reports PMI in slot 18, TE schedules PDSCH from slot 24, and then PMI delay can be set to 18 slots (=9ms). Figure 1 illustrates the scheduling. This means the CSI report slots offset is 11.

Figure 1	Proposed PMI test scheduling for TDD SCS=30kHz with 64 ports.

For TDD SCS=30kHz with 128 Tx ports with, if introduced, we propose that TE schedules AP-CSI-RS in slot 6, UE reports PMI in slot 28, and then TE schedules PDSCH from slot 34. With this configuration, the PMI delay can be set to 28 slots (=14ms). Figure 2 illustrates the scheduling. This means the CSI report slots offset is 21.

Figure 2	Proposed PMI test scheduling for TDD SCS=30kHz with 128 ports (if introduced).

Proposal 2: For TypeI PMI reporting requirements with 64 CSI-RS ports with FDD SCS=15kHz, configure CSI report slot offset and PMI delay as 7 slots and 11ms, respectively.

Proposal 3: For TypeI PMI reporting requirements with 64 CSI-RS ports with TDD SCS=30kHz, configure CSI report slot offset and PMI delay as 11 slots and 9ms, respectively.

Proposal 4: For TypeI PMI reporting requirements with 128 CSI-RS ports with TDD SCS=30kHz, if introduced, configure CSI report slot offset and PMI delay as 21 slots and 14ms, respectively.

RAN4#116 also discussed the NPZ-CSI-RS configuration. For 64 Tx port scenario, it is assumed one CSI-RS is configured with (l0, l1) = (5, 12), which is same as Rel-15 32 ports. On top of that, the second CSI-RS is added with (l0, l1) = (1, 7). This configuration does not conflict with ZP-CSI-RS where l0=8 and k0=4. However, if CSI-RS for CSI acquisition is scheduled in the same slot as TRS, the second CSI-RS resource is overlapped with TRS as illustrated in Figure 3. Therefore, we should avoid scheduling CSI-RS for CSI acquisition in the slot where TRS is transmitted.

Figure 3	RE mapping of NZP-CSI-RS, ZP-CSI-RS and TRS.

Proposal 5: For TypeI PMI reporting requirements with 64/128 CSI-RS ports, RAN4 should avoid scheduling CSI-RS for CSI acquisition in the same slot as TRS scheduling.

64 CSI-RS ports

For 64 CSI-RS ports scenario, one open issue is the MCS selection. Figure 4 and Figure 5 show our simulation results for 64 CSI-RS ports with FDD and TDD, respectively, according to the scheduling discussed above.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 4: Comparison of MCS20 and MCS22 for 64 CSI-RS ports (FDD).

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 5: Comparison of MCS20 and MCS22 for 64 CSI-RS ports (TDD).

Table 3 summarizes the simulation results, where we show Gamma, which is throughput ratio between follow PMI and random PMI where SNR corresponds to 90% of the maximum throughput with follow PMI. We also show another throughput ratio where random PMI test point is the SNR corresponds the 90% of the maximum throughput with follow PMI with the offset of 5dB. We call it ‘Gamma-5dB’ on the table.

Table 3	Summary of Rel-19 TypeI PMI 64 CSI-RS ports reporting simulation results.

| FDD/TDD | UE Rx | MCS | SNR to achieve 90% of the peak with follow PMI | Throughput ratio between follow PMI and random TypeI PMI (Gamma) | Throughput ratio with SNR offset of 5dB for random TypeI PMI (Gamma-5dB) |
| --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 20 | 6.4 | 16.2 | 5.1 |
|  |  | 22 | 9.6 | 11.7 | 4.3 |
|  | 4Rx | 20 | 0.3 | 40.0 | 7.6 |
|  |  | 22 | 2.7 | 30.7 | 6.7 |
| TDD | 2Rx | 20 | 6.5 | 11.6 | 4.2 |
|  |  | 22 | 9.6 | 8.3 | 3.5 |
|  | 4Rx | 20 | 0.0 | 22.6 | 6.5 |
|  |  | 22 | 2.1 | 18.6 | 5.7 |

From the simulation results, Gamma is generally very large, especially for 4Rx. We don’t think larger MCS decrease the required throughput ratio. As we discussed in RAN4#116, the main reason is of larger Gamma is due to the poor random PMI performance. If we measure the throughput of random PMI with higher SNR test points (e.g., SNRUE+5dB), the throughput ratio can be decreased as expected. We therefore propose configuring MCS20 and measure the random PMI by adding XdB to SNR test point, where X=5dB for example.

Proposal 6: For TypeI PMI reporting requirements with 64 CSI-RS ports, configure MCS20.

Proposal 7: For TypeI PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding. For example, X=5dB.

128 CSI-RS ports

Figure 6 show our simulation results for 128 CSI-RS ports with TDD SCS=30kHz according to the scheduling discussed above.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 6: Comparison of MCS20 and MCS22 for 128 CSI-RS ports (TDD).

Table 4	Summary of Rel-19 TypeI PMI 128 CSI-RS ports reporting simulation results.

| FDD/TDD | UE Rx | MCS | SNR to achieve 90% of the peak with follow PMI | Throughput ratio between follow PMI and random TypeI PMI (Gamma) | Throughput ratio with SNR offset of 5dB for random TypeI PMI (Gamma-5dB) |
| --- | --- | --- | --- | --- | --- |
| TDD | 2Rx | 20 | 3.6 | 28.1 | 9.1 |
|  |  | 22 | 6.8 | 20.0 | 7.0 |
|  | 4Rx | 20 | -2.6 | 57.3 | 14.2 |
|  |  | 22 | -0.4 | 44.2 | 12.1 |

Table 4 also summarized the simulation results. Same as 64 CSI-RS ports, RAN4 should consider adding offset to SNR when TE measures the random PMI throughput, because the throughput ratio is very high.

Proposal 8: Define TypeI PMI reporting requirements with 128 CSI-RS ports for TDD SCS=30kHz.

Proposal 9: For TypeI PMI reporting requirements with 128 CSI-RS ports, configure MCS20.

Proposal 10: For TypeI PMI reporting requirements with 128 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding. For example, X=5dB.

3.1.2	Refined eType II Codebook (eTypeII-r19)

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19 - Interested companies are welcomed to provide simulation results for next meeting

RAN4 has not concluded whether to define the PMI reporting requirements Rel-19 eTypeII. We show our initial simulation results for the progress of the discussion. Figure 7 and Figure 8 show our simulation results of Rel-19 eTypeII with 64 Tx ports for FDD 15kHz and TDD 30kHz, respectively. Table 5 also summarizes the simulation results of Rel-19 eTypeII 64 ports.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 7: Rel-19 eTypeII 64 CSI-RS ports simulation results (FDD).

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 8: Rel-19 eTypeII 64 CSI-RS ports simulation results (TDD).

Table 5	Summary of Rel-19 eTypeII PMI 64 CSI-RS ports reporting simulation results.

| FDD/TDD | UE Rx | MCS | SNR to achieve 90% of the peak with follow PMI | Throughput ratio between follow PMI and random TypeI PMI (Gamma) | Throughput ratio with SNR offset of 5dB for random TypeI PMI (Gamma’) |
| --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 20 | 1.1 | 156.2 | 17.1 |
|  |  | 22 | 3.6 | 111.3 | 15.6 |
|  | 4Rx | 20 | -2.0 | 141.6 | 14.2 |
|  |  | 22 | 0.1 | 103.7 | 12.0 |
| TDD | 2Rx | 20 | 4.7 | 17.8 | 5.5 |
|  |  | 22 | 7.4 | 13.6 | 4.9 |
|  | 4Rx | 20 | -0.3 | 24.1 | 6.9 |
|  |  | 22 | 1.7 | 20.1 | 6.1 |

Same as Rel-19 TypeI simulation results, RAN4 should consider adding offset to SNR when TE measures the random PMI throughput, because the throughput ratio becomes very high.

Since RAN4 has defined PMI reporting requirements with Rel-15 TypeII and Rel-16 eTypeII, we propose to define the PMI reporting requirements for Rel-19 enhancements.

Proposal 11: Define eTypeII PMI reporting requirements with 64 CSI-RS ports for both FDD SCS=15kHz and TDD SCS=30kHz.

Proposal 12: For eTypeII PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding with Rel-19 Type I codebook. For example, X=5dB.

3.1.3	Refined eType II Codebook for predicted PMI (typeII-Doppler-r19)

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19

RAN4 has not concluded whether to define the PMI reporting requirements ‘typeII-Doppler-r19’. In our understanding, the new codebook ‘typeII-Doppler-r19’ is a simple extension of 'typeII-Doppler-r18' to support more than 32 ports. If RAN4 define PMI reporting requirements with eTypeII-r19 with 64 ports, we may not need to define typeII-Doppler-r19 additionally.

Proposal 13: RAN4 does not define PMI reporting requirements for typeII-Doppler-r19.

2.1.4	CSI reporting requirements for CJT calibration

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT Proposal: For new CSI reporting for time/frequency/phase offsets: - Option 1: RAN4 should not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test. For PMI reporting: - Option 1: Further discussion on introducing new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation. Way forward: - Encourage supporters for introducing CJT related CSI reporting test requirements to provide testable test setup.

In RAN4#116, RRM session has agreed with the measurement period of CJT calibration reporting [4], and they will define the delay offset and frequency offset accuracy requirements in the WI performance part. As we discussed in RAN4#116, we think it is sufficient to verify the UE performance on CJT calibration reporting. RAN4 does not need to define new CSI reporting requirements for CJT calibration.

Proposal 14: RAN4 does not define CSI reporting requirements for CJT calibration reporting.

# Conclusion

4.1	BS Demodulation Requirements

Based on the above discussion following observations are captured:

Observation 1: No performance difference was observed between 5, 20 and 50MHz channel bandwidth (CBW) for 15KHz sub-carrier spacing (SCS). For 30KHz SCS, we expect similar behavior for different CBWs.

Observation 2: Considering the test coverage and test effort, we prefer to define performance requirement for minimum and one additional CBW (preferably, the maximum one) per SCS.

Based on the above discussion following proposal is captured:

Proposal 1: Define 3Tx BS demodulation performance requirement as follows:

- 15KHz SCS with 5 and 50MHz CBW

- 30KHz SCS with 10 and 100MHz CBW

4.2	UE Demodulation Requirements

Proposal 2: For TypeI PMI reporting requirements with 64 CSI-RS ports with FDD SCS=15kHz, configure CSI report slot offset and PMI delay as 7 slots and 11ms, respectively.

Proposal 3: For TypeI PMI reporting requirements with 64 CSI-RS ports with TDD SCS=30kHz, configure CSI report slot offset and PMI delay as 11 slots and 9ms, respectively.

Proposal 4: For TypeI PMI reporting requirements with 128 CSI-RS ports with TDD SCS=30kHz, if introduced, configure CSI report slot offset and PMI delay as 21 slots and 14ms, respectively.

Proposal 5: For TypeI PMI reporting requirements with 64/128 CSI-RS ports, RAN4 should avoid scheduling CSI-RS for CSI acquisition in the same slot as TRS scheduling.

Proposal 6: For TypeI PMI reporting requirements with 64 CSI-RS ports, configure MCS20.

Proposal 7: For TypeI PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding. For example, X=5dB.

Proposal 8: Define TypeI PMI reporting requirements with 128 CSI-RS ports for TDD SCS=30kHz.

Proposal 9: For TypeI PMI reporting requirements with 128 CSI-RS ports, configure MCS20.

Proposal 10: For TypeI PMI reporting requirements with 128 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding. For example, X=5dB.

Proposal 11: Define eTypeII PMI reporting requirements with 64 CSI-RS ports for both FDD SCS=15kHz and TDD SCS=30kHz.

Proposal 12: For eTypeII PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding with Rel-19 Type I codebook. For example, X=5dB.

Proposal 13: RAN4 does not define PMI reporting requirements for typeII-Doppler-r19.

Proposal 14: RAN4 does not define CSI reporting requirements for CJT calibration reporting.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung, RAN#105, September 2024, Melbourne, Australia.

- R1-2501746, “FL Summary Support for 3TX CB-based Uplink; First Round”, April 2025.

[3]  R4-2514016, “Simulation results for FR1 3Tx demodulation requirements”, Ericsson,  RAN4116-bis, October 2025.

[4]	R4-2512131, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.