3GPP TSG-RAN WG4 Meeting #116bis	R4-2514058

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for Ambient-IoT

Agenda Item:	6.14.3

Document for:	Discussion

# Introduction

This contribution outlines our point of view on the performance requirements for Ambient-IoT based on the agreements in last meetings.

# Discussion

Issue 2-1-1: Test scenario

Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

Issue 2-1-2: Test configuration

- Proposals from companies: - Option 1 (CATT): - RAN4 to discuss the following test parameters as a starting point: - CW transmitting power: [33] dBm. - CW2D distance: [5, 10, 20] m. - R2D SCS = 15 kHz only. - R2D transmission bandwidth configuration: [1 PRB, 2 PRB, 3 PRB, 4 PRB]. - Option 2 (OPPO): - Set X=1 for MSG 1 transmission in the test case. - Consider the multiple Toffset1 values as different configuration in the test case: - Configuration 1: Toffset1 = 1333.4us, reference D2R chip length is 133.33us - Configuration 2: Toffset1 = 666.6us, reference D2R chip length is 33.33us - Configuration 3: Toffset1 = 132.32us, reference D2R chip length is 4.17us - Configuration 4: Toffset1 = 33.33us, reference D2R chip length is 2.08us, R2D chip length is 11.11us - Option 3 (Xiaomi): - The common testing configuration for Rel19 AIoT RRM test cases can be defined as: - SCS = 15KHz - BW =[180kHz] - Device 1 only - Option 4 (CMCC): - For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

Some proposals in the demodulation session indicate that the RF requirements for A-IoT Device 1 are to be performed OTA. For the same reason, it is expected that RRM should also take this into consideration.

Proposal 1: RRM tests, if agreed, are performed in OTA.

Although test cases may combine RRM and RF/demod tests, the primary test objectives differ: RRM test targets are distinct from test RF/demod targets, and the relevant side conditions vary across RRM and RF/demod tests.

Proposal 2: RAN4 to define dedicated RRM tests.

With respect to core requirements, it is expected to define 2 test cases accordingly:

- Random Access, including contention based random access test and non-contention based random access test.

- D2R transmit timing, to verify accuracy of the TSFO  from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Proposal 4 in Issue 2-1-2 points out a possibility of combination of random access and D2R transmit timing in one test.

Proposal 3: Support Option 4:

- For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

# Conclusion

Proposal 1: RRM tests, if agreed, are performed in OTA.

Proposal 2: RAN4 to define dedicated RRM tests.

Proposal 3: Support Option 4:

- For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

# References

- R4-2512134, WF on RRM requirements for Ambient_IoT_Solutions, CMCC