3GPP TSG RAN WG4 Meeting #116-bis	R4-2514078

Prague, Czech, 13 – 17 October, 2025

Agenda item:			4.23.1

Source:	vivo

Title:	On remaining issues for mobility enhancement ph4

Document for:	Discussion

# Introduction

In the RAN4#116 meeting, RAN4 completed RRM core requirements for Rel-19 mobility enhancement. One remaining open issue was captured in the WF [1].

#### Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category Agreement: - No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category. - The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

In this contribution, we further provide our views on RRM requirements for event-triggered L1-RSRP reporting in R19 LTM.

# Discussion

For L3 measurement, capabilities for event triggering and reporting are defined as below.

#### 9.1.4.1 Introduction This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 9.1.4.2, the UE shall meet all other performance requirements defined in clause 9 and clause 10. The requirements in this clause also apply for a UE in EN-DC with PSCell on a carrier frequency with CCA or SA NR with PCell on a carrier frequency with CCA. The UE can be requested to make measurements under different measurement identities defined in TS 38.331 [2]. Each measurement identity corresponds to either event-based reporting, periodic reporting, or no reporting. In case of event-based reporting, each measurement identity is associated with an event triggering criterion. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion. The purpose of this clause is to set some limits on the number of different event triggering, periodic, and no reporting criteria the UE may be requested to track in parallel. #### 9.1.4.2 Requirements In this clause a reporting criterion corresponds to either one event (in the case of event-based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1..

The above requirements count criteria for event-based reporting, periodic reporting and no reporting. All the reporting is related to measurement identities.

Firstly, event triggering and measurement reporting for L1 measurement are introduced. Similar to that for L3 measurement reporting, it is better to define corresponding capabilities so that NW and UE can understand each other how many events triggering and reporting can be configured and supported. For L1 measurement, there is no measurement identities. Thus, it is better to define a new separated UE capability for event triggering and reporting for LTM.

For LTM, following events were introduced.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For the number of frequency layers for SSB-based L1-RSRP measurement, it is indicated by UE capabilities supportedMaxIntraInterFreqLayersWithoutGaps-r18 and supportedMaxInterFreqLayersWithGaps-r18. It is differentiated by measurement with gap and without gap, rather than intra frequency layer and inter frequency layer.

For CSI-RS based L1 measurement for event triggered reporting, only intra-frequency measurement without gap is supported. One frequency layer is enough to define reporting criteria requirements.

Considering four events triggered reporting for LTM and periodic reporting, and maximum number of 8 frequency layers for SSB-based measurement and 1 frequency layer for CSI-RS based measurement, the total number of reporting criteria for LTM can be defined as 45 and number 48 could be considered taking into some margin into account. This should be enough even when different thresholds used for an event as 8 frequency layers would rarely be used in practical network.

If it is not possible to conclude the number for reporting criteria requirements, it is proposed to add a sentence in the spec to state UE should perform all the event reporting and periodic reporting as configured by network.

Proposal 1: RAN4 to decide following alternatives to define reporting criteria requirements for LTM.

Alt 1: Reporting criteria for LTM event triggered reporting is 48 in total for all configured L1 measurements for LTM.

Alt 2: It is clarified in the spec that UE should perform all the event reporting and periodic reporting as configured by network.

# Summary

In this contribution, we further provided views on remaining open issues for event triggered reporting for LTM. Following proposals are provided.

Proposal 1: RAN4 to decide following alternatives to define reporting criteria requirements for LTM.

Alt 1: Reporting criteria for LTM event triggered reporting is 48 in total for all configured L1 measurements for LTM.

Alt 2: It is clarified in the spec that UE should perform all the event reporting and periodic reporting as configured by network.

- References

- R4-2512141	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple