3GPP TSG-RAN WG4 Meeting #116bis	R4-2514085

Prague, Czech, 13th – 17th October, 2025

Agenda item:		6.13.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116 [1] regarding evolution of NR duplex operation. TX core requirements for Rel-19 are complete and agreed. Next step is to implement the agreements to the performance part of the specification. Below is Nokia’s view of the existing BS RF TX requirements for SBFD operation and the conformance testing aspects in general.

# Discussion

## Manufacturer declarations

Manufacturer declarations are defined in TS 38.141-1 and TS 38.141-2. It is inevitable that SBFD-capable BS related manufacturer declarations need to be updated in TS 38.141-1 and TS 38.141-2. Clause 4.6 must be updated with SBFD-capable BS information.

We think it might be beneficial to add a column to table 4.6-1 to include SBFD-capable BS type 1-H to TS 38.141-1. BS type 1-C is out of scope for SBFD operation. Similarly, the modification mentioned above is also needed for SBFD-capable BS type 1-H, 1-O and 2-O to TS 38.141-2. This modification would ensure that the existing declaration and the description is kept for legacy BS. It must be noted that SBFD-capable BS must comply the existing legacy requirements for non-SBFD DL and UL slots so declaration for those is also needed.

New declarations for SBFD-capable BS are needed and, therefore, new declaration identifiers are needed. Sub-band configurations (UL/DL configuration together with GB) are the most important to be declared.

Consider to add a column to table 4.6-1 to include SBFD-capable BS manufacturer declarations.

New manufacturer declarations and declaration identifiers are needed in table 4.6-1.

## RF channels

We propose to follow NR channel placement as much as possible. Different requirements need to have test signals placed either at the bottom or the top of the subbands. Careful consideration is needed for different requirements and different subband configurations.

Depending on the transmitter requirement, DL resource block (for DU or UD) placement within SBFD have to be configured such that for:

- Out-of-band cases, DL RBs are placed in lower edge of channel bandwidth for B (Bottom channel)

- Out-of-band cases, DL RBs are placed in higher edge of channel bandwidth for T (Top channel)

- In-band cases, DL RBs are placed in lower edge of channel bandwidth for T (Top channel)

- In-band cases, DL RBs is placed in higher edge of channel bandwidth for B (Bottom channel)

Depending on the receiver requirement, UL resource block (for DU or UD) placement within SBFD have to be configured such that for:

- Out-of-band cases, UL RBs are placed in lower edge of channel bandwidth for B (Bottom channel)

- Out-of-band cases, UL RBs are placed in higher edge of channel bandwidth for T (Top channel)

- In-band cases, UL RBs are placed in lower edge of channel bandwidth for T (Top channel)

- In-band cases, UL RBs is placed in higher edge of channel bandwidth for B (Bottom channel)

For transmitter requirements, if SBFD-capable BS supports DUD, then DUD configuration may be considered to be the worst case and it would be sufficient to test only DUD. For receiver requirements, all requirements need to be checked individually to find out what configuration is considered the worst case.

Careful consideration is needed for different requirements and subband different subband configurations.

## Test model and test signal

Regarding the scalability of the test model for SBFD DL subband, we find it will be very difficult to achieve such test model due to complexity. It must be noted that a new test model will require not only DL subband PRBs but also UL subband PRBs that are needed for the SBFD slots together with the guard bands that may vary. To even make it possible to have a scalable test model, the test model would need to be based on equation matrix. Otherwise, the FRC tables will explode in size and amount. It would make sense to minimize the test model use for different test cases. Test model definition is linked to test signal definition as described in 4.7.2-1 in TS 38.141-1. Careful consideration is needed to make sure the worst case is covered.

Scalable test model might not be feasible due to the complexity of the test model.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our observations are listed below.

- Consider to add a column to table 4.6-1 to include SBFD-capable BS manufacturer declarations.

Careful consideration is needed for different requirements and subband different subband configurations.

Scalable test model might not be feasible due to the complexity of the test model .

- New manufacturer declarations and declaration identifiers are needed in table 4.6-1.

# References

- R4-2512680 “Way Forward for [116][307] NR_duplex_evo_BSRF”, RAN4#116, Bengaluru, India, 25th-29th August, 2025