3GPP TSG-RAN WG4 Meeting #116bis	R4-2514089

Prague, Czech, 13th – 17th October, 2025

Agenda item:		7.2.2

Source:	Nokia

Title:	On NR BS RF requirement evolution phase 2 general aspects and work plan

Document for:		Approval

# Introduction

A new Rel-20 WID [1] was agreed in TSG RAN#109 regarding NR base station RF requirement evolution for FR1 and testing phase 2. There are two topics introduced in this WID:

- Topic 1: Enhancement of co-location requirements.

- Topic 2: SBFD BS to SBFD BS adjacent channel coexistence

In this contribution we discuss the general aspects and the work plan for these two topics.

# Discussion

## Enhancement of co-location requirements

The following justification is given in the WID:

The upper bands in NR FR1 are becoming more important. A number of issues for BS type 1-O requirements were discussed in the previous releases. To make BS type 1-O more relevant for AAS BS operating in the upper bands of FR1, RAN4 need to investigate and enhance the co-location requirements.

Also, the following objectives are given in the WID:

Enhancement of co-location requirements - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any.

It is important for this enhancement of co-location requirements topic to consider all co-location requirements listed above. Listed requirements need to be carefully evaluated and considered individually. Regarding the two options listed to improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies, we feel that evaluating both is very important.

Our proposal for the work plan is depicted in the following table:

| #RAN4 meeting | Work plan |
| --- | --- |
| 116-bis | - Start discussion on the core part - Agree on the work plan |
| 117 | - Continue discussions on the core part - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 - Start discussion on the two options mentioned in WID |
| 118 | - Continue discussions on core part - Provide more simulation/measurement results for BS-to-BS isolation and start to evaluate effect on co-location requirements - Continue discussion on the two options mentioned in WID |
| 118-bis | - Continue discussions on core part - Continue to provide more simulation/measurement results for BS-to-BS isolation and continue to evaluate effect on co-location requirements - Continue discussion on the two options mentioned in WID |
| 119 | - Continue discussions on core part - Continue to provide more simulation/measurement results for BS-to-BS isolation and and continue to evaluate effect on co-location requirements - Start discussion on initial text proposals for TR 37.941 - Continue discussion on the two options mentioned in WID |
| 120 | - Continue discussions on core part - Provide more simulation/measurement results for BS-to-BS isolation and continue discussion on initial text proposals for TR 37.941 - Continue discussion on the two options mentioned in WID and start discussion on initial text proposals for TR 37.941 - Initiate discussion on core specification draftCRs |
| 120-bis | - Continue discussions on core part - Finish collecting simulation/measurement results for BS-to-BS isolation and continue discussion on text proposals for TR 37.941 - Continue discussion on the two options mentioned in WID and continue discussion on text proposals for TR 37.941 - Continue discussion on core specification draftCRs |
| 121 | - Continue discussions on core part - Conclude discussion on the two options mentioned in WID and conclude discussion on text proposals for TR 37.941 - Continue discussion on core specification draftCRs |
| 122 | - Conclude discussion on the core part and agree CRs to core specification. - Start discussion on the performance part |
| 122-bis | - Continue to discuss the performance part - Start to discuss initial text proposals and initiate discussion on draftsCRs |
| 123 | - Continue to discuss the performance part - Continue to discuss text proposals and update draftsCRs |
| 124 | - Conclude discussion on the performance part and agree CRs to test specifications |

RAN4 to agree on the proposed work plan for enhancement of co-location requirements.

## SBFD BS to SBFD BS adjacent channel co-existence

The following justification is given in the WID:

SBFD (Subband Full Duplex) is anevolution technique of TDD in the 5G-A. In Rel-19 TDD BS to SBFD BS co-existence study was conducted. And it was agreed that RAN4 do not introduce the specific requirements for SBFD BS to SBFD BS co-existence for FR1 WA or MR BS in Rel-19 and SBFD BS to SBFD BS co-existence case can be considered for Rel-20. Compared to TDD BS to SBFD BS co-existence case, ACIR in SBFD BS to SBFD BS co-existence case could be improved and better co-existence performance might be achieved, because of the nature of SBFD design where the SBFD downlink of operator B can be adopted as guard band for filter implementation to protect the SBFD uplink of adjacent operator A. Therefore, in Rel-20 it is proposed to specify BS TX and RX RF requirement for SBFD-capable BS for the scenario of SBFD BS to SBFD BS adjacent channel coexistence.

Also, the following objectives are given in the WID:

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs.

Most importantly, the work load with this topic should be tried to kept as limited as possible. It was clearly visible in Rel-19 work regarding SBFD, that the widened scope and increased flexibility in subband configurations led to a world of trouble when trying to introduce the core requirements for SBFD. As the TU for this work item is limited, it should be everyone’s priority to keep the scope as limited as possible.

Our proposal for the work plan is depicted in the following table:

| #RAN meeting | Work plan |
| --- | --- |
| 116-bis | - Agree on the work plan - Start discussion on the core part - Start discussion on simulation assumptions |
| 117 | - Continue discussion on the core part - Agree simulations assumptions |
| 118 | - Continue discussion on the core part - Provide and collect simulations results |
| 118-bis | - Continue discussion on the core part - Provide and collect simulations results |
| 119 | - Continue discussion on the core part - Provide and collect simulations results |
| 120 | - Discuss BS TX and RX RF requirement for SBFD-capable BS based on simulation results |
| 120-bis | - Discuss BS TX and RX RF requirement for SBFD-capable BS based on simulation results |
| 121 | - Discuss BS TX and RX RF requirement for SBFD-capable BS based on simulation results - Discuss draft CRs for core part. |
| 122 | - Start performance part - Conclude on core requirements - Approve CRs for core specifications for BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence |
| 122-bis | - Continue discussion on the performance part - Discuss conformance specifications |
| 123 | - Continue discussion on the performance part - Discuss draft CRs for BS conformance test specifications |
| 124 | - Approve CRs for BS conformance test specifications |

RAN4 to agree on the proposed work plan for SBFD BS to SBFD BS adjacent channel co-existence.

# Conclusion

This document shares Nokia’s views on NR BS RF requirement evolution phase 2 general aspects and work plan. Following observations and proposals were introduced:

- RAN4 to agree on the proposed work plan for enhancement of co-location requirements.

RAN4 to agree on proposed work plan for SBFD BS to SBFD BS adjacent channel co-existence.

# References

- RP-252905 “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, RAN#109, Beijing, China, 15th-18th September, 2025.