# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514103

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements on two-sided models

Agenda item:	7.9.4

Document for:	Approval

Introduction

At RAN plenary #108 meeting, the latest WI“Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”[1] was approved. In current WID, the goal is to define RRM requirements to specify LCM procedures. The following description states RAN4 RRM work in future stage:

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this meeting, the initial views will be provided from our side.

Discussion

RAN4 #116bis meeting will kick off the study of RRM requirements on LCM procedure for AI CSI compression especially two-sided model. In Rel-19 discussion, RAN4 has already studied several LCM requirements including prediction delay, activation/deactivation delay etc. To our understanding, the discussion of LCM in Rel-19 could be the baseline for our analysis.

Observation 1: In Rel-19 discussion, RAN4 has already studied several LCM requirements including prediction delay, activation/deactivation delay etc.

Proposal 1: The LCM discussion of Rel-19 shall be the baseline for Rel-20 LCM discussion on two-sided CSI compression.

  - For beam management, positioning accuracy enhancement and CSI prediction, prediction reporting delay is still under discussion, but the concrete components have been defined as below:

For beam management: Agreement - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion For positioning accuracy enhancement: Agreement Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report For CSI prediction: Agreement wait for RAN1 decision

  - For beam management, prediction delays contains three parts including measurement delay, inference delay and reporting delay. For aperiodic report, the prediction reporting delay includes measurement delay +reporting delay, and the reporting delay includes the inference delay. For semi-persistent and periodic report, the prediction reporting delay includes measurement delay +inference delay+time for the first available reporting occasion.

  - For positioning accuracy enhancement, reporting delay includes measurement delay, inference delay and the time needed until the UE send the report. The concrete measurement samples will be further discussed.

  - For CSI prediction, the CSI reporting delay shall wait for RAN1 discussion and further to define.

  - In view of the above, reporting delay shall be considered for CSI compression.

  - Observation 2: For beam management, positioning accuracy enhancement and CSI prediction, prediction reporting delay was considered.

  - Proposal 2: RAN4 shall firstly consider prediction reporting delay for CSI compression with two-sided model.

  - During legacy discussion, the context of LCM procedure has been defined as below:

Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

  - It is observed that switching, fallback to non-AI and activation/deactivation are considered. We deem that these LCM requirements shall be studied. However, several agreements could be seen as below:

Agreement: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. Agreement: - No need to create deactivation requirements for AI functionality Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited.

  - For switching delay and fallback delay, RAN4 has agreed that deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirements, including both AI/non-AI discussion. Consequently, switching delay and fallback delay shall also be deprioritized in current stage.

  - Observation 3: RAN4 has agreed that deprioritize the discussion on switching and fallback delay requirements

  - Proposal 3: Switching delay and fallback delay shall be deprioritized in initial stage for studying AI CSI compression with two-sided model.

  - On the contrary, RAN4 focused on activation delay and deactivation delay. Based on RAN1 and RAN2 outcomes, AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference. Functionality deactivation refers to the process of network deactivating an active functionality. It is noted that RAN4 agreed that no need to create deactivation requirements for AI functionality.

  - Observation 4: It is noted that RAN4 agreed that no need to create deactivation requirements for AI functionality.

  - Proposal 4: For CSI compression with two-sided model, RAN4 can firstly focus on activation delay based on further RAN2 design.

  - For performance monitoring, RAN4 deemed that performance monitoring delay requirements for LCM purpose shall not be defined. However, it says if the monitoring delay is identificated as critical in the future, this issue can be revisited. Hence, we shall not preclude performance monitoring instantly for AI CSI compression with two-sided model.

  - Proposal 5: For CSI compression with two-sided model, performance monitoring requirements shall not be precluded.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Observation 1: In Rel-19 discussion, RAN4 has already studied several LCM requirements including prediction delay, activation/deactivation delay etc.

Proposal 1: The LCM discussion of Rel-19 shall be the baseline for Rel-20 LCM discussion on two-sided CSI compression.

  - Observation 2: For beam management, positioning accuracy enhancement and CSI prediction, prediction reporting delay was considered.

  - Proposal 2: RAN4 shall firstly consider prediction reporting delay for CSI compression with two-sided model.

  - Observation 3: RAN4 has agreed that deprioritize the discussion on switching and fallback delay requirements

  - Proposal 3: Switching delay and fallback delay shall be deprioritized in initial stage for studying AI CSI compression with two-sided model.

  - Observation 4: It is noted that RAN4 agreed that no need to create deactivation requirements for AI functionality.

  - Proposal 4: For CSI compression with two-sided model, RAN4 can firstly focus on activation delay based on further RAN2 design.

  - Proposal 5: For CSI compression with two-sided model, performance monitoring requirements shall not be precluded.

- References

- RP-251870, WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements.