3GPP TSG RAN4 Meeting #116bis					    	         R4-2514135

Prague, The Czech Republic, Oct 13-17, 2025

Agenda item:	7.7.4

Source:	Huawei, HiSilicon

Title:	Discussion on UAV RF requirements

Document for:	Discussion

Background

The new WID for enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) was approved at RAN#109 [1]. The objectives were agreed as below:

The core part of WI:

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

This document provides our initial thinking on the RF requirements aspect.

Discussion

The objective of Rel-20 UAV WI is to introduce the requirements for up to 33dBm max total conductive output power. RAN4 needs to decide how to capture the new requirements. One option is to follow the ATG approach, i.e. allow UE to declare the rated maximum output power up to 33dBm. With this approach, there is no need to specify a set of common MPR/AMPR requirements for all the UAV UEs. It depends on UE implementation to meet the emission requirements.

Proposal 1: The maximum output power of up to 33dBm is specified as UE declaration.

For PC3 UAV UE, the ACLR requirements refer to TN UE requirements in 6.5. For the high power UAV UE, the ACLR requirements should follow the output of co-existence evaluation.

Proposal 2: The ACLR requirements for high power UAV UE should follow the output of co-existence evaluation.

For PC3 UAV UE, the additional spurious emission requirements are specified according to the ECC Decision (22)07 for CEPT countries. In Rel-18 discussion companies clarified the regulatory requirements apply to PC3 only. For the high power UAV UE, it’s not clear how to handle the regulatory aspect. In addition, the ECC Decision (22)07 applies to non-AAS aerial UE only. For high power UE equipped with antenna array, the ECC Decision doesn’t apply.

Proposal 3: RAN4 to study how to handle the regulatory requirements for high power UAV UE.

The FDD band n3 is considered as the example band for studying the high power UAV UE. There is potentially impact on the REFSENS requirements when the transmit power is increased. In the case of ATG UE, the REFSENS requirements refer to TN UE requirements although the transmit power could be up to 40dBm. RAN4 needs to decide whether the same principle could be used for high power UAV UEs.

Proposal 4: RAN4 to study the impact on REFSENS from increased transmit power.

The remaining RF requirements including power dynamic, transmit signal quality, general spurious emission, etc are less impacted by the transmit power. Similar as PC3 UAV UEs, the current requirements of TN UE could still be reused. A general description is need to indicate the reference point of the requirements, i.e. on the antenna connector or on the TAB connector.

Proposal 5: A general description is needed to indicate the reference point of the requirements, i.e. on the antenna connector or on the TAB connector.

Conclusion

This document provides our view on the UAV RF requirements. Following proposals and observations are for RAN4 discussion.

Proposal 1: The maximum output power of up to 33dBm is specified as UE declaration.

Proposal 2: The ACLR requirements for high power UAV UE should follow the output of co-existence evaluation.

Proposal 3: RAN4 to study how to handle the regulatory requirements for high power UAV UE.

Proposal 4: RAN4 to study the impact on REFSENS from increased transmit power.

Proposal 5: A general description is needed to indicate the reference point of the requirements, i.e. on the antenna connector or on the TAB connector.

References

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), CMCC